3GPP TSG RAN WG2 #125 		                                    		   R2-2401009

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	7.2.2

Source:	InterDigital, Inc.

Title:	Discussion on correction for TS 38.305

Document for:	Discussion and Decision

# Introduction

Stage 2 specification TS 38.305 was endorsed [1]. In this contribution, we discuss a potential correction in stage 2 specification. It should be noted that we make a similar proposal in the contribution for the LPP specification in our companion contribution [2].

# Rx hopping for DL-AoD

In TS 38.305, the following table shows the measurement report sent from the UE to the LMF. In the report, whether the UE used Rx hopping to obtain RSRP is not indicated, namely an indication for the use of single or multi-hop.

“Table 8.11.2.2-1: Measurement results that may be transferred from UE to the LMF”

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Latitude/Longitude/Altitude, together with uncertainty shape | No | Yes |
| PCI, GCI, ARFCN, PRS resource ID, PRS resource set ID and PRS ID for each measurement | Yes | No |
| DL-PRS-RSRP measurement | Yes | No |
| Time stamp of the measurements | Yes | No |
| Time stamp of location estimate | No | Yes |
| DL-PRS receive beam index | Yes | No |
| DL-PRS-RSRPP measurements | Yes | No |
| LOS/NLOS information for UE measurements | Yes | No |
| Protection Level, optionally together with achievable Target Integrity Risk | No | Yes |

In TS 38.214 [3], as shown below, it is stated that the UE can measure DL PRS RSRP based on Rx hopping.

##### 5.1.6.5.1 PRS receiver frequency hopping The reduced capability UE may be configured to measure and report, subject to UE capability, via [nr-Requested-DL-PRS-measurementBasedOnMultihopRx] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with a requested bandwidth of all hops that may be greater than the maximum reduced capability UE bandwidth. ---Texts omitted---

Therefore, there is a discrepancy between the description in TS 38.214 and the stage 2 description shown in Table 8.11.2.2-1. We propose to send an LS to RAN1 to clarify whether Rx hopping is supported for the DL-AoD method.

Proposal 1: Send an LS to RAN1 to clarify whether Rx hopping is supported for the DL-AoD method.

# Conclusion.

In this contribution, the following proposal is made.

Proposal 1: Send an LS to RAN1 to clarify whether Rx hopping is supported for the DL-AoD method.

# Reference

- 3GPP, TS 38.305, “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, ” v 18.0.0., Dec.. 2023

- R2-2401010, “Discussion on correction for LPP,” InterDigital, RAN2#125, Athens, Greece, Feb., 2024.

- 3GPP, TS 38.214, “Physical layer procedures for data (Release 18), ” v 18.1.0., Dec.. 2023