3GPP TSG-RAN WG2 #125	R2-2401036

Athens, Greece, 26th February - 1st March 2024

Source:                    	DENSO CORPORATION

Title:  	Remaining consideration on MUSIM early indication

Document for:        	Discussion and decision

Agenda Item:         	7.17.2	RRC

- Introduction

In the MUSIM Open Issue list, the following issues are listed up:

| Issue number | Issue description |
| --- | --- |
| Issue 1 | Issue: Network’s action upon receiving of the early indication |
| Issue 2 | Issue: UAI/Early indication processing during handover procedure |
| Issue 3 | Issue: Whether when a band combination is indicated as forbidden, the fallback combinations of the reported band combination can be considered as forbidden, i.e., Fallback relationship of the forbidden BC and affected BC |
| Issue 4 | Issue: FFS whether UE should start a timer, e.g., Timer T348, after UE submits preference on the measurement gap requirement information. |
| Issue 5 | Issue: FFS whether all fields in musim-CapRestriction should be sent to SN. |
| Issue 6 | Issue: FFS on additional info on how the network set the content of MUSIM band list filter. |
| Issue 7 | Issue: How to understand the relation between MIMO/BW and CCs within the band, and whether the reactive timer or the proactive timer shall be used for the musim-MaxCC reporting |
| Issue 8 | Issue: A NOTE was added for early indication saying that the UE does not apply failure handling in case the UE is unable to apply part of the configuration and what the baseline configuration is, the similar issue may also occur after UE enters RRC_CONNECTED state, so FFS similar NOTE may be needed for RRC Reconfiguration. |

This paper attempts to investigate about remaining consideration on “early indication” based on the above open issue list.

- Discussion

As mentioned in Issue 8, a NOTE was added for the case where UE is unable to apply part of configuration on receiving RRCResume, in Rel-18 specification[1]. Additionally, the description of Issue 8 mentions about necessity to add similar NOTE for RRCReconfiguration.

During Rel-18 discussion, one of assumption for early indication seems that network provides limited configuration on RRCReconfiguration just after entering RRC_CONNECTED, then, actual configuration is provided based on capability restriction information sent via UEAssistanceInformation.

Figure 1. Sequence diagram for “early indication”

However, the “limited configuration” is not clearly defined, and there might be cases where UE is unable to apply part of the “limited configuration”. Therefore, we propose to clarify the UE behavior in such case.

Proposal 1:  RAN2 to clarify the UE behavior if the UE is unable to apply “limited configuration” due to MUSIM capability restriction on receiving RRCReconfiguration just after entering RRC_CONNECTED

We think the UE does not need to go to failure handling immediately even if the UE is unable to apply part of the “limited configuration”, as long as the configuration of sending UEAssisranceInformation for MUSIM capability restriction is provided. Additionally, we would like to propose to add a NOTE in the specification to clarify the UE behavior, similar to RRCResume.

Proposal 2: A NOTE would be added to clarify the UE behavior that UE does not need to go to failure handling immediately even if the UE is unable to apply part of “limited configuration” on receiving RRCReconfigration just after entering RRC_CONNECTED

- Summary and proposal

This paper discussed about consideration on MUSIM early indication.

Proposal 1:  RAN2 to clarify the UE behavior if the UE is unable to apply “limited configuration” due to MUSIM capability restriction on receiving RRCReconfiguration just after entering RRC_CONNECTED

Proposal 2:  A NOTE would be added to clarify the UE behavior that UE does not need to go to failure handling immediately even if the UE is unable to apply part of “limited configuration” on receiving RRCReconfigration just after entering RRC_CONNECTED

- References

[1] TS38.331, “NR; Radio Resource Control (RRC); Protocol specification”