3GPP TSG-RAN WG2 Meeting #125	R2-2401104

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.14.2

Source:	CATT

Title:	Discussion on the remaining issues for UE capabilities for QoE

Document for:	Discussion and Decision

# 1 Introduction

In previous RAN2 meeting [1], some agreements on UE capability have already achieved. But there are still some open issues on UE capability.

Open issue 1: Whether RedCap UE should have the same minimum memory size for QoE in RRC_IDLE/RRC_INACTIVE as non-RedCap UE? Open issue 2: Whether eRedCap UE shoud have the same minimum memory size for QoE in RRC_IDLE/RRC_INACTIVE as RedCap UE?

In this discussion paper, we will further analyse the remaining issues.

# 2 Discussions

In previous RAN2 meeting, it was agreed for non-Redcap UE, minimum memory requirement for IDLE/INACTIVE reports is 64KB. This memory is in addition to 64KB used for QoE report storage during pause. But for Redcap/eRedCap UE, it is FFS whether the minimum requirement is 64kB total for both IDLE/INACTIVE and paused reports.

In previous discussion on QoE, we did not distinguish the (e)RedCap UEs and non-RedCap UEs, the when the network configure the QoE measurement to UE, the network may not send Redcap UE specific QoE configuration to RedCap UEs. The motivation of introduce the additional buffer for QoE measurement in RRC_IDLE/INACTIVE state is that UE may stay in IDLE/INACTIVE state for a long time, 64kB buffer is not sufficient for QoE configuration in RRC_IDLE/INACTIVE state. So we agree to introduce additional at least 64kB. If we limit the Redcap UEs only reuse the 64kB buffer for paused reports, the QoE measurement in RRC_IDLE/INACTIVE for Redcap UEs may not work well. So we think in this release, for QoE measurement in RRC_IDLE/INACTIVE state, there is no need to introduce more enhancements for RedCap/eRedCap UEs and not define RedCap/eRadCap specific UE capability.

Proposal: In this release, not introduce specific enhancement for Redcap/eRedcap UEs and not define RedCap/eRadCap specific UE capability.

# 3 Conclusions

According to the above discussion, the following proposals are made:

Proposal: In this release, not introduce specific enhancement for Redcap/eRedcap UEs and not define RedCap/eRadCap specific UE capability.

# 4 References

- Draft Report of 3GPP TSG RAN2 meeting #123bis