3GPP TSG-RAN WG2 Meeting #125       	       	R2-2401254

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	7.17.2

Source:	China Telecom

Title:	Discussion on remaining open issues for MUSIM

WID/SID:	NR_DualTxRx_MUSIM - Release 18

Document for:	Discussion and Decision

1	Introduction

| Issue number | Issue description |
| --- | --- |
| Issue 2 | Issue: UAI/Early indication processing during handover procedure |
| Issue 3 | Issue: Whether when a band combination is indicated as forbidden, the fallback combinations of the reported band combination can be considered as forbidden, i.e., Fallback relationship of the forbidden BC and affected BC |
| Issue 4 | Issue: FFS whether UE should start a timer, e.g., Timer T348, after UE submits preference on the measurement gap requirement information. |
| Issue 5 | Issue: FFS whether all fields in musim-CapRestriction should be sent to SN. |
| Issue 6 | Issue: FFS on additional info on how the network set the content of MUSIM band list filter. |
| Issue 7 | Issue: How to understand the relation between MIMO/BW and CCs within the band, and whether the reactive timer or the proactive timer shall be used for the musim-MaxCCreporting |
| Issue 8 | Issue: A NOTE was added for early indication saying that the UE does not apply failure handling in case the UE is unable to apply part of the configuration and what the baseline configuration is, the similar issue may also occur after UE enters RRC_CONNECTED state, so FFS similar NOTE may be needed for RRC Reconfiguration. |

In this contribution, we discuss the above open issues.

2	Discussion

Issue 2: UAI/Early indication processing during handover procedure

When performing handover procedure, HandoverPreparationInformation message including UAI containing temporary capability restrictions would be sent from source gNB/source RAN to target gNB or CU to DU in RRC spec. In Rel-18, UE can send early indication to Network if it is allowed in SIB1. If the handover procedure occurs before the UAI containing temporary capability restrictions is reported, adding early indication musim-CapRestrictionInd in HandoverPreparationInformation message can help to generate corresponding RRCReconfiguration message.

Proposal1: Early indication for UE capability restriction needs to be included in HandoverPreparationInformation message.

Issue 3: Whether when a band combination is indicated as forbidden, the fallback combinations of the reported band combination can be considered as forbidden, i.e., Fallback relationship of the forbidden BC and affected BC

When a band combination is indicated as forbidden, it doesn’t mean its fallback combinations are considered as forbidden. For example, if band C is used in NW-A, the combination of band A+band B+band C is forbidden in NW-B, but its fallback combination of band A+ band B is available. Thus, The UE needs to explicitly report each forbidden/affected band combinations.

Proposal2: There is no fallback mechanism for the reported forbidden/affected band combinations in UAI. The UE needs to explicitly report each forbidden/affected band combinations.

Issue 4: FFS whether UE should start a timer, e.g., Timer T348, after UE submits preference on the measurement gap requirement information.

The value of musim-NeedForGapsInfoNR-r18 is NeedForGapsInfoNR-r16, so we think the timer handling should comply with the legacy case. It is more suitable to start a prohibit timer.

MUSIM-Assistance-v1800 ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18     MUSIM-GapPriorityPreferenceList-r18           OPTIONAL,

musim-GapKeepPreference-r18             ENUMERATED {true}                             OPTIONAL,

musim-CapRestriction-r18                MUSIM-CapRestriction-r18                      OPTIONAL,

musim-NeedForGapsInfoNR-r18             NeedForGapsInfoNR-r16                         OPTIONAL

}

| T346n | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for constraint combination of bands and/or band(s) or combination bands to avoid | Upon releasing musim-CapabilityRestrictionConfig during the connection re-establishment/resume procedures, or upon receiving musim-CapabilityRestrictionConfig set to release. | No action. |
| --- | --- | --- | --- |
| T348 | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for serving cell(s) with capabilities restricted, release of SCell or release of SCG. | Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction transmitted via musim-CapRestriction. | UE may apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction. |

Proposal 3: UE should start a probibit timer after UE submits preference on the measurement gap requirement information.

Issue 5: FFS whether all fields in musim-CapRestriction should be sent to SN.

- The MN can indicate the proactive/reactive temporary capabilities restriction (e.g. musim-CapRestriction-r18) to the SN together with the musim-candidateBandList-r18 (only for the proactive case).

- The configuration of gap priority and the “Keep solution indication” can be transferred from the MN to the SN node in the CG-ConfigInfo inter-node message.

The above agreements have achieved in last meeting, and in our understanding, all fields in musim-CapRestriction-r18 can be sent from MN to SN and can leave it to MN implementation to decide which field(s) need to be sent.

Proposal 4: all fields in musim-CapRestriction-r18 can be sent from MN to SN and can leave it to MN implementation to decide which field(s) need to be sent.

Issue 6: FFS on additional info on how the network set the content of MUSIM band list filter.

In our view, Network sets the content of MUSIM band list based on the available resource. Then, UE can indicate its preference on the concerned band(s) or band combination(s) (e.g. forbidden and/or affected band(s) or band combination(s)) based on a band-filter list configured by network. So, it is up to Network’s implementation on how to set the content of MUSIM band list filter, and no additional info is needed.

MUSIM band list filter is associated with the concerned band(s) or band combination(s) (e.g. forbidden and/or affected band(s) or band combination(s))

Proposal 5: no additional info on how the network set the content of MUSIM band list filter is needed.

Issue 7: How to understand the relation between MIMO/BW and CCs within the band, and whether the reactive timer or the proactive timer shall be used for the musim-MaxCC reporting

According to the current UE capability structure, MIMO and BW are reported per CC within the band. From our perspective, MIMO and BW can be reported in the same manner, i.e. reported on each CC within the band.

Proposal 6: Temporary UE capability restriction of MIMO/BW applies on each CC within the band.

As for musim-MaxCC reporting, it is more like a proactive method, since UE can indicate the specific SCellIndex to release the SCell. Hence, the proactive timer shall be used for the musim-MaxCC reporting.

Proposal 7: The proactive timer shall be used for the musim-MaxCC reporting.

Issue 8: A NOTE was added for early indication saying that the UE does not apply failure handling in case the UE is unable to apply part of the configuration and what the baseline configuration is, the similar issue may also occur after UE enters RRC_CONNECTED state, so FFS similar NOTE may be needed for RRC Reconfiguration.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

The above Note is already in 5.3.5.8.2 in current spec, although it doesn’t specifically refer to the early indication case.

Proposal8: Similar NOTE may not be needed for RRC Reconfiguration.

3	Conclusion

Proposal1: Early indication for UE capability restriction needs to be included in HandoverPreparationInformation message.

Proposal2: There is no fallback mechanism for the reported forbidden/affected band combinations in UAI. The UE needs to explicitly report each forbidden/affected band combinations.

Proposal 3: UE should start a probibit timer after UE submits preference on the measurement gap requirement information.

Proposal 4: all fields in musim-CapRestriction-r18 can be sent from MN to SN and can leave it to MN implementation to decide which field(s) need to be sent.

Proposal 5: no additional info on how the network set the content of MUSIM band list filter is needed.

Proposal 6: Temporary UE capability restriction of MIMO/BW applies on each CC within the band.

Proposal 7: The proactive timer shall be used for the musim-MaxCC reporting.

Proposal 8: Similar NOTE may not be needed for RRC Reconfiguration.