3GPP TSG-RAN WG2 Meeting #125	R2-2401259

# Athens, Greece, 26th Feb – 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0802 | rev | - | Current version: | 18.0.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS corrections to Stage 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, vivo, HiSilicon |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  |  |  | Date: |  |  |  | 2024-02-19 |  |
| Category: | F |  |  |  |  |  |  |  |  | Release: |  |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: | - In clause 7.2, some descriptions/figure for multicast reception in RRC_INACTIVE are missing. - In clause 7.3.1 of the latest version of TS 38.300, the description of SIB24 is missing. - In clause 16.10.3, multicast reception in RRC_INACTIVE is not considered. - In clause 16.10.5.7, supporting slot-level repetition for multicast MTCH PDSCH in RRC_INACTIVE (as mentioned in RAN1 LS R1-2306243) is not captured. To avoid misunderstanding that slot-level repetition is not supported for multicast reception in RRC_CONNECTED, a general description covering both RRC_CONNECTED and RRC_INACTIVE should be added. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | Add some descriptions and one figure for protocol architecture for multicast reception in RRC_INACTIVE in clause 7.2, 7.3.1 and 16.10.3. Add description of supporting slot-level repetition for multicast reception in 16.10.5.7. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Support of multicast reception by RRC_INACTIVE UEs in Stage 2 is not sufficient. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 7.2, 7.3.1, 16.10.3, 16.10.5.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  | x | Other core specifications |  |  |  |  | TS/TR ... CR .. |  |  |  |
| affected: |  |  |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <Start of modification>

## 7.2	Protocol States

RRC supports the following states which can be characterised as follows:

-	RRC_IDLE:

-	PLMN selection;

-	Broadcast of system information;

-	Cell re-selection mobility;

-	Paging for mobile terminated data is initiated by 5GC;

-	Transfer of MBS broadcast data to the UE over MRB(s);

-	DRX for CN paging configured by NAS.

-	RRC_INACTIVE:

-	PLMN selection;

-	Broadcast of system information;

-	Cell re-selection mobility;

-	Paging is initiated by NG-RAN (RAN paging);

-	RAN-based notification area (RNA) is managed by NG- RAN;

-	DRX for RAN paging configured by NG-RAN;

-	5GC - NG-RAN connection (both C/U-planes) is established for UE;

-	The UE Inactive AS context is stored in NG-RAN and the UE;

-	NG-RAN knows the RNA which the UE belongs to;

-	Transfer of MBS multicast/broadcast data to the UE over MRB(s);

-	Transfer of unicast data and/or signalling to/from the UE over radio bearers configured for SDT.

-	RRC_CONNECTED:

-	5GC - NG-RAN connection (both C/U-planes) is established for UE;

-	The UE AS context is stored in NG-RAN and the UE;

-	NG-RAN knows the cell which the UE belongs to;

-	Transfer of unicast data to/from the UE;

-	Transfer of MBS multicast/broadcast data to the UE over MRB(s);

-	Network controlled mobility including measurements.

## <Next of modification>

## 7.3	System Information Handling

### 7.3.1	Overview

System Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI:

-	Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

-	MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH.

-	SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

-	Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured or when the UE configured with inter cell beam management is receiving DL-SCH from a TRP with PCI different from serving cell's PCI). Other SI consists of:

-	SIB2 contains cell re-selection information, mainly related to the serving cell;

-	SIB3 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB4 contains information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters), which can also be used for NR idle/inactive measurements;

-	SIB5 contains information about E-UTRA frequencies and E-UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB6 contains an ETWS primary notification;

-	SIB7 contains an ETWS secondary notification;

-	SIB8 contains a CMAS warning notification;

-	SIB9 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SIB10 contains the Human-Readable Network Names (HRNN) of the NPNs listed in SIB1;

-	SIB11 contains information related to idle/inactive measurements;

-	SIB15 contains information related to disaster roaming;

-	SIB16 contains slice-based cell reselection information;

-	SIB17 contains information related to TRS configuration for UEs in RRC_IDLE/RRC_INACTIVE;

-	SIBpos contains positioning assistance data as defined in TS 37.355 [43] and TS 38.331 [12];

-	SIB18 contains information related to the Group IDs for Network selection (GINs) associated with SNPNs listed in SIB1.

-	SIB19 in TN contains NTN-specific parameters for NTN neighbour cells as defined in TS 38.331 [12].

For sidelink, Other SI also includes:

-	SIB12 contains information related to NR sidelink communication;

-	SIB13 contains information related to SystemInformationBlockType21 for V2X sidelink communication as specified in TS 36.331 clause 5.2.2.28 [29];

-	SIB14 contains information related to SystemInformationBlockType26 for V2X sidelink communication as specified in TS 36.331 clause 5.2.2.33 [29];

-	SIB23 contains information related to ranging and sidelink positioning.

For non-terrestrial network, Other SI also includes:

-	SIB19 contains NTN-specific parameters for serving cell and optionally NTN-specific parameters for neighbour cells as defined in TS 38.331 [12].

-	SIB25 contains TN coverage are information as defined in TS 38.331 [12].

For MBS broadcast, Other SI also includes:

-	SIB20 contains MCCH configuration;

-	SIB21 contains information related to service continuity for MBS broadcast reception.

For ATG network, Other SI also includes:

-	SIB22 contains ATG-specific parameters for serving cell and optionally ATG-specific parameters for neighbour cells as defined in TS 38.331 [12].

For MBS multicast reception in RRC_INACTIVE, Other SI also includes:

-	SIB24 contains the information required to acquire the multicast MCCH/MTCH configuration as defined in TS 38.331 [12].

Figure 7.3.1-1 below summarises System Information provisioning.

Figure 7.3.1-1: System Information Provisioning

For a cell/frequency that is considered for camping by the UE, the UE is not required to acquire the contents of the minimum SI of that cell/frequency from another cell/frequency layer. This does not preclude the case that the UE applies stored SI from previously visited cell(s).

If the UE cannot determine the full contents of the minimum SI of a cell by receiving from that cell, the UE shall consider that cell as barred.

In case of BA, the UE only acquires SI on the active BWP.

If the UE is configured with inter cell beam management:

-	the UE is not required to acquire the SI from the serving cell while it is receiving DL-SCH from a TRP with PCI different from serving cell's PCI.

## <Next of modification>

### 16.10.3	Protocol Architecture

Figure 16.10.3-1(a), 16.10.3-1(b), and 16.10.3-2 depict the downlink Layer 2 architecture for multicast session for reception in RRC_CONNECTED, multicast session for reception in RRC_INACTIVE, and broadcast session respectively, where MBS protocol stack comprises the same layer 2 sublayers as described in clause 6 with the following differences:

-	SDAP sublayer provides only the following functionalities:

-	Mapping between an MBS QoS flow and an MRB;

-	Transfer of user plane data.

-	PDCP sublayer provides only the following functionalities:

-	Transfer of user plane data;

-	Maintenance of PDCP SNs;

-	Header compression and decompression using the ROHC protocol or EHC protocol;

-	Reordering and in-order delivery;

-	Duplicate discarding.

-	For a multicast session, gNB provides one or more of the following multicast MRB configuration(s) to the UE in RRC_CONNECTED via dedicated RRC signalling:

-	Multicast MRB with DL only RLC-UM or bidirectional RLC-UM configuration for PTP transmission;

-	Multicast MRB with RLC-AM entity configuration for PTP transmission;

-	Multicast MRB with DL only RLC-UM entity for PTM transmission;

-	Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for PTP transmission and the other DL only RLC-UM entity for PTM transmission;

-	Multicast MRB with three RLC-UM entities, one DL RLC-UM entity and one UL RLC-UM entity for PTP transmission and the other DL only RLC-UM entity for PTM transmission;

-	Multicast MRB with two RLC entities, one RLC-AM entity for PTP transmission and the other DL only RLC-UM entity for PTM transmission.

-	For a multicast session, gNB may change the MRB type for the UE in RRC_CONNECTED using RRC signalling.

-	For a multicast session, gNB provides the following multicast MRB configuration to the RRC_INACTIVE UE via broadcast RRC signalling or dedicated RRC signalling:

-	Multicast MRB with DL only RLC-UM entity for PTM transmission.

Figure 16.10.3-1(a): Downlink Layer 2 Architecture for Multicast Session for reception in RRC_CONNECTED

Figure 16.10.3-1(b): Downlink Layer 2 Architecture for Multicast Session for reception in RRC_INACTIVE

-	For broadcast session, gNB provides the following broadcast MRB configuration to the UE using broadcast RRC signalling:

-	Broadcast MRB with one DL only RLC-UM entity for PTM transmission.

Figure 16.10.3-2: Downlink Layer 2 Architecture for Broadcast Session

## <Next of modification>

16.10.5.7	Physical Layer

A CFR configured by RRCReconfiguration message is defined for multicast scheduling as an 'MBS frequency region' with a number of contiguous PRBs confined within and with the same numerology as the DL BWP, and multicast scheduling may have specific characteristics (e.g., PDCCH, PDSCH and SPS configurations). The CFR for the multicast reception in RRC_INACTIVE state and the CFR for broadcast can be configured differently. If one CFR is not completely contained within the other CFR, the UE in RRC_INACTIVE state is not required to receive both broadcast and multicast simultaneously.

Two HARQ-ACK reporting modes are defined for MBS:

-	For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block or detects a DCI format indicating an SPS PDSCH release; otherwise, the UE generates HARQ-ACK information with NACK value.

-	For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values.

HARQ-ACK feedback for multicast can be enabled or disabled by higher layer configuration per G-RNTI or per G-CS-RNTI and/or indication in the DCI scheduling multicast transmission.

HARQ feedback is not supported for MBS multicast session data reception for UEs in RRC_INACTIVE state. Slot-level repetition is supported for multicast reception.

## <End of modification>