3GPP TSG-RAN WG2 Meeting #125	R2-2401346

Athens, Greece, February 26th – 1st, 2024

Agenda item:	5.1.3.2

Source: 	Sequans Communications

Title: 	Discussion on max data rate calculation

Document for:	Discussion and Decision

# Introduction

In this contribution, we consider a potential ambiguity on the BW in the max data rate calculation formula for NR, as well as the setting of supportedModulationOrderDL/UL.

# Discussion

The specification 38.306 states the following:

For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by maxNumberMIMO-LayersPDSCH for downlink and maximum of maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by supportedModulationOrderDL for downlink and supportedModulationOrderUL for uplink. is the scaling factor given by scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. is the maximum RB allocation in bandwidth with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where is the UE supported maximum bandwidth in the given band or band combination. is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL NOTE: Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL.

## UE supported maximum bandwidth

The formula involves  which is related to bandwidth  through the following table from in 5.3 TS 38.101-1 (taking FR1 as an example):

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 |

Contrary to other parameters which point to a capability, it might not be completely clear what is meant by “the UE supported maximum bandwidth in the given band or band combination”.

In UE capabilities, there is supportedBandwidthDL/UL, mandatory included in FeatureSetDownlinkPerCC/  FeatureSetUplinkPerCC, which “Indicates maximum DL/UL channel bandwidth supported for a given SCS that UE supports within a single CC, which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2”. The corresponding type is as follows:

#### – SupportedBandwidth The IE SupportedBandwidth is used to indicate the maximum channel bandwidth supported by the UE on one carrier of a band of a band combination. SupportedBandwidth information element -- ASN1START -- TAG-SUPPORTEDBANDWIDTH-START SupportedBandwidth ::= CHOICE { fr1 ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100}, fr2 ENUMERATED {mhz50, mhz100, mhz200, mhz400} } -- TAG-SUPPORTEDBANDWIDTH-STOP -- ASN1STOP

At the time of introduction of 35MHz and 45MHz CBW, it was noted that those CBWs are not in SupportedBandwidth and a CR was agreed (see ‎[1]‎[2]). The following is indicated (with a similar text for UL):

| supportedBandwidthDL […] The UE may report a supportedBandwidthDL wider than the channelBWs-DL; this supportedBandwidthDL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3]. […] | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

With this change, there could be 2 different interpretations of bandwidth :

- bandwidth  is the maximum effectively supported CBW for the CC in the band/BC

  - It depends on supportedBandwidthDL/UL, BCS and channelBWs-DL/UL

- bandwidth  is the supportedBandwidthDL/UL indicated for the CC in the band/BC

As an example, we consider a UE supporting 35MHz CBW on band n8 (it cannot report 35MHz in supportedBandwidthDL/UL). We further assume that the UE BB processing is limited to 350Mbps: it will have to advertise scaling factor/modulation order accordingly to indicate its peak rate limitation.

Depending on the interpretation of bandwidth  by the UE, the advertised Qm/f will be different:

| Interpretation | bandwidth |  | Data Rate (assuming 2layers, Q=8) | Qm | f | Scaled DL Data Rate |
| --- | --- | --- | --- | --- | --- | --- |
| 1) Maximum effectively supported CBW on one CC | 35 MHz | 188 | 402.3 | 8 | 0.8 | 321.9 |
| 2) Reported supportedBandwidthDL | 40 MHz (for instance) | 216 | 462.3 | 8 | 0.75 | 346.7 |

Obviously, different interpretation of bandwidth  between UE and NW would have bad consequences:

- If UE considers effective max CBW (smaller), while NW considers supportedBandwidthDL/UL (larger), there is a risk of higher allocation which would result in the UE dropping PDSCH/PUSCH

  - In above case, NW would calculate a max supported data rate = 462.3*.8 = 369.8 Mbps > 350Mbps

- If UE considers supportedBandwidthDL/UL (larger), while NW considers effective max CBW (smaller), the UE would not be scheduled up to its processing capabilities:

  - In above case, NW would calculate a max supported data rate = 402.3*.75 = 301.7 Mbps << 350Mbps

It seems easier to consider option 2), i.e. the bandwidth  in the data rate formula is always the reported supportedBandwidthDL/UL. However, this would also mean that the calculated data rate is not always achievable, which might be in contradiction with “A UE supporting NR (NR SA, MR-DC) shall support the calculated DL and UL max data rate defined in 4.1.2.” Assuming that statement needs to be fulfilled, a UE supporting 35Mhz up to the peak data rate of 402.3 Mbps would then need to downscale the “40 MHz” non-achievable data rate of 462.3 Mbps, which would yield 369.8 Mbps (f=0.8).

Proposal 1: Discuss/confirm whether BW(j) in the data rate formula is the reported supportedBandwidthDL/UL or the  max CBW effectively supported

The wording used in 38..306 seems to allow setting a supportedBandwidthDL/UL larger than the one actually supported even when supportedBandwidthDL/UL could signal the correct value, i.e. in other cases than 35MHz/45MHz CBW. The supported CBW may be for instance already limited by BCS/channelBWs. For a UE, this can be useful as it enables to reduce the number of FeatureSets to be supported, hence the signaling overhead.

Proposal 2: Discuss/confirm whether larger supportedBandwidthDL/UL is allowed in all cases, or only when the actual max CBW cannot be signaled

## Maximum supported modulation order

The maximum supported modulation order Qm given by supportedModulationOrderDL/UL is defined as follows:

| supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. […] | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

It was discussed in the past in CRs ‎[3]‎[4], where it was mainly clarified that:

- The NW may use a modulation order that is higher than the indicated “supportedModulationOrderDL/UL”

- If not included, the default value is the actually supported modulation order

However, from cover page of ‎[3], it is indicated

To avoid ambiguous, the following 5 cases should be avoided by UE when filling the UE capability IEs sent to the NW:

Case 1,

supportedModulationOrderDL set to pi/2 bpsk or bpsk, as for downlink there is no pi/2 bpsk and bpsk.

Case 2,

supportedModulationOrderUL set to pi/2 bpsk while pusch-HalfPi-BPSK set to ‘not supported’.

Case 3,

supportedModulationOrderUL set to 256QAM while corresponding band’s pusch-256QAM set to ‘not supported’.

Case 4,

supportedModulationOrderDL set to 256QAM for a CC within a FR1 band while pdsch-256QAM-FR1 set to ‘not supported’.

Case 5,

supportedModulationOrderDL set to 256QAM for a CC within a FR2 band while this band’s pdsch-256QAM-FR2 set to ‘not supported’.

There are no such restrictions anywhere in the specification. Moreover, supportedModulationOrderDL/UL is only used in the max data rate calculation with a role very similar to the scaling factor, with increased per-CC granularity. It has no longer any relationship with the actually supported modulation. Hence, there is no reason to have such limitation, while there are benefits in allowing all values, since it increases the granularity of the data rate scaling. In particular, it should be allowed to use Qm=8 even if 256QAM is not supported, or Qm=1 even if (pi/2) BPSK is not supported.

For eRedcap, the following was introduced in 38.306:

For single carrier NR SA operation and for UEs supporting supportOfERedCap-r18, the UE shall support a data rate for the carrier that is the data rate computed using the above formula, with J=1 CC and: - if the UE supports eRedCapNotReducedBB-BW-r18: - component vLayers(j)⋅Qmj⋅fj is 0.75 if vLayers(j)=1, or; - component vLayers(j)⋅Qmj⋅fj is 0.8 if vLayers(j)=2; - else: - component vLayers(j)⋅Qmj⋅fj is 3.2, and; - is 25 if μ = 0 or, 12 if μ = 1;

As it can be seen, Qm.f are mostly handled together to scale the datarate, with no relationship on the actual supported modulation. For instance, v.Qm.f = 0.75 needs Qm=1, which does not correspond to a supported modulation in DL.

Proposal 3: Confirm that supportedModulationOrderDL/UL is not restricted by the supported modulations

# Conclusion

In this contribution, we make the following proposals:

Proposal 1: Discuss/confirm whether BW(j) in the data rate formula is the reported supportedBandwidthDL/UL or the  max CBW effectively supported

Proposal 2: Discuss/confirm whether larger supportedBandwidthDL/UL is allowed in all cases, or only when the actual max CBW cannot be signaled

Proposal 3: Confirm that supportedModulationOrderDL/UL is not restricted by the supported modulations

# References

- R2-2104249, “Further Clarification on the 35M/45M supporting”, ZTE Corporation, Sanechips

- R2-2106691 CR on the 35M45M supporting-R15

- R2-1819022, 38.306 CR 0057 “Clarify for per CC UL/DL modulation order capabilities”

- R2-1908533, 38.306 CR 0118 “Clarification on supported modulation order capability”