3GPP TSG-RAN WG2 Meeting #125                                     R2-2401465

Athens, Greece, 26th February- 1st March, 2024

Source: 			OPPO

Title: 	Discussion on reply to SA3 LS on security aspects for Ranging/Sidelink Positioning

Agenda item:		7.2.1

Document for: 	Discussion and Decision

- Discussion

In the SA3#113 meeting, an LS regarding the sucurity aspects for Ranging/Sidelink Positioning was sent out towards both SA2 and RAN2 [1]. Generally, SA3 indicates that it is necessary to perform privacy check of the Target/Reference UEs for location exposure to SL positioning client UE, and asks RAN2 to confirm that whether or not PC5 message or SLPP message already supports to carry the required information between the UEs or can be supported in Rel-18 timeframe.

From our perspective, it is questionable why the prvacy check of the SL Reference UEs needs to performed to check whether or not the location information of the SL Reference UEs can be exposed to the SL positioning client UE, since we can find that the SL Reference UE’s location information is only useful for the SL positioning server UE to derive the SL ranging/positioning result of the target UE with the aid of the SL-PRS measurement between the SL Reference UE and the target UE, and therfore its location info should be only obtained by the SL positioning server UE, rather than the SL positioning client UE.

Observation 1: From RAN2 perspective, the SL Reference UE’s location information is only useful for the SL positioning server UE to derive the SL ranging/positioning result of the target UE with the aid of the SL-PRS measurement between the SL Reference UE and the target UE, and therfore its location info cannot be exposed to the SL Positioning Client UE.

On the other hand, regarding the case of privacy check on the target UE, since the SL positioning client UE is interested in obtaining the position info of the target UE, privacy check on whether or not the position info of the target UE could be exposed to the SL positioning client UE should be performed. According to the section 6.8 Procedures of Ranging/Sidelink Positioning control of the TS 23.586 v18.0.0, such implementation could be already fulfilled when the SL Positioning Client UE sends Ranging/SL Positioning service request towards the UE2/../UEn over PC5, and therfore no RAN2 impact is forseen.

Observation 2: privacy check on whether or not the position info of the target UE could be exposed to the SL positioning client UE could be already fulfilled when the SL Positioning Client UE sends Ranging/SL Positioning service request towards the UE2/../UEn over PC5.

Proposal 1: RAN2 to agree that no RAN2 impact is forseen on performing privacy check on both of the target UE and the SL Reference UE for location exposure to SL positioning client UE.

Correspondingly, A reply LS regarding clarification on our opinion that no RAN2 impact is forseen could be sent back to SA3.

Proposal 2: RAN2 to agree that an reply LS to clarify our opinion that no RAN2 impact is forseen on performing privacy check on the target UE and the SL Reference UE for location exposure to SL positioning client UE could be sent back to SA3.

- Conclusion:

In this contribution paper, following observations and proposals are made by us:

Observation 1: from RAN2 perspective, the SL Reference UE’s location information is only useful for the SL positioning server UE to derive the SL ranging/positioning result of the target UE with the aid of the SL-PRS measurement between the SL Reference UE and the target UE, and therfore its location info cannot be exposed to the SL Positioning Client UE.

Observation 2: privacy check on whether or not the position info of the target UE could be exposed to the SL positioning client UE could be already fulfilled when the SL Positioning Client UE sends Ranging/SL Positioning service request towards the UE2/../UEn over PC5.

Proposal 1: RAN2 to agree that no RAN2 impact is forseen on performing privacy check on the target UE and the SL Reference UE for location exposure to SL positioning client UE.

Proposal 2: RAN2 to agree that a reply LS to clarify our opinion that no RAN2 impact is forseen on performing privacy check on the target UE and the SL Reference UE for location exposure to SL positioning client UE could be sent back to SA3.

- Reference

[1] S3-235078, Reply LS on security aspects for Ranging/Sidelink Positioning, SA3#113, Chicago, USA, November, 2023