3GPP TSG-RAN2 Meeting #125	R2-2401647

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0030 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR sidelink relay enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In section 5.3a.1, there is no egress link determination for the transmission operation of U2U Remote UE which should be added since there may be multiple U2U Relay links for the remote UE, and the egress link determination is needed. - In section 5.3a.1.2, clarify the UE ID fields corresponding to the Local ID pair of the concerned L2 ID pair to make the specification clearer. - In section 5.3a.2, change the lowercase letter after a dash to uppercase for the practice of the spec. - In section 5.3a.1.3/5.3a.3.3, for the egress RLC channel determination for DRB, it is captured as “Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to RLC channel ID configured for the concerned bearer as specified in TS 38.331 [3].”. But “configured” is not accurate since for the IDLE/INACTIVE/OOC case, the egress RLC channel is derived by the RRC layer based on SIB/Pre-configuration but not configured by the network. Therefore, saying “SRAP configuration indicated by upper layer” to cover all the cases is more accurate. - In 5.3a.3.2, clarify the egress link determination at U2U Relay UE to make the specification clearer. - In section 5.4, how to handle the error case when PDU with an unknown UE ID/Bearer ID is received for U2U Remote and Relay UE should be added. - In section 6.3.2, clarify which of the two fields for the U2U case is for SRC UE ID or the DST UE ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 5.3a.1.2, add the egress link determination at U2U Remote UE together with UE IDs determination. - In section 5.3a.1.2, clarify the UE ID fields corresponding to the Local ID pair of the concerned L2 ID pair. - In section 5.3a.2, change the lowercase letter after a dash to uppercase. - In section 5.3a.1.3/5.3a.3.3, change “RLC channel ID configured” to “SRAP configuration indicated by upper layer”. - In 5.3a.3.2, clarify the egress link determination at U2U Relay UE. - In section 5.4, add the error case handling for U2U Relay case. - In section 6.3.2, clarify which of the two fields for the U2U case is for SRC UE ID or the DST UE ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The egress link determination behavior at U2U Remote UE is missing; - The specification is not clear enough on the UE fields set. - The use of uppercase/lowercase letters after a dash is not unified in the spec. - It is incorrect to say “configured egress RLC channel” for IDLE/INACTIVE/OOC case since there is no configuration on that. - The specification is not clear enough on the egress link determination at U2U Relay UE. - The handling of error case when PDU with unknown UE ID/Bearer ID is received for U2U Remote and Relay UE is missing. - The specification is not clear enough on which of the two fields for the U2U case is for SRC UE ID or the DST UE ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3a.1, 5.3a.2, 5.3a.3.2, 5.3a.3.3, 5.4, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 4549, 4638 TS 38.306 CR 1056 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.3a.1	Transmitting operation of U2U Remote UE

5.3a.1.1	General

The transmitting part of the SRAP entity on the PC5 interface of U2U Remote UE can receive SRAP SDU from upper layer and constructs U2U SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the UE ID fields, egress link and BEARER ID field in accordance with clause 5.3a.1.2;

-	Construct an U2U SRAP Data PDU with SRAP header, where the UE ID fields and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3a.1.3;

-	Submit this U2U SRAP Data PDU to the determined egress PC5 Relay RLC channel of the determined egress link.

5.3a.1.2	UE ID fields and BEARER ID field determination

For an U2U SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID (for SRC) field corresponding to sl-RemoteUE-LocalIdentity configured for the concerned sl-RemoteUE-L2Identity and UE ID (for DST) field corresponding to sl-PeerRemoteUE-LocalIdentity configured for the concerned sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3];

-	Determine the egress link on PC5 interface towards the U2U relay UE based on the concerned sl-RemoteUE-L2Identity and sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3].

-	Determine the BEARER ID field for SL-SRBs as the fixed value (i.e., set 0/1/2/3 for SL-SRB0/1/2/3 respectively) or for SL-DRBs as the 5 LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure as specified in TS 38.331 [3].

5.3a.1.3	Egress RLC channel determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the U2U SRAP Data PDU is for SL SRB (i.e., the BEARER ID field is 0/1/2/3):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3].

-	Else if the U2U SRAP Data PDU is for SL DRB:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to SRAP configuration indicated by upper layer for the concerned bearer as specified in TS 38.331 [3].

Next Change

5.3a.2	Receiving operation of U2U Relay UE

Upon receiving an U2U SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface between the U2U Relay UE and the U2U Remote UE shall:

-	Deliver the SRAP data packet to the transmitting part of the SRAP entity on the PC5 interface between the U2U Relay UE and the peer U2U Remote UE.

Next Change

5.3a.3.2	Egress link determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If there is an entry in sl-LocalID-PairToAddModList, in which the sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity match the UE ID fields in the U2U SRAP Data PDU:

-	Determine the egress link on PC5 interface towards the peer U2U remote UE identified by sl-PeerRemoteUE-L2Identity configured for the concerned sl-PeerRemoteUE-LocalIdentity as specified in TS 38.331 [3].

Next Change

5.3a.3.3	Egress RLC channel determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the U2U SRAP Data PDU is for SRB (i.e., the BEARER ID field is 0/1/2/3):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3].

-	Else if the U2U SRAP Data PDU is for DRB:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to SRAP configuration indicated by upper layer  for the concerned bearer as specified in TS 38.331 [3].

Next Change

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2U Remote UE, if sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity are both configured, when a SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in sl-SRAP-ConfigPC5 or a SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or the 5 bits LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Relay UE, when a SRAP Data PDU with SRAP header that contains a DST UE ID field which does not match sl-PeerRemoteUE-LocalIdentity included in sl-SRAP-ConfigPC5 configured by the U2U Relay UE or a SRAP Data PDU with SRAP header contains BEARER ID field which does not match 0/1/2/3 or the 5 bits LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure is received, or when a SRAP Data PDU that contains a SRC UE ID field which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link is received by U2U Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall:

-	discard the received SRAP PDU.

Next Change

6.3.2	UE ID

Length: 8 bits.

In U2N Relay case, this field carries local identity of U2N Remote UE. In U2U Relay case, there are two UE ID fields: first one carrying the local identity of SRC U2U Remote UE, second one carrying the local identity of DST U2U Remote UE.

End of Change