3GPP TSG-RAN WG2 Meeting #125bis	R2-2402476

Changsha, China, April 15th – 19th, 2024

Agenda item:	8.9.3

Source:	Huawei, HiSilicon

Title:	Overview of capacity enhancement for uplink

Document for:	Discussion and Decision

- Introduction

According to the RAN plenary #103 meeting, the following objectives on IoT-NTN [1] were agreed and are listed as follows:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we will discuss the details about the enhancements to reduce the necessary uplink and downlink signalling to achieve capacity enhancements for uplink.

- Discussion

## 2.1 OCC enhancement

RAN1 has started to discuss OCC enhancement during the last meeting and made the following agreements:

Agreement For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level (including Nslot level) - Repetition-level - RV-level - Intra-symbol pre-DFT spreading OCC

Since only some initial conclusions for further evaluation and study are achieved, we should wait until RAN1 has made sufficient progress on this topic to initiate RAN2 discussion.

Proposal 1: Postpone RAN2 discussion on OCC until sufficient progress is made in RAN1.

## 2.2 EDT enhancement

### 2.2.1 Consideration of contention based PUR

In Rel-15, MO-EDT allows one UL data transmission (in Msg3) for small data optionally followed by one DL transmission (in Msg4) during the random access procedure, avoiding UE’s transition to RRC_CONNECTED mode. In Rel-16, preconfigured uplink resource (PUR) is introduced to further improve latency/efficiency for small data transmission compared to MO-EDT. Msg1 and Msg2 in random access procedure are not needed in PUR. UE can directly begin with data transmission on the UL resource preconfigured in RRCRelease message in case the UE has valid TA.

In order to increase the uplink capacity for IoT-NTN, we have decided to enhance the current EDT procedure to reduce the necessary uplink and downlink signaling overhead for data transmission. According to objectives of IoT-NTN WID, there will be no msg1/ Random Access Response, and msg4 will be optimized. This is very similar with the PUR mechanism which is an existing enhancement of EDT. To avoid repeating the discussion already happened in Rel-16, we can directly take PUR as the baseline in IoT NTN for uplink capacity enhancement.

Proposal 2: RAN2 to take PUR mechanism as the baseline in IoT NTN for uplink capacity enhancement.

What can be further considered for IoT NTN is PUR with shared resource for multiple UEs, i.e., contention-based PUR, which can further increase the uplink capacity. Since this objective is only targeted at RAN2, it is better to clearly state  that there is no RAN1 impact expected for this.

Proposal 3: Contention-based PUR can be considered as one candidate solution with the assumption of no RAN1 impact.

### 2.2.2 Configuration of contention based PUR

If contention based PUR is supported, there are some options on table for the configuration of the Contention based PUR:

Option 1: Broadcast the PUR configuration in the SIB.

Option 2: Use RRCRelease to configure the PUR, similar as legacy.

Option 3: Broadcast the PUR resource in the SIB and configure other information (e.g., TAT) in RRCRelease.

Before selecting an option, RAN2 needs to identify which information is needed to be configured for PUR in IoT NTN and whether it needs to be configured per UE. As far as we can see, the TA related information to maintain TA validation should be UE specific. Legacy transmission using PUR can only be performed when UE meets the TA validation criteria as specified in TS 36.331.

#### 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when the following conditions are fulfilled: 1> either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers; and 1> either pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is not configured or the following conditions are fulfilled: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh;

However, in R17 IoT NTN, RAN4 has agreed that RSRP-based TA validation requirements is not needed for PUR TA validation in RAN4#105 since the RSRP doesn’t change much in NTN:

#### Issue 2-3-1: PUR, RSRP-based TA validation - Agreement - Do not define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO

So at least pur-RSRP-ChangeThreshold is not needed in IoT NTN.

Observation 1: There is no RAN4 requirement for configuring RSRP-based TA validation parameter for PUR.

Another difference between NTN and TN is that, in NTN, UE can pre-compensate TA for UL data transmission based on valid satellite ephemeris information and UE location information. However, it may need further confirmation from RAN1 and RAN4 whether the accuracy of the calculated TA can meet the PUR transmission requirement. It would be safer to send an LS to RAN1/RAN4 before making a conclusion in RAN2.

Proposal 4: The RSRP-based TA validation parameter is not needed for PUR in IoT NTN.

Proposal 5: Send an LS to RAN1/RAN4 to ask whether the calculated TA can meet the PUR transmission requirement in IoT NTN and postpone the discussion on how PUR is configured.

### 2.2.3 Contention resolution of contention based PUR

Since the contention-based PUR resource is shared by multiple UEs, when different UEs choose the same resource, contention occurs just like what may happen during the RACH procedure. Therefore, contention resolution is also needed via Msg4 for the contention-based PUR transmission. To realize this, similar mechanism as contention resolution during RACH can be used. Specifically, the network obtains the UE ID in Msg3 and identifies the winner of the contention. And in order to indicate the contention resolution result to the UE, the NW can send a MAC CE including information of the UE ID via Msg4 for UE to check.

Proposal 6: A MAC CE including UE ID information is used for contention resolution for the contention-based PUR. FFS the detailed design of the MAC CE.

Another discussion point is which RNTI the UE uses to monitor the PDCCH of the Msg4. For legacy PUR, a UE specific PUR-RNTI is configured for this. And for the legacy RACH procedure, TC-RNTI is provided for this in RAR, which is specific for the resource and the preamble that the UEs chose. On the other hand, for monitoring RAR, the UE can calculate an RA-RNTI based on the RACH resource. So basically there are two ways for the UE to acquires the RNTI for MSG4 for contention-based PUR: via NW configuration or via UE performing the calculation by itself. It is simpler to configure a RNTI for the specific PUR resource but since the PUR resource is periodic, one UE using the PUR resource of occasion 1 may also receive the MSG4 for the PUR resource of occasion 2.To avoid such confusion at the UE, it is better if UE calculates the RNTI by itself.

Proposal 7: UE calculates an RNTI by itself for monitoring the PDCCH of MSG4. The details of RNTI calculation is FFS.

### 2.2.4 Fallback of contention based PUR

UE may fail the contention resolution for contention-based PUR when multiple UEs select the same resource for UL data transmission. One possible solution is that the UE re-attempts via contention-based PUR. Another possible solution is that the UE falls back to legacy EDT procedure and performs RACH.

Proposal 8: RAN2 to discuss UE’s fallback mechanism if contention resolution fails.

- Conclusion

In this paper, we discuss the enhancements to reduce the necessary uplink and downlink signalling to achieve capacity enhancements for uplink, and have the following proposals:

OCC enhancement

Proposal 1: Postpone RAN2 discussion on OCC until sufficient progress is made in RAN1.

Consideration of contention based PUR

Proposal 2: RAN2 to take PUR mechanism as the baseline in IoT NTN for uplink capacity enhancement.

Proposal 3: Contention-based PUR can be considered as one candidate solution with the assumption of no RAN1 impact.

Configuration of contention based PUR

Observation 1: There is no RAN4 requirement for configuring RSRP-based TA validation parameter for PUR.

Proposal 4: The RSRP-based TA validation parameter is not needed for PUR in IoT NTN.

Proposal 5: Send an LS to RAN1/RAN4 to ask whether the calculated TA can meet the PUR transmission requirement in IoT NTN and postpone the discussion on how PUR is configured.

Contention resolution of contention based PUR

Proposal 6: A MAC CE including UE ID information is used for contention resolution for the contention-based PUR. FFS the detailed design of the MAC CE.

Proposal 7: UE calculates an RNTI by itself for monitoring the PDCCH of MSG4. The details of RNTI calculation is FFS.

Fallback of contention based PUR

Proposal 8: RAN2 to discuss UE’s fallback mechanism if contention resolution fails.

- References

RP-240449, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.