3GPP TSG-RAN WG2 #125bis	Tdoc R2-2403008

Changsha, China, April 15th – 19th, 2024

Agenda Item:	7.3.2

Source:	Ericsson

Title:	MAC corrections for cell DTX-DRX

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the FFS from RAN2#125 meeting concerning potential MAC corrections for cell DTX.

# 2	Discussion

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

.

In the current version of 38.321 CR, the following is captured regarding for UE actions during the cell DTX non-active period:

During RAN2#125 meeting the following FFS was noted:

=>   FFS to check if the following two conditions should be “shall not” rather than “need not”

- instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

- set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

We understand the overall intention with the procedures captured for non-active cell DTX active period is to have a permissive behaviour, and there is no need to have a special handling for the aspects highlighted above. Hence, we propose to keep the text as it is in the current version of 38.321 CR.

No change is needed in the text concerning the UE actions during the cell DTX non-active period in 38.321 CR.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	No change is needed in the text concerning the UE actions during the cell DTX non-active period in 38.321 CR.