3GPP TSG-RAN WG2 Meeting #125bis	R2-2403130

Changsha, China, April 15th – 19th, 2024

Agenda Item:	7.5.3.3

Source:	OPPO

Title:	Correction on the DRX_SFN_COUNTER initialization

Document for:	Discussion, Decision

Introduction

This paper will correct the text on the DRX_SFN_COUNTER initialization.

Discussion

RAN2#125 agrees to use drx-TimeReferenceSFN to initialize DRX_SFN_COUNTER.

Agreements

1 	Address the issue of DRX-config crossing the H-SFN boundary:

Initialize DRX_SFN_COUNTER to 1 when the DRX-config RRC signalling containing drx-TimeReferenceSFN is received during the first half of a hyper frame (SFN is between 0 and 511).

Otherwise, initialize DRX_SFN_COUNTER to 0.

Based on this agreement, the MAC spec is updated as below.

1> if the drx-NonIntegerLongCycleStartOffset is configured: 2> increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0; 2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1 in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]. 3> else: 4> set DRX_SFN_COUNTER to 0 in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5].

However, the text above may lead to incorrect value-setting for DRX_SFN_COUNTER when performing the “else” branch. This can happen when the RRC (re)configuration message with drx-TimeReferenceSFN is received in the second half of a hyper frame and the first RRCReconfigurationComplete message of this RRC (re-) configuration is sent in the first half of the next hyper frame. Note that there is a 10+ ms delay between the RRCReconfiguration message and the RRCReconfigurationComplete message. To resolve this issue, we recommend initializing DRX_SFN_COUNTER when this RRC (re-)configuration is received.

Update the MAC spec to initialize DRX_SFN_COUNTER upon the corresponding RRC (re-)configuration message for DRX being received.

Conclusion

We have the following proposals:

Proposal 1	Update the MAC spec to initialize DRX_SFN_COUNTER upon the corresponding RRC (re-)configuration message for DRX being received.

Text Proposal

5.7	Discontinuous Reception (DRX)

(Text omitted)

1>	if the drx-NonIntegerLongCycleStartOffset is configured:

2>	increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

2>	if DRX is (re-)configured by RRC:

3>	if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511):

4>	set DRX_SFN_COUNTER to 1 when this RRC (re-)configuration message is received.

3>	else:

4>	set DRX_SFN_COUNTER to 0 when this RRC (re-)configuration message is received.

(Text omitted)

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)