3GPP TSG-RAN WG2 Meeting #125bis	DRAFT_R2-2403736

Changsha, China,  April 15th – 19th, 2024

Source: 	Session chair (Huawei)

Title:	Report from session on R18 MBS, R18 QoE and R19 XR

## 2.4	Instructions

Rel-17 maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

Rel-18 CR Handling

-	CR editors / Rapporteurs continue to support maintenance related to their respective CR / WI and are required to follow drafting rules

-	Single correction CR per spec coordinated by CR editor/rapporteurs will be agreed per feature for RAN#104

-	A list of open issues is expected to be created per CR per WI and shared from CR editors/rapporteurs

-	CR editors / Rapporteurs are to gather miscellaneous and non-controversial issues, if any, for their respective specification prior to submission deadline.  Other companies are expected to give inputs to these CRs and not have contributions on such issues.

-	Companies are should give inputs on editorials and clarifications to the CR editors/rapporteurs and not have individual CRs/contributions on such issues.   Emails to CR editors/rapporteurs should follow the following naming convention when sending emails to rapporteurs:

[Pre_RAN2#125][CR xx.yyy] Clarification CRs

-	The organizational AIs for each WIs are reserved for rapporteurs only.  CR rapporteurs are expected to submit only 1 CR per spec.

-	For RRC corrections, only selected RIL can be submitted in the agenda (i.e. only if RRC editor suggests to discuss the RIL under this agenda)

-	Companies are expected to submit Tdocs with TP (not CRs).   More specifically, the Tdoc should contain description of open issues/proposal and the proposed corrections/TP in the contribution itself..   Small issues can be included in the tdoc with just short justification same level of detail as in cover sheet.

-	RRC ASN.1 changes can be drafted in a NBC way until ASN.1 is frozen, to avoid unnecessary RRC overhead.   The focus should be on drafting the changes in the best possible way.

-	Inter-op analysis on Rel-18 CR coverpages in NOT needed

Remaining/updated Rel-18 RRC parameters and MAC CEs

-	RRC parameters updates/corrections, including those requested by other groups, e.g. RAN1, are covered by WI-specific RRC CRs.

-	MAC CE parameters updates/corrections, including those requested by other groups, e.g. RAN1, are covered by WI-specific MAC CRs

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and only a draft CR per WI is expected and will be merged with the running mega CR

ASN.1 Review

-	Please follow the instructions provided in ASN.1 review rapporteur and read section “Review execution” on what to expect for paper submission.

https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BMisc%5D/ASN1%20review/Rel-18%202024-03

- Contributions on WI specific RILs should be submitted under the corresponding WI specific AI and NOT in the general ASN.1 review AI (7.0.3).  That AI is reserved for common/cross-WI specific identified RILs

- Title of contribution should start with [RIL number] Title, or "[RIL number1][RIL number N] Title” if there are more than one RIL in a Tdoc.

- Proposals related to RIL resolution should include RIL number in the proposal

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	WI rapporteurs input for WI planning etc,

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

-	Spec rapporteur list of open issues for Rel-18 items

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations doesn’t apply to Tdocs related to RILs which has been assigned during ASN.1 review.  Single Tdoc containing 1 or more RIL resolutions per WI is expected.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Tdoc request/submission for RAN2#125bis deadlines:

- Tdoc Submission deadline: April 5th, 2024 1000 UTC  NOTE: NO changes to titles, sourcing companies, or new additional requests are allowed past this date. This should be treated as final deadline similar to all meetings where Tdoc requests/submission deadlines are aligned.

# 7	Rel-18

## 7.11	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.11.1	Organizational

LS in, rapporteur input (e.g. rapporteur CR, open issues list)

R2-2402766	RIL list for MBS	Huawei, HiSilicon	report	Rel-18	NR_MBS_enh-Core	Late

PropAgree: C151, S735, C152, L011, L010, S736

PropReject: S734, S737

?? PropAgree and PropReject RIL resolutions are agreed

?? Companies should not resubmit rejected RILs

R2-2402767	MBS Rapporteur CR for RRC	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4688	-	F	NR_MBS_enh-Core	Late

?? Endorsed as a baseline for further changes

?? One week review after the meeting

### 7.11.2	RRC corrections

Corrections related to RILs from ASN.1 review.

ToDo RILs (high priority)

R2-2402282	[C148][C149][C150] RRC Corrections for eMBS	CATT, CBN, China Broadnet	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: To address RIL [C148], it is clarified that UE can only know the serving cell where the multicast service was received in RRC_CONNECTED for active session. TP in Annex 1 is adopted.

Proposal 3: To address RIL [C150], UE triggers RRC resume if multicast MCCH is not present upon receiving group paging that indicates the multicast session activation. TP in Annex 3 is adopted.

R2-2403508	[S731][S732][S733] Issues for Multicast Reception	Samsung	discussion	Rel-18

Proposal 1: Upon unsuccessful completion of the SDT procedure:

(a)	UE which is configured for multicast reception in RRC_INACTIVE, transits to RRC_IDLE (same as legacy spec).

(b)	UE forwards TMGI(s) to upper layers for multicast session(s) that UE is configured for multicast reception and receiving in RRC_INACTIVE. Adopt the text proposal TP1.

R2-2403597	[N101] [N102] [N103] [N104] [N105] [N106] [N107] [N108][N109] Control plane aspects of multicast reception in RRC_INACTIVE state	Nokia	discussion	Rel-18	NR_MBS_enh-Core

Proposal 3 [N102]: RAN2 discusses the following options:

Proposal 5 [N104]: If the following conditions are satisfied, UE assumes that the network operates MCCH-less for multicast reception in RRC_INACTIVE state:

Proposal 6: Upon UE detecting that the network operates MCCH-less for multicast reception in RRC_INACTIVE state in a cell, UE does not reconnect to the same cell although it cannot find SIB24 scheduled in the cell that sent UE to RRC_INACTIVE.

ToDo RILs (low priority)

R2-2402246	[V523][V531] Remaining Issues on Multicast Reception in INACTIVE	vivo	discussion	Rel-18	NR_MBS_enh-Core	Late

Proposal 1: For clause 5.3.13.1d, change “a multicast session that the UE has joined” to “at least one of the multicast session(s) that the UE has joined”.

Observation: In Rel-15 NR, decoding prioritization is up to UE implementation when more than two PDSCHs are received (e.g. the UE can choose to receive either PDSCH for SI or PDSCh for paging when the PDSCHs are scheduled simultaneously in the same slot).

Proposal 2: RAN2 to clarify that decoding prioritization is up to INACTIVE UE implementation when PDSCH for multicast MTCH and other PDSCH(s) for SI/paging/Msg2/MsgB are simultaneously received.

Proposal 3: RAN2 to adopt the text proposal in the Annex.

R2-2402282	[C148][C149][C150] RRC Corrections for eMBS	CATT, CBN, China Broadnet	discussion	Rel-18	NR_MBS_enh-Core

Proposal 2: To address RIL [C149], the description of MII reporting triggered upon handover or RRC re-establishment scenarios is modified. TP in Annex 2 is adopted.

R2-2402634	[Z695, Z696] Misc issues for multicast reception in RRC_INACTIVE with draft CR	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1	RAN2 to agree the draft CR in section 5 for UE handling in multicast reception in RRC_INACTIVE state.

Proposal 2	The presence of thresholdIndex IE is used as the indication whether the RRC resumption due to reception quality is enabled for one session.

R2-2403508	[S731][S732][S733] Issues for Multicast Reception	Samsung	discussion	Rel-18

Proposal 2: RAN2 to agree and capture that PDCP synchronization does not imply the multicast session availability within all the cells in RNA. Adopt the text proposal TP2.

Proposal 3: It is left up to UE implementation as to how it tracks multicast session inactivity in a MCCH-less cell. Capture in a Note.

R2-2403597	[N101] [N102] [N103] [N104] [N105] [N106] [N107] [N108][N109] Control plane aspects of multicast reception in RRC_INACTIVE state	Nokia	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1 [N101]: UE will consider to be allowed to receive a multicast service in RRC_INACTIVE state in the RRC release message with suspendConfig if the TMGI is present within MulticastConfigInactive-r18 IE is the configuration.

Proposal 2: To reflect above proposals in the specification, the changes in the Annex are adopted.

Proposal 4: Operating RRC_INACTIVE multicast with/without MCCH is a per PLMN configuration.

Proposal 7: MCCH-less operation is either made feasible without additional signalling in the air interface, as proposed within this paper, or removed from the specifications.

Proposal 8: [N103}[N105]When the UE receives stop monitoring G-RNTI indication in RRC release, it stops monitoring for data in the current cell and stop monitoring G-RNTI even after cell reselection to a cell that does not contain SIB24/MCCH (or PTM configuration).

Proposal 9: [N103}[N105] When the UE does not receive stop monitoring G-RNTI in RRC release, but only in MCCH, it stops monitoring data only in the cell where such indication is provided; and goes back to RRC_CONNECTED even after cell reselection to a cell that does not contain SIB24/MCCH (or PTM configuration).

Proposal 10: To reflect above proposals in the specification, the changes in the Annex are adopted.

Proposal 11 [N106]: PTM configuration in RRC release with suspendConfig belongs to the serving PCell of the UE in RRC_CONNECTED state.

Proposal 12: To reflect above proposals in the specification, the changes in the Annex are adopted.

Proposal 13: An LS is sent to RAN3 to define the signalling for information exchange on multicast delivery to UEs in RRC_INACTIVE state between neighbor gNBs.

Proposal 14: Remove the UE behaviour from Stage-3 regarding mbs-NeighbourCellList. as the UE behaviour is already captured in Stage-2.

Proposal 15 [N108]: If MBS multicast session continues in INACTIVE but with a different MRB (different LCID), the connected mode MRB is suspended (not released).

Proposal 16 [N107]: When PTM configuration is updated via MCCH, the UE shall perform multicast MRB modification if the LCID associated with the MRB is the same as in the previous PTM configuration; otherwise, the UE shall perform multicast MRB release/establishment.

Proposal 17 [N109]: A connected mode MRB continued in inactive mode is suspended (not released) when the MRB cannot be continued in cell reselection (pdcpSync not configured) and a new MRB is established in the reselected cell.

R2-2403604	RIL_J009/J010/J011 MBS CP	Sharp	discussion

Proposal 1: update the current text related to receive the RRCRelease message to align with the agreement.

Proposal 2: The acquired multicast MCCH information overwrites any stored multicast MCCH information and the PTM configuration configured in dedicated RRC message.

Proposal 3: To have a clarification on what is PTM configuration.

Non-RIL related

R2-2402849	Discussion on frequency information reported for shared processing	Xiaomi, Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

Proposal: RAN2 is kindly requested to clarify that the frequency band information reported is based on the UE capability supporting MBS.

Discussion on RILs from other companies

R2-2403547	MBS RILs	Ericsson	discussion	Rel-18	NR_MBS_enh-Core	Late

Withdrawn

R2-2402768	[H099] PTM configuration indication in the neighbour cell list for multicast	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core	Late

### 7.11.3	Other corrections

Corrections related to other specs, e.g. 38.300, 38.321, 38.323, UE capabilities.

R2-2402868	Clarification on MAC reset for multicast reception in RRC_INACTIVE	Apple, Samsung, CATT, Nokia, Huawei, HiSilicon, LG Electronics Inc.	CR	Rel-18	38.321	18.1.0	1800	-	F	NR_MBS_enh-Core

R2-2403203	Error data handling for MBS	Langbo	discussion	Rel-18	38.321	NR_MBS_enh-Core

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

Proposal 2: Move the handling of MAC PDU received for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured from clause 5.3.3 to clause 5.13.

Proposal 3: Adopt the TP in the Annex.

R2-2403546	Clarification for (e)RedCap UE supporting MBS in RRC_INACTIVE	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

Observation 1	When a RedCap UE indicates to support 16 DRBs and MBS multicast reception in RRC_INACTIVE it is unclear whether the RedCap UE supports 8 or 16 DRBs in RRC_INACTIVE.

Proposal 1	Clarify that supportOf16DRB-RedCap-r17 is applicable in RRC_CONNECTED and when the UE supports MBS multicast in RRC_INACTIVE it also is applicable in RRC_INACTIVE.

Proposal 2	Clarify that supportOf16DRB-RedCap-r17 is also applicable in RRC_INACTIVE when the UE supports SDT.

## 7.14	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.14.1	Organizational

LSs and rapporteur inputs (e.g. rapporteur CR, open issues list)

R2-2402103	LS on area scope handling for QoE measurement collection (C1-241717; contact: Ericsson)	CT1	LS in	Rel-18	NR_QoE_enh-Core	To:RAN2	Cc:SA4, SA5, RAN3

?? Noted

R2-2403247	RIL issue list for QoE	Ericsson	discussion	Rel-18	NR_QoE_enh-Core	Late

PropAgree: E215, G121

PropReject: G118, G119, G120, G122, G123, H082

?? PropAgree and PropReject RIL resolutions are agreed

R2-2403246	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson	CR	Rel-18	38.331	18.1.0	4711	-	F	NR_QoE_enh-Core	Late

?? Endorsed as a baseline for further changes

?? One week review after the meeting

### 7.14.2	QoE measurements in RRC_IDLE INACTIVE

Corrections related to RILs from ASN.1 review.

R2-2403159	Discussion on serving cell for MBS QoE collection H079H082	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1: For QoE area scope checking, the UE should recheck the area scope only when the cell from which the UE receives the service changes.

Proposal 2: QoE area scope checking should be performed for each QoE configuration separately.

R2-2403249	RIL issues related to QoE measurements	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1	Only include measConfigReportAppLayerAvailable in the first RRCReconfigurationComplete message at handover if the UE has stored QoE configuration with appLayerIdleInactiveConfig not successfully transmitted since the UE entered RRC_CONNECTED.

Proposal 2	Add a note to clarify that once a value of a parameter included in an RVQoE configuration has been forwarded to the UE application layer, the value of the parameter will be maintained in the UE application layer unless explicitly released.

Proposal 3	The UE sends the session status indication to the node that configured the QoE configuration.

### 7.14.3	Other corrections

Corrections related to other specs, e.g. 38.300, 37.340, UE capabilities.

R2-2403075	Consideration on QoE remaining issues	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1: RAN2 agrees to revise the field description of the flag idleInactiveReportAllowed based on the TP in the annex 1.

Proposal 2: UE includes RPLMN identity in QoE reports stored in AS layer when storing QoE reports in non-connected stat.

R2-2403248	Open issues for QoE measurements	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1	Send an LS to SA4 to discuss the problem of “pollution” of the QoE reports and the lack of measurement results. (A draft LS is included in the Annex.1.)

R2-2403486	Correction for NR QoE configurations release in inter-RAT HO	Nokia, Nokia Shanghai Bell	CR	Rel-18	38.331	18.1.0	4749	-	F	NR_QoE_enh-Core

## 7.24	TEI18

Specific items may be allocated to a breakout session for treatment.

Time budget: 1 TU

### 7.24.2	TEI proposals by RAN2

Items initiated in RAN2 for NR and LTE.

No contributions should be submitted under 7.24.2.  They should be submitted under 7.24.x

Tdoc limitation: 1 tdoc, limitation applicable to new proposals.  No new Cat. B proposals expected for this meeting

#### 7.24.2.2	Other RAN2 TEI-18

Contributions should focus only critical issues/corrections for already agreed TEI-18 topics.   New TEI proposals should address critical issues that should be resolved by RAN2#125.  Co-sourcing of such proposals is encouraged.   Contributions on items that were explicitly downprioritized from Rel-18 WIs should not be brought as TEI18.  No new Cat. B proposals expected for this meeting

Including outcome of [POST125][022][RedCap emergency calls] Review CRs (Apple) and [POST125][612][TEI18] CR for MBS operation with eDRX/MICO (Nokia)

MBS with eDRX/MICO

R2-2403598	MBS operation with eDRX MICO [TEI18 NR_MBS_enh]	Nokia, Ericsson	CR	Rel-18	38.304	18.1.0	0399	-	F	TEI18

MBS and (e)RedCap

LSin

R2-2402112	LS on separate CFR introduced in Rel-18 TEI of MBS for RedCap UE applied for eRedCap UE (R1-2401732; contact: ZTE)	RAN1	LS in	Rel-18	NR_redcap_enh-Core	To:RAN2

Search space clarifications

R2-2402324	Remaining Issue on Broadcast CFR for Redcap	vivo	discussion	Rel-18	NR_MBS-Core, NR_redcap-Core, TEI18	R2-2400955	Late

R2-2402631	MCCH Search space for (e)RedCap UE MBS broadcast reception	ZTE, Sanechips	discussion	Rel-18	TEI18

R2-2402769	Clarification on MBS search spaces configuration for (e)Redcap [RedCapMBS_Bcast]	Huawei, CATT, Xiaomi, HiSilicon	discussion	Rel-18	TEI18, NR_MBS_enh-Core, NR_redcap_enh-Core

R2-2402770	Correction on MBS search spaces configuration for (e)Redcap [RedCapMBS_Bcast]	Huawei, CATT, Xiaomi, HiSilicon	CR	Rel-18	38.331	18.1.0	4689	-	F	TEI18, NR_MBS_enh-Core, NR_redcap_enh-Core

Stage-2 clarifications

R2-2402283	Correction to 38.300 for redcap CFR of MBS	CATT, CBN, China Broadnet	discussion	Rel-18	NR_MBS_enh-Core

R2-2403548	MBS RedCap CFR in Stage 2	Ericsson	discussion	Rel-18	TEI18

Proposal 1	Add a short description about MBS RedCap CFR to 38.300.

Proposal 2	Clarify in 38.306 that a RedCap UE supporting MBS broadcast also supports RedCap CFR.

Proposal 3	Capture in 38.300 that a UE only monitors one CFR at a time. A RedCap UE monitors the RedCap CFR, if configured, otherwise the default CFR if the bandwidth of the default CFR is within the UE capability.

Proposal 4	Clarify in 38.300 that the NR-RAN node ensures that the UE does not receive two DCIs for the same G-RNTI.

Other

R2-2403549	MBS and eRedCap UE	Ericsson	discussion	Rel-18	TEI18

Proposal 1	Introduce NR RedCap UE information for eRedCap with or without reduced baseband bandwidth.

Proposal 2	RAN2 assumes that when NR RedCap UE information is absent NG-RAN cannot assume anything about the type of UE the MBS broadcast session is intended for.

# 8	Rel-19

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240791)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.7.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2402836	XR Workplan	Nokia, Qualcomm (Rapporteurs)	Work Plan	Rel-19	NR_XR_Ph3-Core

R2-2402837	SA2 Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402838	Multi-Modal Communication Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.2	Multi-modality support

Objective: Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption.

Including aspects such as: intended use cases, target requirements, relation with SA2/SA4 work, solution directions.

R2-2402278	Discussions on Multi-modality Awareness	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402353	Discussion on XR Multi-modality	Spreadtrum Communications	discussion	Rel-19

R2-2402400	Justification and areas of interest for Multi-modal Services	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402443	Multi-Modality Support in RAN	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402474	Discussion on multi-modal XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402510	Initial Consideration on Multi-Modality	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402549	Discussion on multi-modality support for XR	CMCC, CSPG	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402628	Discussion on Multi-modality	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402676	Discussion on Multi-modality support for XR traffic	Xiaomi Communications	discussion

R2-2402762	RAN enhancements for Multi-Modality support	ZTE Corporation, Sanechips	discussion

R2-2402841	Discussion on Multi-modal support for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402879	Views on Support of Multi-Modality Services in Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402953	Support of Multi-Modal XR applications	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402979	Discussion on Multi-Modality XR	Meta	discussion

R2-2403064	XR multi modal flows	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2403091	Discussion on Multi-modality support for XR	TCL	discussion	Rel-19

R2-2403118	Discussion on multi-modality enhancement for XR traffic	China Telecom	discussion

R2-2403133	Discussion on the multi-modality support	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403223	Discussion on multi-modality	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403294	Multi-modality support for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403407	Discussion on multi-modality support	NEC Corporation	discussion

R2-2403411	Multi-modality work in Rel-19	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403569	Consideration on RAN enhancements for Multi-Modality	China Unicom	discussion	NR_XR_Ph3-Core

R2-2403659	Multi-modality support for XR	Google Inc.	discussion

R2-2403674	Discussion on multi-modality	MediaTek Inc.	discussion	Rel-19

### 8.7.3	RRM measurement gaps restrictions related enhancements

Objective: Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc).

This agenda item will not be treated during RAN2#125bis and no contributions should be submitted for this AI for this meeting.

### 8.7.4	Scheduling enhancements

Objective: For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

Including aspects such as: identification of current scheme drawbacks/limitations, enhancement directions.

R2-2402314	Discussion on Scheduling enhancements in XR	TCL	discussion	Rel-19

R2-2402325	Discussion on scheduling enhancements for XR	OPPO	discussion

R2-2402339	Discussion on XR scheduling enhancement	Spreadtrum Communications	discussion	Rel-19

R2-2402389	Discussion on delay-aware scheduling	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402401	Areas of interest for UL scheduling enhancements of XR traffic	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402444	Scheduling Enhancements for Delay-Critical Data Transmission	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402511	Consideration on XR specific scheduling enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402550	Discussion on scheduling enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402629	Discussion on scheduling enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402675	Discussion on scheduling enhancements of XR traffic	Xiaomi Communications	discussion

R2-2402684	Discussion on delay-based UL scheduling enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402763	Scheduling enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2402880	Views on Delay-Aware Operations in Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402952	Enhanced Uplink Scheduling for XR	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402980	Discussion on Scheduling Enhancement for XR	Meta	discussion

R2-2403045	Considerations on delay-sensitive scheduling for XR	NEC  Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403052	Scheduling Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403065	UL Scheduling enhancements for XR	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2403119	Discussion on scheduling enhancements for XR traffic	China Telecom	discussion

R2-2403143	Delay-aware scheduling enhancements	Huawei, HiSilicon	discussion	Rel-19

R2-2403225	UL scheduling enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403295	Scheduling enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403415	Discussion for scheduling enhancements	III	discussion	NR_XR_Ph3-Core

R2-2403591	Discussion on UL scheduling enhancements	MediaTek Inc.	discussion	Rel-19

R2-2403626	Discussion on resource allocation for XR 	Google Inc.	discussion	NR_XR_Ph3-Core

R2-2403669	Discussion on scheduling enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403690	UL related Scheduling Enhancements for XR	Rakuten Mobile, Inc	discussion	Rel-19

### 8.7.5	RLC enhancements

Objective: RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

Including aspects such as: identification of current scheme drawbacks/limitations, enhancement directions.

R2-2402212	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402254	RLC re-transmission enhancements for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2402279	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402318	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402354	Discussion on RLC enhancements for XR	Spreadtrum Communications	discussion	Rel-19

R2-2402390	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402402	RLC AM retransmission enhancements	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402512	Consideration on RLC Retransmission Enhancement for XR	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402515	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402573	Discussion on RLC enhancements in XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402630	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402685	Discussion on RLC enhancements for XR	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402699	Considerations on RLC re-transmission related enhancements for XR	KDDI Corporation	discussion

R2-2402734	AM RLC enhancement	Lenovo	discussion	Rel-19

R2-2402764	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2402817	RLC AM enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402839	RLC Enhancements for XR	Nokia	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402881	Views on RLC-AM Enhancements for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2402981	Discussion on RLC Enhancements for XR	Meta	discussion

R2-2403090	RLC AM retransmission enhancement for XR	TCL	discussion	Rel-19

R2-2403102	Discussion on RLC enhancements on small packet delay budget scenario	MediaTek Inc.	discussion	Rel-19

R2-2403296	RLC enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403368	Discussion on RLC AM Enhancements	Ericsson	discussion	Rel-19

R2-2403462	Consideration on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403504	Discussion on RLC enhancements for XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2403675	Discussion on RLC Retransmission Enhancements for XR	Rakuten Mobile, Inc	discussion	Rel-19