3GPP TSG-RAN WG2 Meeting #126	R2-2404249

Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item:	5.1.2.1

Source: 	CATT

Title: 	Impact on HARQ RTT timer handling due to UL and SL prioritization

Document for: 	Discussion and Decision

Introduction

In the last meeting, whether the HARQ RTT should be started by an actual UL transmission or a “virtual” UL transmission occasion was discussed and clarified in terms of various Rel-15/16/17 features, based on contribution in [1], with the following progress [2]:

Table A: Progress in RAN2 #125bis

R2-2403384 Clarification of Start Time of HARQ RTT Timer Samsung discussion Rel-15 NR_newRAT-Core, NR_unlic-Core, NR_IIOT-Core, NR_MBS-Core, TEI17 Proposal 1. The definition of drx-LastTransmissionUL is clarified that drx-HARQ-RTT-TimerUL is started after the last PUSCH occasion. - Ericsson thinks that this is a useful. But for NR-U there is no need to specify further, as in 5.21.1 it is already clear. Huawei and Vivo agree. => Captured in apple’s CR Proposal 2. A NOTE is added to clarify that in case of LBT failure drx-HARQ-RTT-TimerUL is started after the PUSCH occasion (not actual transmission). => Not supported Proposal 3. A NOTE is added to clarify that in case of LBT failure drx-HARQ-RTT-TimerDL is started after the PUCCH occasion (not actual transmission). => Not supported

Cited from R2-2403384 - drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last PUSCH occasion within a bundle;

Nevertheless, there is one missing case w.r.t. how the HARQ RTT timer should be started if the UL transmission is dropped due to UL and SL prioritization introduced in Rel-16 NR V2X.

In this contribution, therefore, we discuss this specific case, and propose to clarify intended UE behavior from RAN2 perspective.

Discussion

In [1], below table summarizes how the UL “transmission” that starts the HARQ RTT timer is interpreted for relevant Rel-15/16/17 features that can result in a UL transmission occasion dropped with no actual UL transmission performed:

Table 1. Interpretation of “transmission” over multiple releases of NR Release; WI; Interpretation. Rel-15; Legacy NR; actual transmission. Rel-16; IIOT; actual transmission. Rel-16; NR-U; virtual transmission. Rel-17; MBS; actual transmission. Rel-17; TEI17 drx-LastTransmissionUL; virtual transmission.

However, in Rel-16 NR V2X, there was a UL and SL prioritization mechanism introduced, where the UL transmission can be dropped due to an overlapped SL transmission/reception, and it may also lead to the UL transmission occasion dropped without real transmission performed. The UL and SL prioritization mechanism is specified in TS 38.321 [2] and TS 38.213 [3], as cited in Appendix 1.

On drx-HARQ-RTT-TimerUL

The UL and SL prioritization can result in PUSCH transmission skipping which may further have impact on   drx-HARQ-RTT-TimerUL handling. In more detail, it is observed that the prioritization between PUSCH transmission and PSSCH transmission is jointly specified in MAC and PHY, whilst the prioritization between PUSCH transmission and all other kinds of SL transmission (e.g. S-SSB, PSFCH, etc.) or receptions are specified in the PHY Spec.

Observation 1: PUSCH transmission may be dropped due to UL and SL prioritization. Prioritization between PUSCH and PSSCH transmissions is jointly specified by MAC and PHY, while prioritization between PUSCH transmission and all other SL transmission/reception is specified in PHY.

Then, the issue here is whether the drx-HARQ-RTT-TimerUL should be started after the end of the first actual transmission or the first “virtual” transmission (within a bundle) of the corresponding PUSCH transmission, in case the first 1 or more PUSCH transmissions were dropped due to UL and SL prioritization. See below figure as an example: whether the drx-HARQ-RTT-TimerUL should be started at T1 or T2.

In Rel-16, there was no agreement to address the above case, and so the drx-HARQ-RTT-TimerUL handling in Rel-16 MAC Spec may not be clear in terms of UL and SL prioritization.

Observation 1a: In the current Spec (from Rel-16 on), the intended UE behavior is unclear on whether drx-HARQ-RTT-TimerUL is started after the end of the first “actual” or “virtual” PUSCH transmission (within a bundle), in the case of UL and SL prioritization.

Considering that whether the PUSCH transmission can really be performed will finally be decided by the UL and SL prioritization rule in the PHY, it seems more logical to follow a similar way as NR-U and take the first “virtual” PUSCH transmission (i.e. transmission occasion) as the reference for the start of drx-HARQ-RTT-TimerUL. This needs to be confirmed by RAN2 first, and (if agreeable) can be applied in Rel-16 MAC Spec and the case when drx-LastTransmissionUL is not configured in Rel-17/18 Spec.

Proposal 1: RAN2 to clarify that the drx-HARQ-RTT-TimerUL is started for the corresponding HARQ process in the first symbol after the end of the first transmission occasion (within a bundle) of the corresponding PUSCH transmission, regardless of whether it is dropped due to UL and SL prioritization.

Proposal 1a: Apply Proposal 1(if agreed) in Rel-16 MAC Spec, and for the case when drx-LastTransmissionUL is not configured in Rel-17/18 MAC Specs.

On drx-HARQ-RTT-TimerDL

The UL and SL prioritization can also result in PUCCH transmission skipping which may further result in impact on drx-HARQ-RTT-TimerDL handling (that is started by PUCCH transmission carrying the DL HARQ feedback). Similar to above analyses for drx-HARQ-RTT-TimerUL, we also propose to confirm that the drx-HARQ-RTT-TimerDL is started, regardless of whether PUCCH carrying the DL HARQ feedback is dropped or not due to UL and SL prioritization.

Proposal 2: RAN2 to clarify that the drx-HARQ-RTT-TimerDL is started for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback, regardless of whether the PUCCH transmission is dropped due to UL and SL prioritization.

Spec Impact

Proposal 1, 1a and 2 are actually proposing to take “virtual” transmission occasion as the reference point for HARQ RTT timer start in the case of UL and SL prioritization. This is logically following the way of handling of NR-U and drx-LastTrasnmissionUL agreed in RAN2 #125 [4]. However, as seen from Table A, it was already clarified in the last meeting [4] that for NR-U such “virtual” transmission can already be reflected by in 5.21.1 in the existing MAC Spec, and the change to drx-LastTrasnmissionUL was agreed in a way that it applies to this feature only. Since neither of them is directly applicable to the UL and SL prioritization case, RAN2 considers adopting the TP in Appendix 2 for clarification in the MAC Spec, if Proposal 1, 1a and 2 are agreed.

Proposal 3: RAN2 to adopt the TP in Appendix 2 if proposals in 1, 1a and 2 are agreeable.

Conclusion

This contribution discusses the missing case of UL and SL prioritization, in terms of whether the HARQ RTT timer should be started after an “actual” or “virtual” transmission. Observations and proposals are listed as follows:

Observation 1: PUSCH transmission may be dropped due to UL and SL prioritization. Prioritization between PUSCH and PSSCH transmissions is jointly specified by MAC and PHY, while prioritization between PUSCH transmission and all other SL transmission/reception is specified in PHY.

Observation 1a: In the current Spec (from Rel-16 on), the intended UE behavior is unclear on whether drx-HARQ-RTT-TimerUL is started after the end of the first “actual” or “virtual” PUSCH transmission (within a bundle), in the case of UL and SL prioritization.

Proposal 1: RAN2 to clarify that the drx-HARQ-RTT-TimerUL is started for the corresponding HARQ process in the first symbol after the end of the first transmission occasion (within a bundle) of the corresponding PUSCH transmission, regardless of whether it is dropped due to UL and SL prioritization.

Proposal 1a: Apply Proposal 1(if agreed) in Rel-16 MAC Spec, and for the case when drx-LastTransmissionUL is not configured in Rel-17/18 MAC Specs.

Proposal 2: RAN2 to clarify that the drx-HARQ-RTT-TimerDL is started for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback, regardless of whether the PUCCH transmission is dropped due to UL and SL prioritization.

Proposal 3: RAN2 to adopt the TP in Appendix 2 if proposals in 1, 1a and 2 are agreeable.

Reference

- R2-2403384	Clarification of Start Time of HARQ RTT Timer	Samsung

- RAN2 #125bis Meeting Minutes

- TS 38.321

- TS 38.213

- Appendix 1: UL and SL prioritization (TS 38.321/38.213)

TS 38.321, V16.15.0 #### 5.4.2.2 HARQ process Each HARQ process is associated with a HARQ buffer. New transmissions are performed on the resource and with the MCS indicated on PDCCH or indicated in the Random Access Response (i.e. MAC RAR or fallbackRAR), or signalled in RRC or determined as specified in clause 5.1.2a for MSGA payload. Retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH, or on the same resource and with the same MCS as was used for last made transmission attempt within a bundle, or on stored configured uplink grant resources and stored MCS when cg-RetransmissionTimer is configured. If cg-RetransmissionTimer is configured, retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS. When cg-RetransmissionTimer is configured and the HARQ entity obtains a MAC PDU to transmit and LBT failure indication is received from lower layer, the corresponding HARQ process is considered to be pending. For a configured uplink grant, configured with cg-RetransmissionTimer, each associated HARQ process is considered as not pending when: - a transmission is performed on that HARQ process and LBT failure indication is not received from lower layers; or - the configured uplink grant is initialised and this HARQ process is not associated with another active configured uplink grant; or - the HARQ buffer for this HARQ process is flushed. If the HARQ entity requests a new transmission for a TB, the HARQ process shall: 1> store the MAC PDU in the associated HARQ buffer; 1> store the uplink grant received from the HARQ entity; 1> generate a transmission as described below. If the HARQ entity requests a retransmission for a TB, the HARQ process shall: 1> store the uplink grant received from the HARQ entity; 1> generate a transmission as described below. To generate a transmission for a TB, the HARQ process shall: 1> if the MAC PDU was obtained from the Msg3 buffer; or 1> if the MAC PDU was obtained from the MSGA buffer; or 1> if there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer or the MSGA buffer: 2> if there are neither transmission of NR sidelink communication nor transmission of V2X sidelink communication at the time of the transmission; or 2> if the transmission of the MAC PDU is prioritized over sidelink transmission or can be simultaneously performed with sidelink transmission: 3> instruct the physical layer to generate a transmission according to the stored uplink grant. If a HARQ process receives downlink feedback information, the HARQ process shall: 1> stop the cg-RetransmissionTimer, if running; 1> if acknowledgement is indicated: 2> stop the configuredGrantTimer, if running. If the configuredGrantTimer expires for a HARQ process, the HARQ process shall: 1> stop the cg-RetransmissionTimer, if running. The transmission of the MAC PDU is prioritized over sidelink transmission or can be performed simultaneously with sidelink transmission if one of the following conditions is met: - if there are both a sidelink grant for transmission of NR sidelink communication and configured grant(s) for transmission of V2X sidelink communication on SL-SCH as determined in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and neither the transmission of NR sidelink communication is prioritized as determined in clause 5.22.1.3.1a nor the transmission(s) of V2X sidelink communication is prioritized as determined in clause 5.14.1.2.2 of TS 36.321 [22]; or - if there are both a sidelink grant for transmission of NR sidelink communication and configured grant(s) for transmission of V2X sidelink communication on SL-SCH as determined in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and the MAC entity is able to perform this UL transmission simultaneously with the transmission of NR sidelink communication and/or the transmission(s) of V2X sidelink communication; or - if there is only configured grant(s) for transmission of V2X sidelink communication on SL-SCH as determined in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and either none of the transmission(s) of V2X sidelink communication is prioritized as determined in clause 5.14.1.2.2 of TS 36.321 [22] or the MAC entity is able to perform this UL transmission simultaneously with the transmission(s) of V2X sidelink communication; or - if there is only a sidelink grant for transmission of NR sidelink communication at the time of the transmission, and if the transmission of NR sidelink communication is not prioritized as determined in clause 5.22.1.3.1a, or there is a sidelink grant for transmission of NR sidelink communication at the time of the transmission and the MAC entity is able to perform this UL transmission simultaneously with the transmission of NR sidelink communication; or - if there are both a sidelink grant for transmission of NR sidelink communication and configured grant(s) for transmission of V2X sidelink communication on SL-SCH as determined in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and either only the transmission of NR sidelink communication is prioritized as determined in clause 5.22.1.3.1a or only the transmission(s) of V2X sidelink communication is prioritized as determined in clause 5.14.1.2.2 of TS 36.321 [22] and the MAC entity is able to perform this UL transmission simultaneously with the prioritized transmission of NR sidelink communication or V2X sidelink communication: NOTE 1: Among the UL transmissions where the MAC entity is able to perform the transmission of NR sidelink communication prioritized simultaneously, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed. NOTE 2: Among the UL transmissions that the MAC entity is able to perform simultaneously with all transmission(s) of V2X sidelink communication prioritized, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed. NOTE 3: Among the UL transmissions where the MAC entity is able to perform the transmission of NR sidelink communication prioritized simultaneously with all transmission(s) of V2X sidelink communication prioritized, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed. NOTE 4: If there is configured grant(s) for transmission of V2X sidelink communication on SL-SCH as determined in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and the MAC entity is not able to perform this UL transmission simultaneously with the transmission(s) of V2X sidelink communication, and prioritization-related information is not available prior to the time of the transmission due to processing time restriction, it is up to UE implementation whether this UL transmission is performed. [...] ##### 5.22.1.3.1a Sidelink process The Sidelink process is associated with a HARQ buffer. New transmissions and retransmissions are performed on the resource indicated in the sidelink grant as specified in clause 5.22.1.1 and with the MCS selected as specified in clause 8.1.3.1 of TS 38.214 [7] and clause 5.22.1.1. If the Sidelink process is configured to perform transmissions of multiple MAC PDUs with Sidelink resource allocation mode 2, the process maintains a counter SL_RESOURCE_RESELECTION_COUNTER. For other configurations of the Sidelink process, this counter is not available. Priority of a MAC PDU is determined by the highest priority of the logical channel(s) or a MAC CE in the MAC PDU. If the Sidelink HARQ Entity requests a new transmission, the Sidelink process shall: 1> store the MAC PDU in the associated HARQ buffer; 1> store the sidelink grant received from the Sidelink HARQ Entity; 1> generate a transmission as described below. If the Sidelink HARQ Entity requests a retransmission, the Sidelink process shall: 1> store the sidelink grant received from the Sidelink HARQ Entity; 1> generate a transmission as described below. To generate a transmission, the Sidelink process shall: 1> if there is no uplink transmission; or 1> if the MAC entity is able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission; or 1> if the other MAC entity and the MAC entity are able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission respectively; or 1> if there is a MAC PDU to be transmitted for this duration in uplink, except a MAC PDU obtained from the Msg3 buffer, the MSGA buffer, or prioritized as specified in clause 5.4.2.2, and the sidelink transmission is prioritized over uplink transmission: 2> instruct the physical layer to transmit SCI according to the stored sidelink grant with the associated Sidelink transmission information; 2> instruct the physical layer to generate a transmission according to the stored sidelink grant; 2> if HARQ feedback has been enabled for the MAC PDU according to clause 5.22.1.4.2: 3> instruct the physical layer to monitor PSFCH for the transmission and perform PSFCH reception as specified in clause 5.22.1.3.2. 2> if sl-PUCCH-Config is configured by RRC for the stored sidelink grant: 3> determine transmission of an acknowledgement on the PUCCH as specified in clause 5.22.1.3.2. 1> if this transmission corresponds to the last transmission of the MAC PDU: 2> decrement SL_RESOURCE_RESELECTION_COUNTER by 1, if available. NOTE 1: If the number of HARQ retransmissions selected by the MAC entity has been reached, or if a positive acknowledgement to a transmission of the MAC PDU has been received, or if a negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for the transmission of the MAC PDU, the MAC entity determines this transmission corresponds to the last transmission of the MAC PDU for Sidelink resource allocation mode 2. How to determine the last transmission in other cases is up to UE implementation. 1> if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTransNum; or 1> if a positive acknowledgement to this transmission of the MAC PDU was received according to clause 5.22.1.3.2; or 1> if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for this transmission of the MAC PDU according to clause 5.22.1.3.2: 2> flush the HARQ buffer of the associated Sidelink process. The transmission of the MAC PDU is prioritized over uplink transmission(s) of the MAC entity or the other MAC entity if the following conditions are met: 1> if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmission(s) at the time of the transmission, and 1> if none of the uplink transmission(s) is prioritized by upper layer according to TS 23.287 [19], and 1> if none of the NR uplink MAC PDU(s) includes any MAC CE prioritized as described in clause 5.4.3.1.3, and 1> if ul-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) of all the NR uplink transmission(s) is not lower than ul-PrioritizationThres, and 1> if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) or a MAC CE in the MAC PDU is lower than sl-PrioritizationThres. NOTE 2: If the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmissions as specified in clause 5.4.2.2 of TS 36.321 [22] at the time of the transmission, and prioritization-related information is not available prior to the time of this sidelink transmission due to processing time restriction, it is up to UE implementation whether this sidelink transmission is performed.

TS 38.213, V16.17.0 #### 16.2.4.3 Simultaneous SL and UL transmissions/receptions If a UE - would simultaneously transmit on the UL and on the SL in a carrier or in two respective carriers, and - the UE is not capable of simultaneous transmissions on the UL and on the SL in the carrier or in the two respective carriers the UE transmits only on the link, UL or SL, with the higher priority. If a UE - would simultaneously transmit on the UL and receive on the SL in a carrier, or - would simultaneously transmit on the UL and receive on the SL in two respective carriers and the UE is not capable of simultaneous transmission on the UL and reception on the SL in the two respective carriers the UE transmits on UL or receives on SL, with the higher priority. If a UE - is capable of simultaneous transmissions on the UL and on the SL in two respective carriers, - would transmit on the UL and on the SL in the two respective carriers, - the transmission on the UL would overlap with the transmission on the SL over a time period, and - the total UE transmission power over the time period would exceed PCMAX the UE - reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX - reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX ##### 16.2.4.3.1 Prioritizations for sidelink and uplink transmissions/receptions A UE performs prioritization between SL transmissions/receptions and UL transmissions after performing the procedures described in clause 9, clause 9.2.5, and clause 9.2.6, and in clause 6.1 of [6, TS 38.214]. PSFCH transmissions in a slot have a same priority value as the smallest priority value among PSSCH receptions with corresponding HARQ-ACK information provided by the PSFCH transmissions in the slot. PSFCH receptions in a slot have a same priority value as the smallest priority value among PSSCH transmissions with corresponding HARQ-ACK information provided by the PSFCH receptions in the slot. A priority of S-SS/PSBCH block transmission or reception is provided by sl-SSB-PriorityNR. For prioritization between SL transmission or PSFCH/S-SS/PSBCH block reception and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR and its retransmission, or a PUSCH corresponding to Type-2 random access procedure and its retransmission, or a PUCCH with sidelink HARQ-ACK information report - if the UL transmission is for a PUSCH or for a PUCCH with priority index 1, - if sl-PriorityThreshold-UL-URLLC is provided - the SL transmission or reception has higher priority than the UL transmission if the priority value of the SL transmission or reception is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UL transmission has higher priority than the SL transmission or reception - else - the UL transmission has higher priority than the SL transmission or reception - else - the SL transmission or reception has higher priority than the UL transmission if the priority value of the SL transmission(s) or reception is smaller than sl-PriorityThreshold; otherwise, the UL transmission has higher priority than the SL transmission or reception A PRACH transmission, or a PUSCH scheduled by an UL grant in a RAR and its retransmission, or a PUSCH for Type-2 random access procedure and its retransmission, or a PUCCH with HARQ-ACK information in response to successRAR, or a PUCCH indicated by a DCI format 1_0 with CRC scrambled by a corresponding TC-RNTI has higher priority than a SL transmission or reception. A PUCCH transmission with a sidelink HARQ-ACK information report has higher priority than a SL transmission if a priority value of the PUCCH is smaller than a priority value of the SL transmission. The priority value of the PUCCH transmission is as described in clause 16.5. If the priority value of the PUCCH transmission is larger than the priority value of the SL transmission, the SL transmission has higher priority. A PUCCH transmission with a sidelink HARQ-ACK information report has higher priority than a PSFCH/S-SS/PSBCH block reception if a priority value of the PUCCH is smaller than a priority value of the SL reception. If the priority value of the PUCCH transmission is larger than the priority value of the PSFCH/S-SS/PSBCH block reception, the SL reception has higher priority. When one or more SL transmissions from a UE overlap in time with multiple non-overlapping UL transmissions from the UE, the UE performs the SL transmissions if at least one SL transmission is prioritized over all UL transmissions subject to the UE processing timeline with respect to the first SL transmission and the first UL transmission. When one or more UL transmissions from a UE overlap in time with multiple non-overlapping SL transmissions, the UE performs the UL transmissions if at least one UL transmission is prioritized over all SL transmissions subject to the UE processing timeline with respect to the first SL transmission and the first UL transmission. When one SL transmission overlaps in time with one or more overlapping UL transmissions, the UE performs the SL transmission if the SL transmission is prioritized over all UL transmissions subject to both the UE multiplexing and processing timelines with respect to the first SL transmission and the first UL transmission, where the UE processing timeline with respect to the first SL transmission and the first UL transmission is same as when one or more SL transmissions overlap in time with multiple non-overlapping UL transmissions. When one SL transmission overlaps in time with one or more overlapping UL transmissions, the UE performs the UL transmission if at least one UL transmission is prioritized over the SL transmission subject to both the UE multiplexing and processing timelines with respect to the first SL transmission and the first UL transmission, where the UE processing timeline with respect to the first SL transmission and the first UL transmission is same as when one or more SL transmissions overlap in time with multiple non-overlapping UL transmissions.

- Appendix 2: TP to TS 38.321 (V16.15.0)

TEXT PROPOSAL START

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, and AI-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	If Sidelink resource allocation mode 1 is configured by RRC, a DRX functionality is not configured.

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-ShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX cycle for each DRX group.

2>	else:

3>	use the Long DRX cycle for each DRX group.

1>	if drx-ShortCycleTimer for a DRX group expires:

2>	use the Long DRX cycle for this DRX group.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1>	if the Short DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle):

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if the Long DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS or configured grant type 2 is considered to indicate a new transmission.

2>	if a HARQ process receives downlink feedback information and acknowledgement is indicated:

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	not report semi-persistent CSI on PUCCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

2>	if CSI masking (csi-Mask) is setup by upper layers:

3>	in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

4>	not report CSI on PUCCH in this DRX group.

NOTE 4:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

NOTE X:	drx-HARQ-RTT-TimerUL and drx-HARQ-RTT-TimerDL are started as specified in this subclause, irrespective of the prioritization between uplink transmission and sidelink transmission or reception as specified in subclauses 5.4.2.2, 5.22.1.3.1a and in subclause 16.2.4.3.1 in TS 38.213 [6].

TEXT PROPOSAL END

As UL and SL prioritization was introduced in Rel-16 NR Spec w/o big changes afterward, the citations are based on the latest version of Rel-16 Specs as an example.