3GPP TSG-RAN WG2 Meeting #126	R2-2404770

# Fukuoka, Japan, 20th - 24th May., 2024

Title:		Discussion on the reply LS for SL positioning measurement

Source:	Huawei, HiSilicon

Agenda item:	7.2.1

Document for:	Discussion and Decision

- Background

The following has been included in the RAN4 LS R4-2406386:

RAN4 has been discussing RRM requirements for SL PRS-RSRP and SL PRS-RSRPP measurements for SL positioning and observed that it is not fully clear in TS 38.355 whether or not SL PRS-RSRP and SL PRS-RSRPP can be reported standalone without any other SL positioning measurements (e.g., without SL AzimuthAoA, SL ZenithAoA, SL RSTD, SL Rx-Tx, or SL RTOA). RAN4 also observed that according to TS 38.355 in the current SL-AoA-RequestLocationInformation signalling, all measurements including SL AzimuthAoA, SL ZenithAoA, SL PRS-RSRP, and SL PRS-RSRPP are based on server’s request and can be reported as a single measurement or any combination of them. So, RAN4 would like to understand whether it is possible to request and/or report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning. RAN4 respectfully asks RAN2 to clarify: - whether SL PRS-RSRP or SL PRS-RSRPP can be requested and/or reported standalone specifically for SL-AoA positioning.

In this paper, we discuss the changes and clarifications need to be made by RAN2.

2	Discussions

The current SLPP spec for SL-AoA positioning, as RAN4 LS has mentioned, is specified as follows:

SL-AoA-ProvideLocationInformation ::= SEQUENCE { sl-AoA-SignalMeasurementInformation SL-AoA-SignalMeasurementInformation OPTIONAL, ... } SL-AoA-SignalMeasurementInformation ::= SEQUENCE { sl-AoA-MeasList SEQUENCE (SIZE(1.. maxNrOfUEs)) OF SL-AoA-MeasElement, ... } SL-AoA-MeasElement ::= SEQUENCE { applicationLayerID OCTET STRING, sl-LCS-GCS-Translation LCS-GCS-Translation OPTIONAL, -- sl-LCS-to-GCS-translation los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-AngleQuality MeasurementAngleQuality OPTIONAL, -- sl-AngleQuality sl-AoA-AdditionalPathList SL-AoA-AdditionalPathList OPTIONAL, sl-AzimuthAoA-Result INTEGER (0..3599) OPTIONAL, -- sl-PRS-AoA sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-TimeStamp SL-TimeStamp OPTIONAL, -- sl-Timestamp sl-ZenithAoA-Result INTEGER (0..1799) OPTIONAL, -- sl-PRS-AoA ... }

We have actually covered this issue during the first phase of the ASN1 discussion on the "core measurement" of each positioning methods and what can be requested in the requestLocationInformation per positioning method. For SL-AOA, the core measurement should be Azimuth AOA and Zenith AOA result. However, it is possible that only Azimuth or Zenith is requested and only Azimuth or Zenith is reported. Hence, in terms of ASN1 coding, both the field Azimuth and Zenith results are optional. However, from our point view, either one of them (Azimuth and Zenith) or both of them need to be present in the ProvideLocationInformation for SL-AOA. Hence, it should not be possible for SL-AOA measurement to report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning.

Proposal1: Specify in the field description of SL-AOA measurement that either one of Azimuth and Zenith results or both shall be present in SL-AOA-MeasElement. Adopt the TP in Annex A

Then, coming back to the RAN4 LS, RAN2 asked RAN2 whether it is possible to request and/or report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning. Based on the discussion above, this mode of measurement report should not be possible with the A-AOA and Z-AOA as the core measurement.

Proposal2: Reply to RAN4 that it is not possible for SL-AOA measurement to report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning. Adopt the rely LS in Annex B.

3	Conclusions

Proposal1: Specify in the field description of SL-AOA measurement that either one of Azimuth and Zenith results or both shall be present in SL-AOA-MeasElement. Adopt the TP in Annex A

Proposal2: Reply to RAN4 that it is not possible for SL-AOA measurement to report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning. Adopt the rely LS in Annex B.

Annex A: SLPP TP

#### –	SL-AoA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-START

SL-AoA-ProvideLocationInformation ::= SEQUENCE {

sl-AoA-SignalMeasurementInformation   SL-AoA-SignalMeasurementInformation    OPTIONAL,

...

}

SL-AoA-SignalMeasurementInformation ::= SEQUENCE {

sl-AoA-MeasList                         SEQUENCE (SIZE(1.. maxNrOfUEs)) OF SL-AoA-MeasElement,

...

}

SL-AoA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING,

sl-LCS-GCS-Translation                LCS-GCS-Translation       OPTIONAL,  -- sl-LCS-to-GCS-translation

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-AngleQuality                       MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AoA-AdditionalPathList             SL-AoA-AdditionalPathList OPTIONAL,

sl-AzimuthAoA-Result                  INTEGER (0..3599)         OPTIONAL,  -- sl-PRS-AoA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-ZenithAoA-Result                   INTEGER (0..1799)         OPTIONAL,  -- sl-PRS-AoA

...

}

SL-AoA-AdditionalPathList ::= SEQUENCE (SIZE(1..2)) OF SL-AoA-AdditionalPath

SL-AoA-AdditionalPath  ::= SEQUENCE {

sl-AngleQuality                            MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AzimuthAoA-AdditionalPathResult         INTEGER (0..3599)         OPTIONAL,  -- additionalPath-SL-PRS-AoA

sl-AzimuthAoA-LCS-GCS-Translation          LCS-GCS-Translation       OPTIONAL,  -- sl-LCS-to-GCS-translation

sl-ZenithAoA-AdditionalPathResult          INTEGER (0..1799)         OPTIONAL,  -- additionalPath-SL-PRS-AoA

sl-ZenithAoA-LCS-GCS-Translation           LCS-GCS-Translation       OPTIONAL,  -- sl-LCS-to-GCS-translation

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)          OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

...

}

MeasurementAngleQuality ::= SEQUENCE {

azimuthQuality              INTEGER (0..255),

zenithQuality               INTEGER (0..255)        OPTIONAL

}

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

SL-AoA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-AngleQuality This field specifies the angle quality for measurement results reported. Scale factor 0.1 degree.

sl-AoA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-AzimuthAoA-Result This field specifies the first path result of SL-AzimuthAoA. Either one of sl-AzimuthAoA-Result and sl-ZenithAoA-Result or both shall be present in SL-AOA-MeasElement.

sl-AzimuthAoA-LCS-GCS-Translation This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TimeStamp This field specifies the time instance at which the SL-PRS RSRP (if included) measurement is performed.

sl-ZenithAoA-Result This field specifies the first path result of SL-ZenithAoA. Either one of sl-AzimuthAoA-Result and sl-ZenithAoA-Result or both shall be present in SL-AOA-MeasElement.

sl-ZenithAoA-LCS-GCS-Translation This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

Annex B: Reply LS to RAN4

3GPP TSG-RAN WG2 Meeting #126	R2-240

# Fukuoka, Japan, 20th - 24th May., 2024

Title:	LS on SL positioning measurements

Response to:	R4-2406386

Release:	Release 18

Work Item:	NR_pos_enh2

Source:	Huawei, HiSilicon [To be TSG RAN WG2]

To:	TSG RAN WG4

Cc:

Contact Person:

#### Name:	Yinghao Guo

E-mail Address:	yinghaoguo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN4 for the LS regarding SLPP measurement for SL-AOA. During the discussion in RAN2#126, the following agreement was reached in RAN2:

- Specify in the field description of SL-AOA measurement that either one of Azimuth and Zenith results or both shall be present in SL-AOA-MeasElement.

- Reply to RAN4 that it is not possible for SL-AOA measurement to report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning.

2. Actions:

To RAN4:

ACTION: 	RAN2 respectfully asks RAN4 to take the above RAN2 agreement into account.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#127	from 2024-08-19	to 2024-08-23		Maastricht, NL

RAN2#127bis	from 2024-10-14	to 2024-10-18		China (TBC)