3GPP TSG-RAN WG2 Meeting #126	R2-2405169

Fukuoka, Japan May 20th – 24th, 2024

Agenda item:	7.7.3

Source:	Samsung

Title:	Discussion on E302, H115

WID/SID:	NR_NTN_enh -Core

Document for:	Discussion and Decision

# Introduction

In this document, we share view on RIL E302 and H115.

# Discussion

## E302

MeasObjectNR ::=                    SEQUENCE {

…

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                    OPTIONAL    -- Cond NeighbourCell

]]

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Need R

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17

}

| NeighbourCell | This field is mandatory present if this MeasObject is configured by the serving cell for a neighbour cell served by a NTN Earth moving system. Otherwise, it is absent. |
| --- | --- |

In our view, it is beneficial to allow NW optionally include neighbor cell ephemeris in MO even when CondEventD2/EventD2 is not configured, for example for the purpose of RRM. It would be expensive for NW to broadcast all neighbor cells’ ephemeris in SIB19.

Proposal 1: Agree E302.

## H115

We share same view that only SIB19 needs to be reacquired after satellite switch with resync. If any other SIBs are changed, NW should indicate a notification of SIB update and UE follows legacy SIB modification procedure to reacquire the updated SIBs. This is already included above as “upon receiving an indication that the system information has changed”.

Proposal 2: Agree H115.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: Agree E302.

Proposal 2: Agree H115.

# References

[1] R2-2403733	Report from Break-Out Session on NR NTN and IoT NTN	Session chair (ZTE)	Report