3GPP TSG RAN WG2#126	R2-2404110

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 #116-bis	R1-2403619

Changsha, Hunan Province, China, April 15th – 19th, 2024

Title:	Reply LS on Parallel Tx Capability

Response to:	R2-2401741 (R1-2401943)

Release:	Rel-17

Work Item:	TEI17, NR_newRAT-Core,NR_2step_RACH, NR_IIOT_URLLC_enh

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Xianghui Han

Email Address:	han.xianghui@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN1 thanks RAN2 for their LS (R2-2401741/R1-2401943) on parallel Tx capability.

RAN1 has discussed the questions in the RAN2 LS and would like to reply as below.

Q1: For the application of “Simultaneous PUSCH and PUCCH transmissions of same priority on different inter-band cells” capability, which understanding is aligned with RAN1’s intention? - Understanding #1: This capability can also be applied to the NR-DC band combination with the inter-band CA operation on the MCG/SCG; - Understanding #2: This capability can only be applied to the inter-band CA band combination

RAN1’s answer to Q1: From RAN1 specification point of view, this capability can also be applied to the NR-DC band combination with the inter-band CA operation on the MCG/SCG.

Q2: Whether Features listed in the Table 1 follow understanding #1 or understanding #2? Table 1: Parallel Tx Capability with Inter-band CA FG Index; Ran1 Description in the Feature list Table; Current Ran2 Field description. 4-25(R15); Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA; parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination.. 4-26(R15); Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA; parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination.. 9-3(R16); Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell; parallelTxMsgA-SRS-PUCCH-PUSCH-r16 Indicates whether the UE supports parallel transmission of MsgA and SRS/ PUCCH/ PUSCH across CCs in an inter-band CA band combination. A UE supporting this feature shall also indicate support of parallelTxPRACH-SRS-PUCCH-PUSCH.. 25-18(R17); Support simultaneous PUCCH and PUSCH transmissions of different priority on different cells for inter-band CA.; parallelTxPUCCH-PUSCH-r17 Indicates whether the UE supports simultaneous PUCCH and PUSCH transmissions of different priority on different cells for inter-band CA..

RAN1’s answer to Q2: From RAN1 specification point of view, the UE features listed in the Table 1 can also be applied to the NR-DC band combination with the inter-band CA operation on the MCG/SCG.

Q3: Whether Features listed in the Table 2 can also be applied to the NR-DC band combination with the intra-band non-contiguous CA operation on the MCG/SCG or only to intra-band non-contiguous CA band combination? Table 2: Parallel Tx Capability with Intra-band non-contiguous CA FG Index; Ran1 Description in the Feature list Table; Current Ran2 Field description. 39-2(R17); Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA; parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an intra-band non-contiguous CA band combination.. 39-4(R17); Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA(Prerequisite 9-3); parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of MsgA and SRS/ PUCCH/ PUSCH across CCs in an intra-band non-contiguous CA band combination. The UE indicating support of this field shall also indicate support of parallelTxMsgA-SRS-PUCCH-PUSCH-r16 and parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17.. 39-1(R17); Parallel SRS and PUCCH/PUSCH transmission across CCs in intra-band non-contiguous CA; parallelTxSRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an intra-band non-contiguous CA band combination..

RAN1’s answer to Q3: From RAN1 specification point of view, the UE features listed in the Table 2 can also be applied to the NR-DC band combination with the intra-band non-contiguous CA operation on the MCG/SCG.

# 2. Actions:

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above responses into account in their further work.

# 3. Date of Next TSG-WG1 Meetings:

TSG RAN WG1 Meeting #117	20th May– 24th May, 2024	Fukuoka, Japan

TSG RAN WG1 Meeting #118	19th August– 23rd August, 2024	Maastricht, NL