3GPP TSG-RAN WG2 Meeting #127												R2-2406480

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.7.1.1

Source: 	Sharp

Title:  	Discussion on XRM and UL PSI-based PDU Discard

Document for: 	Discussion

# Introduction

This document discusses RAN2 aspects of LSes from SA2 and RAN3 [1,2].

# Discussion:

## XRM

The first question for XRM LS [1] is about dynamic in-band signalling of traffic periodicity. The scenario assumes that the periodicity may change dynamically, and gNB may need to prepare for scheduling of data with dynamically changing periodicity. The scenario may be related to the efficiency of SPS. Without this information, configuration of SPS might be either inefficient or never used. If the scenario is valid, this information will be useful for gNB resource scheduling.

RAN2 to respond that the indication of periodicity via in-band signalling is useful for RAN resource scheduling.

The second question is about the time to next burst. In RAN2#126 meeting, RAN2 sent an LS to SA2 on data burst size indication from the CN [3]. The LS captures that RAN2 understands the size of incoming burst is useful for gNB resource scheduling if it can be provided early enough. The burst size information will be more useful for the resource scheduling, when the arrival timing information is combined. Especially, in case that the network experiences congestion and jitter, it may not be easy to estimate the data arrival time at gNB. Based on the timing information, resource reservation and estimation of future resource usage are possible.

RAN2 to respond that the time to next burst is useful for RAN resource scheduling.

## UL PSI-based PDU Discard

There is one question on UL PSI-based PDU discard in NR-DC split bearer. For a split bearer, the UE may receive a PSI-Based SDU Discard Activation/Deactivation MAC CE on both MCG MAC and SCG MAC. The question is how the UE reacts. According to Rel-18 MAC specification, no special handling is captured. A split bearer is associated with both MAC entities, any MAC CE can indicate the activation/deactivation of this function. If the UE receives different MAC CEs from MCG MAC and SCG MAC, the UE always follows the latest MAC CE regardless of the MAC entity.

This scenario is quite similar to Rel-15 DC duplication, i.e., PDCP duplication via both MCG and SCG. UE may receive Duplication Activation/Deactivation MAC CE from both MCG and SCG. UE follows the latest MAC CE without special handling. It is totally up to the network implementation whether or how to coordinate activation/deactivation.

RAN2 to confirm and respond that the UE follows latest PSI-based SDU Discard Activation/Deactivation MAC CE for split bearers, similar to Duplication Activation/Deactivation MAC CE.

# Conclusions

Based on the discussion given above, we have the following proposals:

- RAN2 to respond that the indication of periodicity via in-band signalling is useful for RAN resource scheduling.

RAN2 to respond that the time to next burst is useful for RAN resource scheduling.

- RAN2 to confirm and respond that the UE follows latest PSI-based SDU Discard Activation/Deactivation MAC CE for split bearers, similar to Duplication Activation/Deactivation MAC CE.

# Reference

[1] S2-2407351, LS on FS_XRM Ph2, SA2

[2] R3-243957, LS on UL PSI based PDU discarding in NR-DC, RAN3

[3] R2-2405779, Reply LS to SA2 on FS_XRM PH2, RAN2