3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500080

Athens, Greece, Februsry 17th – 21st, 2025                                 Revision of R2-2409538

Agenda item:	8.8.4

Source:	vivo

Title:	Discussion on MBS Broadcast Service Continuity in NTN

Document for:	Discussion and Decision

- Introduction

In the past RAN2 meetings, the discussion on MBS broadcast service continuity in NTN case with intended service area was launched. No conclusion/agreement was made in the end [1] [2].

In this contribution, we would like to provide our consideration of broadcast service continuity in NTN.

- Discussion

In the existing MBS broadcast framework, for service continuity, the IDLE/INACTIVE UE may consider the frequency that providing the MBS broadcast service the UE interested in to be the highest priority or consider the frequency that not providing the MBS broadcast service to be the lowest priority. Additionally, the IDLE/INACTIVE UE may request unicast reception (thus triggering RRC setup/resume request procedure) of the broadcast service content before moving to a cell not providing the MBS broadcast service(s) via PTM mode, according to the neighbour cell list in MCCH. Regarding CONNECTED UE, it may send the MBS Interest Indication to the gNB, including the information on MBS frequencies/services UE is receiving or interested to receive and the priority between broadcast and unicast/multicast. With that the gNB may provide the UE with an RRC reconfiguration, select a proper target cell, and deprioritize unicast/multicast scheduling, to allow the UE to receive the MBS services the UE is interested in.

To guarantee service continuity in NTN case with the intended service area, it is worth investigating whether the existing solution is sufficient or not. For the frequency prioritization function of MBS broadcast, the following UE behaviours are captured in TS 38.304 and TS 38.300, respectively,

TS 38.304 sub-clasuse 5.2.4.1 Reselection priorities handling -------------------------<the unrelated part is omitted>----------------------- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell. -------------------------<the unrelated part is omitted>----------------------- TS 38.300 sub-clasuse 16.10.6.5.1 Service Continuity in RRC_IDLE or RRC_INACTIVE -------------------------<the unrelated part is omitted>----------------------- In RRC_IDLE and RRC_INACTIVE, the UE applies the normal cell reselection rules with the following modifications: - the UE which is receiving or interested to receive MBS broadcast service(s) via PTM and can only receive these MBS broadcast service(s) via PTM while camping on the frequency providing these MBS broadcast service(s) is allowed to make this frequency highest priority when the conditions described in TS 38.304 [10] are met; - when the MBS broadcast service(s) which the UE is interested in are no longer available (after the end of the session) or the UE is no longer interested in receiving the service(s), the UE no longer prioritises the frequency providing these MBS broadcast service(s).

In the Rel-19 case, it seems a spontaneous logic that the UE may set one frequency associated with broadcast service provision with the highest frequency priority only when it will be in the intended service area after the cell reselection. Otherwise, there is no need to do this as the service, even though the user is interested in that, is not intended to be received. Specifically, if UE is receiving or interested in receiving an MBS broadcast service associated with the intended service area and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session when its location is within the intended service area of the corresponding session after cell reselection. And once the UE goes outside of the intended service area, the UE should no longer prioritize the frequency that providing the MBS broadcast service. In addition, to facilitate the above frequency prioritization behaviour, SIB21 should be allowed to include the service area information so that the UE can obtain the intended service area of the interested broadcast service that is not provided in the current cell.

Proposal 1: SIB21 can include a service area ID list for broadcast service continuity.

Proposal 2: UE may consider the frequency that providing the broadcast service to be the highest priority only when the UE is located within the service area of that interested broadcast service.

Proposal 3: UE no longer prioritizes the frequency that providing the MBS broadcast service when the UE is outside the service area of that interested broadcast service.

For the unicast reception request for the broadcast service content, since it is up to UE implementation to do this without stage-3 spec normative text, we think we can leave it to pure UE implementation in the Rel-19 case. For example, when the UE is moving to a cell not providing the MBS broadcast service and will continue to stay within the intended service area, it may trigger an RRC setup/resume request for the unicast reception of the broadcast service content. No further enhancement is needed for the neighbour cell list solution.

Proposal 4: No enhancement is considered for the MCCH neighbour cell list mechanism in Rel-19.

Last but not least, we would like to look at the MBS Interest Indication (MII) reporting mechanism. In TS 38.331, the following MBS Interest Indication reporting behaviour is specified for MBS broadcast,

5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

We can observe that the RRC_CONNECTED UE may initiate the MII reporting procedure upon entering or leaving the broadcast service area (i.e. cell/tracking area) based on the USD, as highlighted above. We think the same technical logic can be applicable to the Rel-19 case. That is the UE may report MII when entering or leaving the intended service area, as shown in Figure 1.

Specifically, for CONNECTED UE, MII can be used for scheduling prioritization handling between broadcast and unicast/multicast (e.g. if the UE prioritizes broadcast service(s) over unicast/multicast, the network may release some DRBs/multicast MRBs to save more radio bearer capability for broadcast reception). Then the CONNECTED UE should report it to the network when it goes inside the intended service area so that the network can do the proper scheduling prioritization handling. Furthermore, if the UE goes outside the intended service area, the UE should also indicate the MII since the network has to know it does not need to consider scheduling prioritization handling between broadcast and unicast/multicast anymore for this UE.

Figure 1: MII reporting when moving inside/outside the intended service area

Based on the above, we propose,

Proposal 5: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area indicated by AS signalling.

3 Conclusion

In this contribution, we have further discussed broadcast service continuity solutions in NTN. And we make the following proposals:

Proposal 1: SIB21 can include a service area ID list for broadcast service continuity.

Proposal 2: UE may consider the frequency that providing the broadcast service to be the highest priority only when the UE is located within the service area of that interested broadcast service.

Proposal 3: UE no longer prioritizes the frequency that providing the MBS broadcast service when the UE is outside the service area of that interested broadcast service.

Proposal 4: No enhancement is considered for the MCCH neighbour cell list mechanism in Rel-19.

Proposal 5: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area indicated by AS signalling.

4 Reference

[1] R2-2409502, RAN2#127bis Meeting Report.

[2] R2-2500002, RAN2#128 Meeting Report.