3GPP TSG-RAN WG2 Meeting #129                                                               R2-2500082

Athens, Greece, February 17th– 21st, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Further Discussion on S&F Operation

Document for:	Discussion and Decision

- Introduction

The S&F operation issue was further discussed in RAN2#128 [1], and the following agreements were made.

1. Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start) 2. It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable 3. “S&F operation” indication is provided in SIB1 4. RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN). 5. RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation 6. RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite 7. RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

However, there are some open issues as we highlight in yellow. Besides, some companies propose to introduce the S&F cell lists in SIB4 and SIB5 to assist UE measurements and cell reselection [2], this issue was discussed but still remains unsolved.

In the contribution, we will analyse all these issues and give our view.

- Time information between S&F mode and normal mode transition;

- FFS new cause value within RRC Release upon feeder link is unavailable;

- FFS introduce S&F cell list for cell reselection.

- Discussion

## 2.1 Time information between S&F mode and normal mode transition

Time information from S&F mode to normal mode

As we listed above, RAN2 already agreed to support to indicate the time info from S&F mode to normal mode. However, it is unclear which SIB the information is included in and what format the time info is. From our perspective, since the S&F mode or the normal mode is specific for NTN cells, thus the SIB31 is mandatory. Further, according to current spec, SIB31 is specific for satellite-specific assistance information, thus it is straightforward to include the new time info from S&F mode to normal mode in the SIB31.

Proposal 1:Time information from S&F mode to normal mode is included in SIB31.

Moreover, similar to the T-service/ t-ServiceStartNeigh or the time-based CHO, the time information can be designed as the absolute time, i.e., the UTC time information [3]. The absolute time format is simple, and there is no ambiguity in the reference time.

#### – TimeOffsetUTC The IE TimeOffsetUTC provides the time offset to the beginning of week (Monday 00:00:00 UTC). Units in seconds. TimeOffsetUTC information element -- ASN1START TimeOffsetUTC-r17 ::= INTEGER (0..1048575) -- ASN1STOP

Proposal 2: Time information from S&F mode to normal mode is expressed as the absolute time on when the feeder link begins available, i.e., the UTC time.

From RAN2 perspective, after UE acquired the time information from S&F mode to normal mode, UE can forward the time information to the NAS layer, since the time information may be useful for the NAS procedure, just as the following assumption RAN2 made in RAN2#128.

RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed

However, whether such time information is needed for the NAS procedure, and how the NAS layer uses the time information is within CT1's scope, and also CT1 is discussing the issue. Thus, it is proposed that RAN2 send LS to CT1 to ask them whether UE need to forward the time information from S&F mode to normal mode to the NAS layer .

Proposal 3: RAN2 send LS to CT1 to ask whether UE need to forward the time information from S&F mode to normal mode to the NAS layer.

Time information from normal mode to S&F mode

If the cell is currently working on regeneration payload mode, both the S&F capable UEs and the S&F not capable UEs may work in the cell. Then, when the cell changes its operation mode from regeneration payload to S&F mode, UEs without S&F capability may not work well in the cell. If S&F not capable UE is in RRC CONNECTED, it can rely on NW implementation to handle these UEs, i.e., NW can trigger the handover of the UEs to cells in regeneration payload mode or release the UE. For S&F not capable UE in RRC IDLE, it is beneficial if UEs can camp on other cells before the cell changes to S&F mode. To enable this, we think the simple way is to reuse the new S&F operation indication, i.e., up to NW implementation, NW indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation, then the legacy UEs and R19 UEs not supporting the S&F operation consider the cell is barred and perform the cell reselection, while the S&F capable UE can continue work in the cell.

Observation 1: For S&F not capable UE in RRC CONNECTED state, NW can release the UE or the trigger handover to a cell in regeneration payload mode before the serving cell changes from regeneration payload mode to S&F mode.

Observation 2: For S&F not capable UE in RRC IDLE state, NW can indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation.

Proposal 4: Up to NW implementation, NW can enable the S&F not capable UE camps/works in a regeneration payload mode cell before the serving cell changes from regeneration payload mode to S&F mode, no further spec is foreseen.

Proposal 5: No additional information except the S&F operation indication needs to be provided by NW.

## 2.2 FFS new cause value within RRC Release upon feeder link is unavailable

If the cell is in normal mode upon UE access to the network, while later the feeder link is not available, i.e., the S&F mode, the network may release the UE to IDLE mode. Once the network gives UE the information that the release is due to the S&F operation, it will be beneficial for UE to perform cell reselection if the UE initiates the non-delay tolerant service.

Observation 3: There are benefits foreseen if NW indicates to UE the release is due to the cell transfer to S&F mode, i.e., UE may not reselect the cell if UE initiates the non-delay tolerant service.

Proposal 6: A new cause value “S&F” is indicated by the network if the RRC Connection release is due to the cell changes from normal mode to S&F mode.

After UE acquires the new cause value within the RRC Connection release message, UE can deliver the information to the NAS layer to assist the NAS layer procedure. Besides, from the AS layer perspective, the corresponding UE behaviour is UE may not reselect the cell, but we think this can just be left to UE implementation to realize.

Proposal 7: After UE acquire the new cause value “S&F”, UE forward the information to the NAS layer.

Proposal 8: How to avoid UE reselecting the S&F cell upon initiating the non-delay tolerant service based on the new cause value is up to UE implementation.

## 2.3 FFS introduce S&F cell list for cell reselection

According to [2[, introducing the S&F cell lists in SIB4 and SIB5 may be beneficial for UE to prioritise the S&F cell upon performing cell reselection. From our perspective, introducing such S&F cell list means the network has to change the SIB4/SIB5 frequently especially. Besides, there is also unclear motivation on why UE need to prioritise the S&F cell the S&F cell since the normal mode cell can provide better service. Finally, how to prioritise the S&F cell upon cell reselection can also depend on implementation. Based on these, there is no need to introduce such S&F cell lists for cell reselection.

Proposal 9:  No enhancements on cell selection/reselection specific for S&F is needed.

- Conclusion

In this contribution, we have further discussed the S&F operation. And we make the following observations and proposals:

Time information between S&F mode and normal mode transition

Observation 1: For S&F not capable UE in RRC CONNECTED state, NW can release the UE or the trigger handover to a cell in regeneration payload mode before the serving cell changes from regeneration payload mode to S&F mode.

Observation 2: For S&F not capable UE in RRC IDLE state, NW can indicate the S&F operation indication with the setting “0” and set the legacy NTN barred indication with the value “barred” before the cell changes to S&F mode operation.

Proposal 1:Time information from S&F mode to normal mode is included in SIB31.

Proposal 2: Time information from S&F mode to normal mode is expressed as the absolute time on when the feeder link begins available, i.e., the UTC time.

Proposal 3: RAN2 send LS to CT1 to ask whether UE need to forward the time information from S&F mode to normal mode to the NAS layer.

Proposal 4: Up to NW implementation, NW can enable the S&F not capable UE camps/works in a regeneration payload mode cell before the serving cell changes from regeneration payload mode to S&F mode, no further spec is foreseen.

Proposal 5: No additional information except the S&F operation indication needs to be provided by NW.

FFS new cause value within RRC Release upon feeder link is unavailable

Observation 3: There are benefits foreseen if NW indicates to UE the release is due to the cell transfer to S&F mode, i.e., UE may not reselect the cell if UE initiates the non-delay tolerant service.

Proposal 6: A new cause value “S&F” is indicated by the network if the RRC Connection release is due to the cell changes from normal mode to S&F mode.

Proposal 7: After UE acquire the new cause value “S&F”, UE forward the information to the NAS layer.

Proposal 8: How to avoid UE reselecting the S&F cell upon initiating the non-delay tolerant service based on the new cause value is up to UE implementation.

FFS to introduce S&F cell list for cell reselection

Proposal 9:  No enhancements on cell selection/reselection specific for S&F is needed.

- Reference

- Draft_RAN2_128_Meeting_Report_v1.

- R2-2409585, Discussion on Store and Forward operation, Xiaomi.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.