3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500083

Athens, Greece, February 17th – 21st , 2025

Agenda item:	8.9.3

Source:	vivo

Title:	Discussion on CB-Msg3 Mechanism

Document for:	Discussion and Decision

1 Introduction

In the last RAN2 meetings, RAN2 had discussed several general aspects on the CB-Msg3 mechanism and had achieved the following agreements [1]:

RAN2#128 Agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B). - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels. - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels). - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC). - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific. - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts). - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time. - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs). - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4). - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4). - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started).

In this contribution, we will further discuss the CB-Msg3 mechanism used for IoT-NTN.

2 Discussion

In the following, we will give our analysis of the RAN1/RAN2 spec impacts for supporting CB-Msg3.

  - 2.1 CB-Msg3 configuration & triggering

In the previous, there is an FFS that if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B. In our understanding, to support CB-Msg3 transmission in 15kHz SCS NB-IoT and eMTC CE mode B, TA validation based on timer solution is needed for performance robustness (i.e. guaranteeing that the UE transmits the CB-Msg3 in the same cell where it is released from CONNECTED to IDLE). If so, the corresponding RAN4 requirement is needed for the NTN case. However, currently,  RAN4 is only discussing the TA validation requirement for CG-SDT for RedCap UE in NR-NTN. No NB-IoT specific requirement can be defined in Rel-19. Thus, we think CB-Msg3 transmission in 15kHz SCS NB-IoT or eMTC CE mode B should be deprioritized considering the performance gain can not be guaranteed.

Proposal 1: RAN2 deprioritizes the CB-Msg3 in 15kHz SCS NB-IoT or eMTC CE mode B cases.

In the LS R2-2500010, it is confirmed that OCC2 can be applied to CB-Msg3. Then for CB-Msg3, we have to consider whether DSA and OCC2 can be combined together. From the technical point of view, it is feasible and beneficial for UL throughput enhancement. So we think RAN2 should support DSA and OCC2 combination. Moreover, for resource selection simplicity, we would like to say the SIB should configure only one CB-Msg3 resource configuration. And that single resource configuration can be SA, or DSA, or OCC2, or DSA with OCC2. Otherwise, if SIB includes multiple CB-Msg3 resource configurations, then the UE has to determine which resource should be prioritized (e.g. the UE has to select one resource amongst the resource for SA, the resource for OCC2, and the resource for DSA) or support parallel transmission amongst different resources. It is quite complicated.

Proposal 2: SIB only includes at most one CB-Msg3 resource configuration.

Proposal 3: From RAN2 perspective, DSA and OCC2 can be combined.

Next, we would like to provide our understanding of the CE-related configuration setting, regarding the following,

- FFS on the number of levels.

- FFS if the maximum TBS is same or different for different CE levels.

Since the link budget of NPRACH/Msg3 is obviously different than the CB-Msg3 as TBS and channel type are different, we think it is a spontaneous logic to have a separate RSRP threshold for CE level selection for CB-Msg3. Specifically, we can first assume the same maximum number of CE levels is 2 considering CB-Msg3 is intended for small data transmission. It is not suitable for the case belonging to CE level 3 (basically for extremely poor-condition cases). Then separate RSRP thresholds for CB-Msg3 can be configured independently of the RACH configuration. An example of CB-Msg3 CE configuration is given below,

Figure 1: CB-Msg3 CE configuraion

Further, the maximum TBS should be separately configured for different CB-Msg3 CE level, similar to the existing EDT framework, as quoted below. It is obvious that the NW should have the flexibility to control different max TBS of different CE levels, so that the UL transmission can be successfully decoded.

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

Proposal 4: CE level 0, CE level 1, and CE level 2 are supported for CB-Msg3.

Proposal 5: From RAN2 perspective, DSA and OCC2 can be combined.

Proposal 6: The maximum TBS can be separately configured for different CE levels

Regarding the OCC2-related configuration, in our view, the NW should indicate an RSRP range per CE level. To facilitate OCC performance, the network should guarantee the UEs using OCC2 should have a similar pathloss status (so that the received power from different UEs is generally at the very close level, i.e., power balance) and the frequency offset error of those UEs should be quite close and smaller than the requirement as well. From the UE point of view, as long as the measured RSRP is within the configured range, then the UE may select to use the Cn-Msg3 with OCC2.

Proposal 7: For CB-Msg3 with OCC, a RSRP range is indicated on CE level basis.

Proposal 8: The UE can only trigger CB-Msg3 with OCC when the measured RSRP is within the RSRP range of the selected CE level.

Further, the eNB should configure the CB-Msg3 PUSCH in a periodic manner (similar way as NPRACH/DPUR) so that there soon will be a next available CB-Msg3 PUSCH for the UE with unpredictable UL bursty data. Another option is that the CB-Msg3 PUSCH resources are only valid for one shot (similar to one-shot DPUR) or for N shots. As the CB Msg3 PUSCH is shared amongst multiple UEs, resource waste might be not a big issue. So we may not consider one-shot or N-shot configuration for Rel-19 CB-Msg3 PUSCH resources. Aside from time domain resource allocation, configuration in other aspects, such as frequency and reference signal for PUSCH, should be considered as well. In our understanding, from PHY transmission point of view, the point-to-point uplink transmission condition and DL response reception are similar to the DPUR UL/DL transmission. Thus we assume it is simplest to reuse DPUR allocation configuration for CB-Msg3 configuration as a baseline. Meanwhile, for the coverage enhancement aspect, we think how to associate the maximum supported TBS with the RSRP threshold per CE level is totally up to NW implementation. And for simplexity, RAN2 should assume the existing TBS framework from DPUR is reused for CB-Msg3. That is it is up to network implementation to configure a suitable TBS for a CB-Msg3 resource linked with a CE level configuration. The configurable parameters should be inherited from the existing parameters designed for DPUR (e.g. possible TBS size) or EDT (e.g. RSRP threshold per CE level).

Proposal 9: Taking the UL/DL resources related configuration fields in PUR-Config-r16 as a baseline reference for CB-Msg3resources configuration.

  - 2.2 DSA modeling in MAC:

After CB-Msg3 occasion selection, the UE should do UL-SCH data transfer and the actual transport block transmission via the physical layer. The main preparation steps for the PUSCH transmission are given as follows:

- The MAC entity receives the UL grants;

- The MAC entity delivers the UL grants and the associated HARQ information to the HARQ entity;

- The HARQ entity instructs the multiplexing and assembly entity to generate a MAC PDU;

- The HARQ entity obtains the MAC PDU to transmit from the multiplexing and assembly entity;

- The HARQ entity triggers the corresponding HARQ process to perform a new transmission/retransmission;

- The corresponding HARQ process	instructs the PHY layer to generate a transmission;

- The PHY layer transmits the PUSCH for the obtained MAC PDU with PHY procedure (e.g. power control).

To minimize the potential RAN1 impacts with the introduction, we think each replica should be modeled as a new transmission. On one hand, since the network side cannot perform soft combining for DSA replicas due to the lack of information on which occasions the same UE selects, there is no need to model it as a retransmission. On the other hand, if we treat the replica as a retransmission, we have to consult with RAN1 whether the same transmit power can be reused for this retransmission and what RV index should be considered for it, and so on.

Further, by treating the replica transmission as a new transmission, then the MAC should generate a MAC PDU for each replica transmission. As the content in each replica should be the same, a possible and simple modeling is to obtain the MAC PDU from the HARQ buffer for the 2nd replica. At the physical layer, it can treat each replica transmission as the new transmission as the legacy case (e.g. independently doing the power control, resource mapping, etc), as depicted in Figure 3 below. Moreover, we think that HARQ process id 0 and RV0 can be reused, similar to Msg3 transmission.

Figure 2: DSA replica transmission

Proposal 10: MAC considers each replica transmission as a new transmission.

Proposal 11: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 12: HARQ process id 0 and RV 0 are used for CB-Msg3.

  - 2.3 Response reception:

After the CB-Msg3 transmission, the UE needs to start a PDCCH monitoring window for response reception. For DSA case, in the previous meeting, a majority of companies thought that PDCCH monitoring should be started after every first replica transmission so that the UE can monitor the response and potentially discard the remaining replica transmission for power saving and avoid necessary CB-Msg3 collision. As different occasions for different replicas may be associated with different RNTI values, for simplicity, multiple response windows can be started to monitor the corresponding DL response associated with different RNTIs, as shown in the figure below. The UE, by implementation based on its capability on PDCCH reception, decides which PDCCH associated with an RNTI to be monitoring when there are multiple windows are overlapping.

If no matched responses are received while all the response windows have expired, the UE should attempt the next CB-Msg3 transmission, selecting the CB-Msg3 resources again.

Figure 3: PDCCH monitoring for DSA replica transmission

Proposal 13: The UE starts a PDCCH monitoring window (associated with one RNTI) for each replica transmission, with the same starting point as the SA case.

Proposal 14: If no matched responses are received while all the response windows have expired, the UE attempts to perform the next CB-Msg3 transmission.

  - 2.4 Backoff:

As analyzed in our paper RR2-2408018, the PLA performance gain difference between SA and SA-BI is quite obvious. Thus the BI-reattempt (like PRACH) solution should be adopted for collision randomization. Regardless of which access mode (e.g. DSA, SA, OCC2) is adopted, a common backoff mechanism should be supported.

Proposal 15: Support re-attempt after backoff time (like PRACH) for CB-Msg3.

3 Conclusion

In this contribution, we have provided our understanding of CB-Msg3 procedure. And we made the following proposals:

CB-Msg3 configuration & triggering :

Proposal 1: RAN2 deprioritizes the CB-Msg3 in 15kHz SCS NB-IoT or eMTC CE mode B cases.

Proposal 2: SIB only includes at most one CB-Msg3 resource configuration.

Proposal 3: From RAN2 perspective, DSA and OCC2 can be combined.

Proposal 4: CE level 0, CE level 1, and CE level 2 are supported for CB-Msg3.

Proposal 5: From RAN2 perspective, DSA and OCC2 can be combined.

Proposal 6: The maximum TBS can be separately configured for different CE levels.

Proposal 7: For CB-Msg3 with OCC, a RSRP range is indicated on CE level basis.

Proposal 8: The UE can only trigger CB-Msg3 with OCC when the measured RSRP is within the RSRP range of the selected CE level.

Proposal 9: Taking the UL/DL resources related configuration fields in PUR-Config-r16 as a baseline reference for CB-Msg3resources configuration.

DSA modelling in MAC:

Proposal 10: MAC considers each replica transmission as a new transmission.

Proposal 11: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 12: HARQ process id 0 and RV 0 are used for CB-Msg3.

Response reception:

Proposal 13: The UE starts a PDCCH monitoring window (associated with one RNTI) for each replica transmission, with the same starting point as the SA case.

Proposal 14: If no matched responses are received while all the response windows have expired, the UE attempts to perform the next CB-Msg3 transmission.

Response reception:

Proposal 15: Support re-attempt after backoff time (like PRACH) for CB-Msg3.

4 Reference

[1] R2-2500002, RAN2#128 Meeting Report.