3GPP TSG-RAN WG2 Meeting #129	R2-2500088

Athens, Greece, 17th – 21st February 2025

Agenda Item		:	8.2.2 (Ambient_IoT_solutions)

Source		:	LG Electronics Inc.

Title		:	Discussion on ambient IoT paging

Document for	:	Discussion and Decision

1.	Introduction

In WID [1], RAN2 aims to standardize the RAN aspects of Ambient IoT (A-IoT or AIoT) with the following objectives. In this contribution, we show our views on the A-IoT paging.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

2.	Discussion

In RAN2#127 meeting [2], RAN2 agreed to support a mechanism that allows the reader to trigger multiple/subsequent A-IoT paging messages associated with the same request from the CN. Since the reader triggers multiple A-IoT paging messages for the same request, there is a possibility of duplicate response from A-IoT devices. To prevent this, in RAN2#127bis meeting [3], RAN2 agreed that the A-IoT paging message could include information to avoid duplicate responses from devices to the reader. Then, in RAN2#128 meeting [4], RAN2 agreed that an ID in paging message is beneficial in preventing duplicated response. The table below summarizes the abovementioned agreements from the three meetings. Here we further discuss how to design this information or ID.

Agreement - RAN2#127 - Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices Agreement - RAN2#127bis - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. Agreement - RAN2#128 - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size

As agreed in RAN2#127bis, based on this information or ID, an A-IoT device determines whether to send a response to the paging message or not. Specifically, the A-IoT device responds to an A-IoT paging message if the message is associated with a service request that has not yet been completed. For example, an A-IoT device would respond and perform subsequent A-IoT procedures if the paging message is associated with a new service request. If the paging message is a subsequent message of a previously received one whose associated service has not yet been completed, the A-IoT device would respond and complete the remaining A-IoT procedures (hereafter referred to as ‘re-access’). Conversely, the A-IoT device does not respond if the paging message is associated with a service request that has already been successfully completed.

In short, the key point is that, based on the information or ID stored in the A-IoT paging message, the A-IoT device should be able to determine whether the A-IoT paging message is associated with an A-IoT service that it has already completed or if it pertains to an incomplete or new service. Regarding this information or ID, during the discussion in RAN2#127bis meeting, two options were proposed as follows.

- Option 1: A transaction ID is included in an A-IoT paging message, i.e., multi-bit indication.

  - The transaction ID is increased when the reader receives a new service request for the same service from the CN. For subsequent A-IoT paging messages related to the same service request, the transaction ID remains unchanged. The A-IoT device can perform the re-access procedure if the transaction ID is not changed in the A-IoT paging message.

Figure 1. Example of Option 1: transaction ID

- Option 2: An NDI-like indication is included in an A-IoT paging message, i.e., one-bit indication.

  - The NDI-like indication is toggled when the reader receives a new service request for the same service from the CN. For subsequent A-IoT paging messages related to the same service request, the NDI-like indication remains unchanged. The A-IoT device can perform the re-access procedure if the NDI-like indication is not toggled in the A-IoT paging message.

Figure 2. Example of Option 2: NDI-like indication

The main difference between Option 1 and Option 2 lies in the use of a multi-bit indication versus a one-bit indication. In our view, both options may lead to an error where the A-IoT device mistakenly considers a new A-IoT paging message as a subsequent A-IoT paging message for the re-access procedure. The following figure illustrates the problematic case for each option.

Figure 3. Example of problematic case for each option

As shown in Fig.3, neither option can completely eliminate the error. However, the multi-bit indication can significantly reduce the probability of the error occurring compared to the one-bit indication. In addition, even if the multi-bit indication introduces additional signalling overhead for the A-IoT paging message, the overhead is not a big problem, as only a two-bit indication may be sufficient for the multi-bit indication. Therefore, we propose to introduce the multi-bit indication to avoid duplicate responses from the A-IoT devices.

Proposal 1: A-IoT paging message should include 2-bit ‘transaction ID’ field to avoid duplicated responses from A-IoT devices.

As the trigger for the entire A-IoT paging procedure, the A-IoT paging message includes the identifiers of target devices that need to respond to the paging message and perform the subsequent procedures. We need to discuss how the A-IoT paging message should store these device identifiers. Here, we would like to focus on the format of the device identifiers, specifically their length.

According to TR 22.840 [5], there are various use cases for A-IoT. Among them, some assume that the length of the device identifier could be variable. For example, in the use case for automated warehousing, the device identifier is expected to be 96 or 128 bits long. In the use case for intralogistics in automobile manufacturing, the device identifier is expected to be 96 or 240 bits. The EPC code, which is the identifier used in RFID, also has a variable length ranging from 96 to 496 bits.

As discussed above, considering the representative use cases, example cases from reference technologies (e.g., the EPC of RFID), as well as extendibility and forward compatibility, we believe it is natural to expect that the length of the device identifier used in the A-IoT system may be variable. That is, there could be different types of A-IoT devices identifiers with different lengths.

Observation 1: Considering the SA1 discussion and the example cases from reference technologies (e.g., RFID), the length of the device identifier used in the A-IoT system may vary. There could be different types of A-IoT device identifiers with different lengths.

Then, to enable A-IoT devices to read the A-IoT device identifier of variable length stored in the A-IoT paging message, a ‘type field’ for each A-IoT device identifier should be included in the A-IoT paging message, accompanying the related A-IoT device identifier. We therefore propose the following.

Proposal 2: Considering that the A-IoT device identifier may have various types with different lengths, the A-IoT paging message should include a ‘type field’ for each device identifier included.

Meanwhile, as outlined in WID, RAN2 aims to design a paging message format that allows multiple identifiers to be contained in a single paging message for forward compatibility purposes. In line with this, we propose the following Proposal 4. Additionally, to distinguish each identifier in multiple-identifier scenarios, we believe that the ‘type field’ for each identifier should also be included in the A-IoT paging message alongside the corresponding A-IoT device identifier.

Proposal 3: To ensure forward compatibility with scenarios where multiple device identities can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple device identities.

Proposal 3a: If a reserved field for multiple device identities is defined, the A-IoT paging message should include a ‘type field’ for each identifier stored in the field.

All A-IoT procedures begin with A-IoT paging, followed by the random access (RA) procedure. For the RA procedure, either contention-based random access (CBRA) or contention-free random access (CFRA) can be performed. In the case of CBRA, A-IoT devices need to know the number of access occasions to properly choose an access occasion. Since the A-IoT paging precedes and triggers the CBRA procedure, the most appropriate way to indicate the number of access occasions before the CBRA procedure begins is through the A-IoT paging message. Therefore, we propose the following in Proposal 4. We note that, even in the case of CFRA, the ‘number of access occasions’ field could be maintained with a specific value (e.g., zero or one) without requiring any changes to the A-IoT paging message format.

Proposal 4: A-IoT paging message should include a ‘number of access occasions’ field for the subsequent random access procedure, so that A-IoT devices are aware of the number of access occasions.

Regarding the random access type following the A-IoT paging message, we believe that the A-IoT device could implicitly determine the type of random access procedure to be performed. For example, A-IoT devices could use the number of access occasions stored in the A-IoT paging message. If the A-IoT paging message targets a single device and the number of access occasions is one, the target device will autonomously know that CFRA will take place. If the A-IoT paging message targets all devices and the number of access occasions is non-zero, the target devices will autonomously know that CBRA will be performed. If the A-IoT paging message includes multiple identifiers for target devices and the number of access occasions matches the number of target devices, then CFRA will be conducted. Given this, we prefer an implicit indication of the random access type, without introducing additional signaling or field to the A-IoT paging message.

Proposal 5: A-IoT paging message should not include subsequent random access procedure type (e.g., whether CBRA or CFRA); an implicit indication (e.g., based on the number of access occasions) is sufficient.

As summarized in the table below, RAN1 decided that the frequency domain multiple access (FDMA) of D2R transmission will be studied, while FDMA is not supported for R2D transmission [6][7].

Agreement – RAN1#116bis - Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS Agreement – RAN2#118bis - Capture in the TR that for R2D: - Time-domain multiplexing is the baseline. - Code-domain multiplexing is not considered for device 1/2a/2b. - Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1

Based on the above agreements, the following scenario can be considered.

Figure 4. Example scenario of FDMA applied to D2R transmission

As shown in Fig.4, if FDMA is applied, the A-IoT device needs to support the transmission of D2R, i.e., Msg1 transmission, on specific frequencies. In other words, the reader should specify the frequency for D2R transmission to the A-IoT device. We believe the simplest approach is to include a list of frequency information for D2R transmission in the A-IoT paging message. Then, when the A-IoT device receives the A-IoT paging message, the device can randomly select an access occasion and the frequency to transmit Msg1.

Proposal 6: A-IoT paging message should include a list of frequency information for D2R transmission, i.e., Msg1 transmission.

As captured in WID, RAN has decided on Topology 1 as the general scope. Notably, according to TR 22.840, even within Topology 1, there are use cases where A-IoT devices may be within the coverage of multiple readers (e.g., the flower auction scenario). In such cases, an A-IoT device located at the intersection of coverage areas of multiple readers could receive multiple A-IoT paging messages from different readers (hereafter referred to as the multiple-reader scenario).

The issues related to locating A-IoT devices, as captured in TR 38.769 [8], further motivate our study of multiple-reader scenarios. As outlined in TR 38.769, if the A-IoT CN sends a location-related command to multiple readers, an A-IoT device may receive multiple A-IoT paging messages associated with the command from different readers.

TR 38.769 A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported. NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future.

Considering the above discussion, RAN2 needs to consider the implications of the multiple-reader scenario.

Here, we briefly analyse the issues that could arise from the multiple-reader scenario. A situation may occur where the A-IoT device should decide whether to respond to only one paging message (e.g., the first) or to all paging messages from different readers. For example, during an inventory service, if a target device has never been inventoried, its location relative to the readers is unknown to the CN. In this case, to quickly locate the target device, the CN may request inventory service from all readers, prompting them to initiate the A-IoT paging procedure. In this scenario, it is sufficient for the A-IoT device to respond to the first received A-IoT paging message.

Conversely, the A-IoT device may need to respond to all A-IoT paging messages from different readers. As captured in TR 38.769, for accurate location purposes, it may be necessary to inventory a specific A-IoT device using multiple readers. In this scenario, the A-IoT device should respond to each A-IoT paging message from different readers.

However, it remains beneficial to avoid duplicate responses to the same reader while allowing multiple responses to different readers. To facilitate this, the A-IoT device should be able to distinguish the source of each A-IoT paging message, which requires the A-IoT paging message to include the reader identity of the sender. Additionally, to support selective multiple responses from A-IoT devices, the A-IoT paging message should include an indication informing whether the A-IoT paging is associated with a specific service requested by the CN, even if sent from different readers, and an indication of whether multiple responses are needed, or a single response is sufficient.

Considering the above discussion and the need for forward compatibility, we derive following observation and proposals.

Observation 2: In A-IoT systems, there could be scenarios where multiple readers transmit A-IoT paging messages for the same A-IoT service. In some cases, an A-IoT device may be required to respond to each of these paging messages, while in other cases, the device may only need to respond to one of the messages (e.g., the first one received).

Proposal 7: For forward compatibility with multiple-reader scenarios and to enable A-IoT devices to operate properly, A-IoT paging message should include a reserved field that could be used to accommodate the reader ID of the sender of the A-IoT paging message.

Proposal 8: To ensure proper operation of A-IoT devices in multiple-reader scenarios, the A-IoT paging message should include a reserved field that could be used to accommodate a 'service ID.' This service ID indicates which A-IoT service, requested by CN, is associated with the paging messages, regardless of which reader transmits them.

Proposal 9: To enable A-IoT devices to determine whether to respond to each A-IoT paging message or only one A-IoT paging message in multiple-reader scenarios, A-IoT paging message should include a reserved field that could be used to accommodate an indication whether multiple responses are needed, or a single response is sufficient.

3.	Conclusion

This contribution makes the following observations and proposals:

Proposal 1: A-IoT paging message should include 2-bit ‘transaction ID’ field to avoid duplicated responses from A-IoT devices.

Observation 1: Considering the SA1 discussion and the example cases from reference technologies (e.g., RFID), the length of the device identifier used in the A-IoT system may vary. There could be different types of A-IoT device identifiers with different lengths.

Proposal 2: Considering that the A-IoT device identifier may have various types with different lengths, the A-IoT paging message should include a ‘type field’ for each device identifier included.

Proposal 3: To ensure forward compatibility with scenarios where multiple device identities can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple device identities.

Proposal 3a: If a reserved field for multiple device identities is defined, the A-IoT paging message should include a ‘type field’ for each identifier stored in the field.

Proposal 4: A-IoT paging message should include a ‘number of access occasions’ field for the subsequent random access procedure, so that A-IoT devices are aware of the number of access occasions.

Proposal 5: A-IoT paging message should not include subsequent random access procedure type (e.g., whether CBRA or CFRA); an implicit indication (e.g., based on the number of access occasions) is sufficient.

Proposal 6: A-IoT paging message should include a list of frequency information for D2R transmission, i.e., Msg1 transmission.

Observation 2: In A-IoT systems, there could be scenarios where multiple readers transmit A-IoT paging messages for the same A-IoT service. In some cases, an A-IoT device may be required to respond to each of these paging messages, while in other cases, the device may only need to respond to one of the messages (e.g., the first one received).

Proposal 7: For forward compatibility with multiple-reader scenarios and to enable A-IoT devices to operate properly, A-IoT paging message should include a reserved field that could be used to accommodate the reader ID of the sender of the A-IoT paging message.

Proposal 8: To ensure proper operation of A-IoT devices in multiple-reader scenarios, the A-IoT paging message should include a reserved field that could be used to accommodate a 'service ID.' This service ID indicates which A-IoT service, requested by CN, is associated with the paging messages, regardless of which reader transmits them.

Proposal 9: To enable A-IoT devices to determine whether to respond to each A-IoT paging message or only one A-IoT paging message in multiple-reader scenarios, A-IoT paging message should include a reserved field that could be used to accommodate an indication whether multiple responses are needed, or a single response is sufficient.

4.	References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR)

- R2-2409210, Report of 3GPP TSG RAN WG2 meeting #127

- R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis

- Draft Report of 3GPP TSG RAN WG2 meeting #128

- 3GPP TR 22.840, Study on Ambient power-enabled Internet of Things

- R1-2403821, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0

- R1-2409341, Final Report of 3GPP TSG RAN WG1 #118b v1.0.0

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR