3GPP TSG-RAN WG2 Meeting #129	R2-2500089

Athens, Greece, February 17-21, 2025

Agenda item:	7.0.2.16

Source:	Qualcomm Incorporated

Title:	Discussion on PDCP SN gap during mobility

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss what RAN2 should reply to RAN3’s LS (R3-247891) on PDCP SN gap during mobility.

# Discussion

In LS R3-247891, RAN3 describe the following issue:

For Rel-18 XR, it is unclear how to handle the PDCP SN gap report during UE’s inter-gNB mobility. If a UE discarded some UL packets before handover, the UE reports the PDCP SN gap report to the source gNB, but the target gNB may not be aware of those discarded packets if no PDCP SN gap is reported to target gNB. This may result in unnecessary delay in the target gNB.

RAN3 then list three options for resolving this issue:

- Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB. - Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB. - Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB.

RAN3 specifically ask if Alternative 3 can be supported.

In our understanding, one of the design objectives for PDCP is that it is transparent to changes in connectivity in the lower layers, such as handover, changes in cell groups, etc. Impact of those events are handled by gNBs. For example, source gNB forwards the information necessary for the continuity of the connection to the target gNB during handover.

Observation 1. 	In legacy, PDCP is transparent to changes in connectivity in the lower layers, such as handover, changes in cell groups, etc.

Most L2/3 procedures that follow this principle too. For example, during handover, UE is not required to resubmit assistance information (e.g. UE Assistance Information, BSR) that it previously sent to gNB. It is source gNB’s responsibility to forward the relevant state information to the target gNB.

Observation 2. 	It is network’s responsibility to handle state transfers during mobility, which is a design principle followed by most L2/3 procedures.

We do not see anything special with PDCP SN gap reporting that justify a deviation from this design principle. Therefore, Alternative 3 should not be supported.

Proposal 1.	During mobility, UE is not required to re-submit PDCP SN gap report to the target gNB, if there is no new gap.

Proposal 2.	Reply to RAN3 that Alternative 3 should not be supported.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1. 	In legacy, PDCP is transparent to changes in connectivity in the lower layers, such as handover, changes in cell groups, etc.

Observation 2. 	It is network’s responsibility to handle state transfers during mobility, which is a design principle followed by most L2/3 procedures.

Proposal 1.	During mobility, UE is not required to re-submit PDCP SN gap report to the target gNB, if there is no new gap.

Proposal 2.	Reply to RAN3 that Alternative 3 should not be supported.