3GPP TSG-RAN WG2 Meeting #129	R2-2500091

Athens, Greece, February 17-21, 2025

Agenda item:	8.7.3

Source:	Qualcomm Incorporated

Title:	Discussion on measurement gap enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss configuration aspects of dynamic measurement gap (MG) cancelation and options other than DCI for MG cancelation.

# Discussion

There are different types of MGs designed for different purposes, e.g. positioning MG, MUSIM gaps, inter-frequency MGs, etc. Not every type of MG should be subject to dynamic MG cancelation. In addition, UE’s mobility status may also affect whether a MG may be dynamically cancelled. Therefore, we think it is necessary to have a mechanism to enable and disable dynamic MG cancelation. To do that, a simple option is to set the granularity of this mechanism to an individual MG configuration. It then can be left to network configuration to ensure dynamic cancelation is enabled only for the right type of MGs.

Proposal 1.	Dynamic measurement gap cancelation is enabled/disabled per measurement gap configuration by RRC.

Currently, the only agreed solution for canceling a MG is by DCI. That requires network to have full knowledge of UE’s mobility status, delay status of UE’s data, etc to cancel a MG. Network can obtain UE’s mobility status from measurement reports. However, network may not have knowledge of UE’s need for urgent uplink transmissions.

Observation 1.	Network may not be aware of UE’s need for urgent uplink transmission and cancel a measurement gap for it in time.

In some cases, those uplink transmissions warrant higher priority over measurement gaps, e.g. when UE has data who remaining time is very low, or when UE has a pending DSR triggered by a high-priority logical channel, etc. In those cases, if cancelation of a MG does not significantly impact UE’s mobility performance, it makes sense for UE to autonomously cancel the MG and perform the uplink transmission instead.

To enable autonomous cancellation of MGs in those cases, we think two aspects need to be studied:

- Which uplink transmissions can be considered urgent enough for UE to autonomously cancel an MG;

- Under what mobility conditions a UE is allowed to autonomously cancel an MG for urgent UL transmissions.

Proposal 2. 	Under proper conditions, UE is allowed to autonomously cancel a measurement gap for urgent UL transmissions. FFS which UL transmissions can be considered and what the conditions are.

Another option is to use RRC to pre-configure a set of MGs that can be canceled.

Since UE performs measurements on DL reference signals during MGs, time locations of MGs follow those of DL reference signals, which are configured per-cell and periodic (e.g. once every 20 msec). XR traffic is also periodic but has non-integer periodicity. As a result, there are always some occasions in a MG configuration overlapping with data bursts in XR traffic, regardless of how DL reference signals are configured (see Figure 1 for an example). On the other hand, since both MGs occasions and XR traffic are periodic, the time locations where they may overlap, at least those overlap with the start of data bursts, with jitter also taken into account, are also periodic and can be pre-determined once their periodicities and relative start offsets are known.

Observation 2. 	Measurement gaps that overlap with the start of data bursts are periodic and can be identified deterministically based on their periodicities and relative start offsets.

To mitigate the impact of MGs on XR traffic, among the occasions where MGs overlap with data, network can choose a subset of them and allow them to be overridden by data. This selection can be periodic, e.g. once every N overlapping MG occasions. And this selection can be relatively static, because they can be determined based on the periodicities of a MG configuration and XR traffic, and their relative start offset. It may need to be adjusted only when UE’s mobility status changes and so on.

Given the periodic and relatively static nature of these occasions, it is more efficient to indicate them to UE by RRC configuration instead of dynamic signaling methods such as DCI. That not only reduces signaling overhead, but also greatly simplifies UE implementation, because pre-determined occasions give UE more time to prepare for RF tuning and related procedures.

Based on the above analysis, we think RAN2 should discuss the option of using RRC configuration to indicate whether a measurement gap can be overridden by data.

Proposal 3. 	Network can RRC configure a set of measurement gaps that can be overridden by data.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	Dynamic measurement gap cancelation is enabled/disabled per measurement gap configuration by RRC.

Observation 1.	Network may not be aware of UE’s need for urgent uplink transmission and cancel a measurement gap for it in time.

Proposal 2. 	Under proper conditions, UE is allowed to autonomously cancel a measurement gap for urgent UL transmissions. FFS which UL transmissions can be considered and what the conditions are.

Observation 2. 	Measurement gaps that overlap with the start of data bursts are periodic and can be identified deterministically based on their periodicities and relative start offsets.

Proposal 3. 	Network can RRC configure a set of measurement gaps that can be overridden by data.