3GPP TSG-RAN WG2 Meeting #129	R2-2500095

Athens, Greece, February 17-21, 2025

Agenda item:	8.7.6

Source:	Qualcomm Incorporated

Title:	Discussion on XR rate control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss issues in supporting uplink rate control for XR services. Namely, how to support per-QoS-flow rate indication, rate query procedures by UE, designs of the rate indication MAC CE.

# Discussion

Procedures for rate indication

At RAN2#128, the following agreements on the granularity of rate indication were agreed:

- gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

We discuss these two FFS issues in the following.

For the first issue (i.e. whether the rate control indication for gNB should be coming from UE or CN), we have the following observations on the possible options.

For uplink rate control, it is either UE or application tethered to the UE is the traffic source. Hence the UE has the necessary information for identifying which set of QoS flows can benefit from or are subject to rate control. There are two possible options for UE to provide this information to RAN:

- Option A. UE provides this information directly to RAN. The reasoning behind this option is that congestion estimation and rate control are primarily MAC-layer procedures. Hence it is more direct and faster for UE to interact with RAN directly by MAC-layer signaling.

- Option B. UE provides this information to CN by NAS signaling; CN then forwards it to RAN or use it to configure RAN accordingly. The justification for this option is that the CN needs to be involved in rate control too. When the UE adapts its uplink bit rate, the CN needs to adjust its traffic parameters accordingly.

In our understanding, within the current QoS and traffic management framework, CN should be involved in the uplink rate control. For example, CN needs to know which QoS flows support rate adaptation, so that it can set up alternative QoS/traffic profiles for them. When RAN adjusts UE’s rates, it can notify CN of which QoS/traffic profile to use. However, in the current framework, there is no mechanism/signaling for RAN to initiate and lead these steps. Therefore, we think CN should be involved in the rate indication loop.

The third option that CN instead of UE should lead the indication for rate control. More specifically,

- Option C. CN receives the rate control information from application server and then informs RAN about it. The motivation behind this option is that CN receives QoS requirements from an application server. It is thus natural for the application server to inform the CN which QoS flows should be subject to rate control during this step. After the CN receives this information from the application server, it then configures RAN to ensure appropriate radio bearers and RAN resources are allocated to meet the QoS requirements. This step can be a good vehicle for CN to indicate to gNB which QoS flows are subject to rate control.

Based on the above analysis, we can see that CN should be involved in the RAN configurations related to rate control, although it is a MAC-layer procedure. For this reason, we think RAN2 should coordinate with SA2 before making design decisions on this topic.

Proposal 1.	Core network is involved in the RAN configurations related to rate control.

Proposal 2.	RAN2 coordinate with SA2 (e.g. by LSs) in the decision on how gNB is informed about which QoS flows are subject to rate control.

For the bit indication between RAN and UE, there are also two options:

- Option A. Bit rates of each QoS flow are included in MAC CE.

- Option B. Only the bit rate for the whole DRB/LCH is included in MAC CE. After receiving this total bit rate, UE derives the bit rates for different QoS flows based on some RRC configured parameters, e.g. split ratio between the flows.

During the online discussion at RAN2#128, there were concerns about Option A by both infra vendors and UE makers. For example, some RAN implementations may not be able to estimate rates for individual flows, while some UE makers explained the challenges they have if rates are indicated dynamically for individual flows.

Considering the variations in network and UE implementations, and the differing impacts of per QoS flow indication on the existing implementations, we think UE capability can serve as a good compromise for moving forward. More specifically,

- UEs support Option B as a baseline, while Option A can be a UE capability;

- Given a UE’s capability, it is up to network implementation to choose which option it applies.

Proposal 3.	RRC configured per-QoS-flow rate indication is the baseline. Support for per-QoS-flow rate indication within MAC CE is a UE capability.

Proposal 4.	If rate indication for a DRB is based on RRC configuration, network configures UE with a ratio for allocating rates among QoS flows in the DRB.

In the legacy Recommended Bit Rate procedure, UE is allowed to send a query to gNB and ask for available bit rate for a DRB. We think this feature is applicable and useful to XR rate control too. For example, when there is a change in UE/application’s state, e.g. there is a change in scenes requiring higher resolution, or when the UE switches from battery to being plugged in, the UE may want to know the bit rates that the network can support before adjusting.

Proposal 5.	UE can query available uplink bit rate for either a DRB or a QoS flow.

MAC CE design

SA4 replied to RAN2’s inquiries about the range and required granularity of bit rates in S4-242182. In their reply, SA4 have indicated that

- The range of uplink bit rate varies from 100 Kbps to 40 Mbps;

- The desired granularity can be as small as 1% but coarser granularity such as 10% may also be sufficient.

With these two requirements, we can evaluate which type of encoding scheme and how many code points should be used.

In legacy Recommended Bit Rate MAC CE, bit rates are encoded with a piecewise linear distribution. We first check if linear or piecewise linear encoding is also suitable for XR rate control.

If linear encoding (i.e. constant step size) is used, the largest quantization error is associated with the smallest code point. If we apply the largest quantization error required by SA4, which is 10%, then the quantization step size = 100 Kbps x 10% = 10 Kbps. Consequently, the number of required code points = (40,000 – 100)/10 +1 = 3991.

If piecewise linear encoding is used, suppose M is the end point of the 1st segment (also the start of the 2nd segment) and p = quantization error. Then the total number of code point = (M - 100)/(100 x p) + 1 + (40000 – M) / (M x p) +1. It is not difficult to derive the optimal value of M that minimizes the number of code point from this equation, which is 2,000 Kbps. The corresponding number of code points = 38/p. Therefore, the total number of code point is 380 at p=10%, 760 at 5% and 3800 at 1%.

From the calculations above, we may observe that

Observation 1. 	If linear or piecewise linear distribution is used to encode the bit rates provided by SA4, it would require excessive number of code points to achieve the required granularity of 10% or less.

If exponential encoding is used, then for a quantization error p, the number of code points N can be found from the equation Rmax = Rmin x (1 + p)^N, where Rmin and Rmax are the minimum and maximum rates, respectively. The table below lists N as a function of p:

| Q.E. | # of code points | # of bits |
| --- | --- | --- |
| 1% | 603 | 10 |
| 2% | 303 | 9 |
| 3% | 203 | 8 |
| 4% | 153 | 8 |
| 5% | 123 | 7 |
| 6% | 103 | 7 |
| 7% | 89 | 7 |
| 8% | 78 | 7 |
| 9% | 70 | 7 |
| 10% | 63 | 6 |

We may observe from the table that the resulting encoding is much mor efficient than linear or piecewise linear ones. For example, quantization error of only 3% can be achieved with only 203 code points (or 8 bits). And even 6 bits is sufficient to achieve quantization error of 10%.

Observation 2.	Exponential encoding can achieve a good balance between low quantization error and low overhead.

Therefore, we propose to use exponential distribution to encode the range of bit rates required by SA4. In addition, with exponential distribution, the full range of the bit rates can be covered by a single field. Hence no rate multiplier similar to the one in the legacy Recommended Bit Rate MAC CE is needed.

But we think the choice for the number of codes point can be tentatively postponed until all other fields in the MAC CE are determined. The reason is that it is related to the length of the bit rate field, whose choice may also depend on the choice and design of other fields in the MAC CE.

Proposal 6.	Apply exponential distribution to encode the bit rates required by SA4. FFS The number of code points (e.g. after all fields in the MAC CE are finalized).

Proposal 7.	No rate multiplier is needed.

One of the special characteristics of XR applications is that it generates multiple flows with different QoS requirements, e.g. video, audio, haptic, control, etc. When an application performs rate adaptation, it typically adjusts the rates of multiple flows simultaneously. Although it is possible to send multiple MAC CEs (one for each flow or LCH) at once in this case, it is more efficient and customary (like many other DL MAC CEs) to include all of them in a single MAC CE.

Observation 3. 	XR applications typically adopt bit rates of multiple QoS flows or LCHs together.

Proposal 8. 	The rate control MAC CE can include bit rate indications for multiple QoS flows or logical channels.

So far, there are at least several differences in the necessary fields required by XR rate control and the legacy Recommend Bit Rate procedure. For example, XR rate control require per-flow indication. The rate indication field likely to be longer, but no rate multiplier probably is needed. Therefore, in our opinion, it is simpler to design a new MAC CE for XR rate control instead of extending the legacy Recommended Bit Rate MAC CE.

Proposal 9.	Introduce a new MAC CE for XR rate control, instead of enhancing the legacy Recommended Bit Rate MAC CE.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	Core network is involved in the RAN configurations related to rate control.

Proposal 2.	RAN2 coordinate with SA2 (e.g. by LSs) in the decision on how gNB is informed about which QoS flows are subject to rate control.

Proposal 3.	RRC configured per-QoS-flow rate indication is the baseline. Support for per-QoS-flow rate indication within MAC CE is a UE capability.

Proposal 4.	If rate indication for a DRB is based on RRC configuration, network configures UE with a ratio for allocating rates among QoS flows in the DRB.

Proposal 5.	UE can query available uplink bit rate for either a DRB or a QoS flow.

Observation 1. 	If linear or piecewise linear distribution is used to encode the bit rates provided by SA4, it would require excessive number of code points to achieve the required granularity of 10% or less.

Observation 2.	Exponential encoding can achieve a good balance between low quantization error and low overhead.

Proposal 6.	Apply exponential distribution to encode the bit rates required by SA4. FFS The number of code points (e.g. after all fields in the MAC CE are finalized).

Proposal 7.	No rate multiplier is needed.

Observation 3. 	XR applications typically adopt bit rates of multiple QoS flows or LCHs together.

Proposal 8. 	The rate control MAC CE can include bit rate indications for multiple QoS flows or logical channels.

Proposal 9.	Introduce a new MAC CE for XR rate control, instead of enhancing the legacy Recommended Bit Rate MAC CE.