3GPP TSG-RAN WG2 Meeting #129	R2-2500096

Athens, Greece, Feb. 17th – Feb. 21th, 2025

Source: 	NTU

Title:  	Further discussion on CB-Msg3 Mechanism

Agenda Item:	8.9.3

Document for:	Discussion and Decision

- Introduction

In previous 3GPP RAN2 meetings, RAN2 has discussed several aspects of CB-Msg3 mechanism and made some agreements. Here are previous agreements related to our proposal:

RAN2#127 Agreements (partial): For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition. RAN2#127bis Agreements (partial): RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. RAN2#128 Agreements (partial): Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

In this proposal, we give some analysis about some issues about CB-Msg3, including congestion management, number of replicas for DSA and retransmission.

- Discussion

  - CB-Msg3 congestion management

Congestion management is a challenging problem for contention-based random access scheme, especially in IoT scenarios. Random access acquirement from large amounts of IoT devices causes lots of collisions, leading to high PLR (packet loss ratio) and lower system throughput.

For previous 3GPP RAN2 discussions about CB-Msg3, simulations demonstrated that system PLR increases sharply beyond certain traffic load levels. During the RAN2 #127bis meeting, it was decided to focus primarily on scenarios where PLR remains below 10%. However, in real IoT networks, system loads may exceed this level, leading to higher PLR. By adopting effective congestion management methods, it is possible to handle higher system loads while staying within acceptable PLR limits.

Access Class Barring (ACB) is a widely used and standardized congestion control mechanism in LTE IoT networks. In ACB, the eNB broadcasts a parameter called "ACB factor" to UEs. Each UE randomly generates a number between 0 and 1 and compares it to the ACB factor. If the generated number is smaller than the ACB factor, the UE is granted network access and can initiate transmission. Otherwise, the UE is barred and must wait for another transmission opportunity. By adjusting the value of ACB factor, the eNB can control the system load and avoid large numbers of collisions.

In figure 1, we simulate the performance of ACB for CB-Msg3, where the simulation setting can be found in Appendix. The simulation result indicates that ACB can greatly increase the throughput on high traffic load scenarios, especially when applying DSA. A smaller ACB factor performs well in high traffic loads but results in a higher PLR when the system is not congested due to unnecessary barring.

Figure 1: Performance of ACB

Observation 1: ACB (Access class barring) can improve performance in high traffic load scenarios.

Proposal 1: RAN2 could consider adopting congestion management techniques, such as Access Class Barring (ACB), to improve system performance in high traffic load scenarios.

  - Dynamic ACB and load-aware parameter control

In Figure 1, it is shown that ACB (Access Class Barring) can improve system performance in high traffic load case. However, if the ACB factor is fixed, it costs unnecessary barring when channel is not congested. In such case, ACB causes system performance degradation. To solve this problem, lots of works have been proposed [2] and many of them focus on the adjustment of ACB factor along with load conditions. For example, an ACB factor adjustment scheme is proposed in [3]. In this work, the ACB factor is initially set as 1 and the eNB adjust it by comparing the expected value and actual number of collisions. If the number of collisions is higher than expected, the system is considered overload and the eNB reduces ACB factor. Otherwise, the eNB increases the ACB factor to access more UE.

In an uplink random access system, time-frequency resource blocks can exist in one of three states: idle (no packet), successful transmission (single packet), or collision (multiple packets). In a dynamic ACB system, the eNB needs to collect historical resource block information to adjust the ACB factor accordingly.

Beyond the ACB factor, other system parameters, such as the number of replicas, can also be dynamically adjusted based on varying traffic load conditions. According to Figure 1, when traffic load is higher, using fewer replicas (adopting SA) helps reduce collisions. Conversely, when traffic load is lower, using more replicas (adopting DSA) is preferable for lowering the PLR. In the following discussion, all parameters adjusted based on traffic load are collectively referred to as "CB-Msg3 parameters."

Figure 2 illustrates a potential random access procedure using CB-Msg3 with load-aware parameter control. At the start of a time frame, the eNB provides PUSCH resources via SIB and includes the current CB-Msg3 parameters. Active UEs utilize the provided parameters to execute the ACB mechanism and initiate the random access procedure if not barred. UEs that are barred or suffer transmission failure must wait for the next transmission opportunity. On the eNB side, after sending Msg4, the eNB calculates the CB-Msg3 parameters for the next frame based on the status of PUSCH resources (idle, successful transmission, or collision) in the current frame.

Figure 2: CB-Msg3 with Dynamic ACB

Proposal 2: For CB-Msg3 load-aware parameter control, such as dynamically adjusting the ACB factor, number of replicas or other parameters, the eNB can collect historical resource block status, update CB-Msg3 parameters, and transmit them to UEs via SIB. FFS on the specific CB-Msg3 parameters to be adjusted (e.g., ACB factor, number of replicas, or others) and the detailed design of SIB.

  - Retransmission enhancements

To ensure successful data transmission, retransmissions are necessary for CB-Msg3. In an NTN scenario, the distance between the ground and the satellite introduces significant propagation delays, making excessive retransmissions unacceptable. To mitigate this issue, one potential solution is to prioritize retransmissions to improve their success rate.

Figure 3: Performance of different replica configurations for first attempt and retransmitting UEs.

In previous RAN2 meetings, DSA was introduced for CB-Msg3. In a DSA-based system, the more replicas a UE can transmit, the higher the probability of successful transmission. Building on this, one potential method to enhance retransmission is using more replicas for retransmissions. During traffic load intervals when the system remains stable, the number of retransmitting UEs is fewer than that of UEs during the first attempt, so prioritizing retransmissions does not significantly impact overall performance. Figure 3 illustrates the results of different replica configurations for first-attempt UEs and retransmitting UEs. It is noted that the maximum retransmission attempt is 3 in the simulation. In the figure, notation “x/y” indicates that x replicas are used for first attempt and y replicas is used for retransmissions.

Based on Figure 3, it is shown that if the number of replicas on retransmission is fixed, using less replicas on first attempt could achieve better PLR and throughput. For example, configuration 2/3 achieve better performance than 3/3 in all traffic load scenarios, and similar result comes in configuration 1/2 and 2/2. The simulation results imply that using more replicas on retransmission is an effective technique for system performance enhancement.

Observation 2: Using more replicas for retransmissions can improve the retransmission success rate, leading to better overall system performance.

Proposal 3: The NW could configure different numbers of replicas for first-attempt UEs and retransmitting UEs, with more replicas used for retransmissions.

- Conclusions

This contribution will discuss in Rel-19 IoT NTN and give the following observations and proposals.

Observation 1: ACB (Access class barring) can improve performance in high traffic load scenarios.

Observation 2: Using more replicas for retransmissions can improve the retransmission success rate, leading to better overall system performance.

Proposal 1: RAN2 could consider adopting congestion management techniques, such as Access Class Barring (ACB), to improve system performance in high traffic load scenarios.

Proposal 2: For CB-Msg3 load-aware parameter control, such as dynamically adjusting the ACB factor, number of replicas or other parameters, the eNB can collect historical resource block status, update CB-Msg3 parameters, and transmit them to UEs via SIB. FFS on the specific CB-Msg3 parameters to be adjusted (e.g., ACB factor, number of replicas, or others) and the detailed design of SIB.

Proposal 3: The NW could configure different numbers of replicas for first-attempt UEs and retransmitting UEs, with more replicas used for retransmissions.

Reference

[1] Chairman_notes of 3GPP TSG RAN WG2 meeting #128

[2] Iiyambo, Loini, Gerhard Hancke, and Adnan M. Abu-Mahfouz. "A survey on NB-IoT random access: approaches for uplink radio access network congestion management." IEEE Access (2024).

[3] Duan, Suyang, Vahid Shah-Mansouri, and Vincent WS Wong. "Dynamic access class barring for M2M communications in LTE networks." 2013 IEEE Global Communications Conference (GLOBECOM). IEEE, 2013.

Appendix

Simulation setting

| Parameter | Value |
| --- | --- |
| Number of EDT resources (subcarriers) per occasion, Nsc | 48 |
| Number of devices | 5000 |
| Number of EDT occasions, N | 4 |
| Duration of an EDT occasion, Tedt | 96 ms |
| Number of replicas for DSA | 2 |
| Round trip time, RTT | 510 ms |
| Transmission error probability | 0.01 |
| Buffer size | 10 packets |
| Maximum retransmission number | 3 |