3GPP TSG-RAN WG2 Meeting #129	R2-2500097

Athens, Greece, Feburary 17th – Feburary 21st, 2025

Agenda Item	: 7.6 (NR_SL_enh2-Core)

Source	: LG Electronics Inc.

Title	: Discussion on left issue of MCSt

Document for	: Discussion and Decision

1.	Introduction

This contribution discusses the left issue on MCSt.

2.	Discussion

Issue 1. Allowing MCSt for multiple TBs in resource pool with PSFCH.

At the #128bis meeting [1], RAN2 discussed as follows regarding the MCSt.

R2-2410612	Stage 2 Corrections on MCSt	InterDigital Inc, Apple	CR	Rel-18	38.300	18.3.0	0939	-	F	NR_SL_enh2

Check what was agreed/implemented in RAN2, whether both approach 1 and approach 2 (regarding MCSt and PCFCH) are enough covered, offline.

CR will be revised in R2-2410943

[LG, Vivo]: To be more accurate, it should be restricted to single TB case. [OPPO]: Not sure if it is clear whether there is the restriction in MAC spec. If we don’t have such restriction in MAC, this CR should be ok. [Huawei]: In MAC, there is a case of MCSt where ACK is used and assume that it is for the multiple TB case. [Qualcomm]: Have different understanding. We agreed with no MCSt for both single TB case and multiple TB case, with PSFCH. [Qualcomm]: In addition, the current MAC is not clear on the differentiation of approach 1 and approach 2.

[AT128][112][V2X/SL] (ZTE)

Scope: Check what was agreed/implemented in RAN2, whether both approach 1 and approach 2 (regarding MCSt and PSFCH) are enough covered, whether we need to update MAC, actual MAC changes if needed, and attempt to reach RAN2 consensus.

Intended outcome: Discussion summary in R2-2410945. F2F offline discussion schedule will be announced by offline discussion rapporteur.

Deadline: Comeback in CB session.

R2-2410945	Discussion on MCSt	ZTE Corporation, Sanechips	Discussion	NR_SL_enh2

Proposal1: Case 1c is supported, that is N=1 or no value N is indicated to PHY layer for sensing, for a resource pool configured with PSFCH resource, MAC layer may select consecutive slots for transmissions of multiple TBs, if minimum time gap for HAQR feedback is ensured.

Postponed.

[Qualcomm]; PSFCH needs 4 symbols. Gap should be less than 16us/25us for MCSt. [Session Chair]: Question is if a resource pool with PSFCH is supported in RAN1? [Huawei]: Understand it is clear PSFCH with 4 symbols between two TBs will not impact MCSt in RAN1. Should be allowed. [OPPO]: Think it is good to postpone the discussion and it will be good to see the corresponding spec impacts if we allow case 1c. [Ericsson]: If 1c is related only to approach 1 then since approach 1 is done based on best effort manner, if the UE receives ACK in the middle of MCSt, then another solution is that the UE can stop MCSt operation for the remaining slots.

R2-2410943	Stage 2 Corrections on MCSt	InterDigital Inc, Apple	CR	Rel-18	38.300	18.3.0	0939	1	F	NR_SL_enh2

Postponed.

According to the RAN2 agreements below, it has been agreed to support MCSt in a best effort manner for multiple TBs.

Approach 1: “best effort for multiple TBs”

- Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior.

- Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior.

- Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt.

- Step 4: Repeat Step 1-3 for different TB if required.

Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs”

- Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means.

- Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change)

- Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior).

- Step 4: Repeat Step 1-3 for different TB if required.

It was also agreed that restriction (“For a resource pool configured with PSFCH resource, UE can NOT select consecutive slots (i.e., MCSt) for transmissions of a single TB.”) on resource pool is only applied in single TB cases.

RAN2#122 - Agreements on MCSt - For Qustion-1 from RAN1 (Q1 in R1-2304257), R2 replies that it is feasible to select the resource for a single TB in MAC layer and concatenate across separate resource selection triggers across TBs in a best-effort manner. #### RAN2#123bis Regarding sidelink unlicensed, the following agreements and working assumptions were made: - Agreements on resource (re)selection: - R2 not pursue the UE behavior of prioritizing the resources within a shared COT during resource selection step. - R2 not pursue the UE behavior of triggering a resource reselection upon reception of a usable shared COT. - MAC layer, based on UE implementation, decides whether to indicate a “number of consecutive slots for MCSt” larger than 1. - MAC layer, based on UE implementation, decides the value of “number of consecutive slots for MCSt”, as long as it meets the CAPC maximum COT duration requirement. - For a resource pool configured with PSFCH resource, UE can NOT select consecutive slots (i.e., MCSt) for transmissions of a single TB.

In summary, RAN2 has agreed that MCSt can be configured in multiple TBs, even though the PSFCH is configured in resource pool.

Observation 1. RAN2 has agreed that MCSt can be configured in multiple TBs, even though the PSFCH is configured in resource pool.

Also, RAN1 agreed the followings to reduce the gap between the two transmissions for MCSt support in #113 meeting.

Agreement Specification supports that CPE can be transmitted between any two consecutive SL transmissions by the same UE to reduce the gap between the two transmissions so that it does not exceed 16µs. - Note: for this case, the CPE length should not be longer than up to 2 symbols, as per previous agreements - FFS: details if needed (e.g., considering outcome of discussion on PSFCH-like signal in PHY agenda) - FFS whether PSSCH can be transmitted instead of or in addition to CPE - FFS: how to determine the CPE starting position

In other words, according to the RAN1 agreement above, as shown in the example of Figure 1 below, the UE can reduce the GAP to less than 16 microseconds through CPE transmission even in the resource pool where PSFCH is configured, so there is no problem at all with MCSt being supported. As in the example below, the UE can transmit a CPE to reduce the GAP within the switching GAP to less than 16 microseconds to support MCSt. However, in order to configure MCSt in a resource pool where PSFCH is configured, there must be a PSFCH to be transmitted by the UE between consecutive slots.

Also, even if the PSFCH interval (sl-PSFCH-Period-r16) is "2", MCSt is not always guaranteed. Since the CPE length is determined based on the priority of the PSCCH/PSSCH transmission, if the length sufficient to reduce the switching GAP (1 OFDM symbol) to less than 16us is not guaranteed, the UE cannot configure MCSt even if there are no PSFCH symbols between consecutive slots.

We understand that the best effort manner is introduced in that MCSt can be configured when the above situation (1. there must be a PSFCH to be transmitted by the UE between consecutive slots, 2. CPE length sufficient to reduce the switching GAP (1 OFDM symbol) to less than 16us) is satisfied when MAC does not provide “n_MCSt” to PHY.

Figure 1. Example of MCSt support (i.e., best effort for multiple TBs) in a resource pool with PSFCH configured (15 khz SCS, sl-PSFCH-Period-r16 is sl1)

Observation 2. According to the #113 meeting’s RAN1 agreement for CPE transmission, UE can reduce the GAP to less than 16us through CPE transmission even in the resource pool where PSFCH is configured, so there is no problem at all with MCSt being supported.

Observation 3. It is understood that the best effort manner is introduced in that MCSt can be configured when the situation (1. there must be a PSFCH to be transmitted by the UE between consecutive slots, 2. CPE length sufficient to reduce the switching GAP (1 OFDM symbol) to less than 16us) is satisfied when MAC does not provide “n_MCSt” to PHY.

Considering the above observations, RAN 2 does not need to prevent the MCSt case for multi-TB in resource pool with PSFCH.  Additionally, it is clear that MCSt is supported on multiple TBs without additional corrections, so RAN2 does not need to perform additional work for the issue 1 (i.e., MCSt for multiple TBs in resource pool with PSFCH).

Proposal 1. RAN2 keep the conclusion of allowing MCSt for multiple TBs in resource pool with PSFCH, and not pursue optimization for the case of PFSCH between two selected consecutive slots.

Issue 2. Behaviour of flushing HARQ buffer and clearing SL grant for multiple TBs MCSt case

A decision by RAN2 is required regarding the TP below in R2-2410073 [2] discussed at the last #128bis meeting.

NOTE *: If a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

NOTE *: If a positive acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may flush the HARQ buffer of the associated Sidelink process.

As shown below, according to the current MAC specification, in order to maintain the MCSt structure so that the UE continues to perform Type 2 LBT-based operation in multiple TBs transmission, the UE does not flush the HARQ buffer if there are remaining slot(s) even if it receives a positive acknowledgment. The SL grant is also not cleared.

1>	if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTransNum; or

1>	if a positive acknowledgement to this transmission of the MAC PDU was received according to clause 5.22.1.3.2, except a positive acknowledgement to Multi-consecutive slots transmission (i.e., multiple TBs case) of the MAC PDU and there is remaining slot(s) for this MAC PDU; or

1>	if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for this transmission of the MAC PDU according to clause 5.22.1.3.2:

2> flush the HARQ buffer of the associated Sidlink process.

And

1>	if a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.1a, except a positive acknowledgement to Multi-consecutive slots transmission (i.e., multiple TBs case) of the MAC PDU and there is remaining slot(s) for this MAC PDU:

2>	clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

Figure 2 below is an example of the operation described in the current MAC specification. As an example, even though the UE receives ACKs for all TBs, if there are remaining slot(s), the UE maintains the MCSt structure by not flushing the HARQ buffer and not clearing the SL grant. And although ACKs are received for all TBs, retransmission is performed in the remaining slot(s) to maintain MCSt. This behaviour is not problematic from the perspective of MCSt, but it may cause problems as the cogestion level in the resource pool increases due to unnecessary transmission.

Figure 2. Behaviour of flushing HARQ buffer and clearing SL grant for multiple TBs MCSt case – No flushing HARQ buffer and clearing SL grant when all TBs are acknowledged.

Observation 4. According to the current MAC specification, even though the UE receives ACKs for all TBs, if there are remaining slot(s), the UE maintains the MCSt structure by not flushing the HARQ buffer and not clearing the SL grant. And although ACKs are received for all TBs, retransmission is performed in the remaining slot(s) to maintain MCSt.

Observation 5. Observation 3's behaviour is not problematic from the perspective of MCSt, but it may cause problems as the congestion level in the resource pool increases due to unnecessary transmission.

As shown in Figure 3 below, if ACKs are received for all TBs, flushing the HARQ buffer and clearing the SL grant can reduce the UE's power consumption by preventing unnecessary retransmission and also can prevent problems that increase the congestion level in the resource pool.

Figure 3. Behaviour of flushing HARQ buffer and clearing SL grant for multiple TBs MCSt case – Flushing HARQ buffer and clearing SL grant when all TBs are acknowledged.

Observation 6. For multiple TBs MCSt case, if ACKs are received for all TBs, flushing the HARQ buffer and clearing the SL grant can reduce the UE's power consumption by preventing unnecessary retransmission and also can prevent problems that increase the congestion level in the resource pool.

Proposal 2. RAN2 agrees following TP proposed in R2-2410073.

NOTE *: If a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

NOTE *: If a positive acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may flush the HARQ buffer of the associated Sidelink process.

3.	Conclusion

Issue 1. Allowing MCSt for multiple TBs in resource pool with PSFCH

Observation 1. RAN2 has agreed that MCSt can be configured in multiple TBs, even though the PSFCH is configured in resource pool.

Observation 2. According to the #113 meeting’s RAN1 agreement for CPE transmission, UE can reduce the GAP to less than 16us through CPE transmission even in the resource pool where PSFCH is configured, so there is no problem at all with MCSt being supported.

Observation 3. It is understood that the best effort manner is introduced in that MCSt can be configured when the situation (1. there must be a PSFCH to be transmitted by the UE between consecutive slots, 2. CPE length sufficient to reduce the switching GAP (1 OFDM symbol) to less than 16us) is satisfied when MAC does not provide “n_MCSt” to PHY.

Proposal 1. RAN2 keep the conclusion of allowing MCSt for multiple TBs in resource pool with PSFCH, and not pursue optimization for the case of PFSCH between two selected consecutive slots.

Issue 2. Behaviour of flushing HARQ buffer and clearing SL grant for multiple TBs MCSt case

Observation 4. According to the current MAC specification, even though the UE receives ACKs for all TBs, if there are remaining slot(s), the UE maintains the MCSt structure by not flushing the HARQ buffer and not clearing the SL grant. And although ACKs are received for all TBs, retransmission is performed in the remaining slot(s) to maintain MCSt.

Observation 5. Observation 3's behaviour is not problematic from the perspective of MCSt, but it may cause problems as the congestion level in the resource pool increases due to unnecessary transmission.

Observation 6. For multiple TBs MCSt case, if ACKs are received for all TBs, flushing the HARQ buffer and clearing the SL grant can reduce the UE's power consumption by preventing unnecessary retransmission and also can prevent problems that increase the congestion level in the resource pool.

Proposal 2. RAN2 agrees following TP proposed in R2-2410073.

NOTE *: If a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

NOTE *: If a positive acknowledgement to all Multi-consecutive slots transmissions (i.e., multiple TBs case) of the MAC PDU, UE may flush the HARQ buffer of the associated Sidelink process.

4.	Reference

[1] Session chair’s note for R18 SL-Evo of #128bis meeting (Samsung)

[2] R2-2410073	Correction on MCSt	ZTE Corporation, Sanechips