3GPP TSG-RAN WG2 Meeting#129	R2-2500125

Athens, Greece, 17th – 21st, Feb. 2025

Source:	vivo

Title:	Discussion on left issues and Reply LS on LMF-based AIML positioning

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

RAN2#128 meeting has made progress on functionality-based LCM of UE-sided model for AI enhanced positioning as follows [1].

Agreements 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered.

Besides, there is a post-meeting e-mail discussion over AI-enhanced UE-based DL positioning as well.

In this paper, we mainly focus on the leftover issues for RAN2 from the following perspectives:

- Consistency for training and inference

- Signaling of performance monitoring/functionality management

- Reply on LMF-based AI/ML Positioning to SA2

- Discussion

## Consistency for training and inference

RAN1#118bis meeting came to the agreement that on-demand PRS procedure can be used for training data collection [2].

Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

For Case 1, since UE has AI/ML model/functionality, it has the knowledge of DL PRS configurations during training stage. When it comes to the stage of inference, to ensure the consistency of training and inference, UE may request for suitable PRS configurations via UE-initiated on-demand PRS scheme. In this way, when assigning DL PRS configurations for inference, LMF can forward such preference to gNB, and consistency can be ensured.

- On-demand PRS scheme can be utilized to ensure consistency of NW-side configuration between training and inference.

The current on-demand PRS scheme is designed to ensure the positioning QoS, therefore only considering certain characteristics (e.g., bandwidth, periodicity) of the PRS, without requesting specific cells to transmit PRS. However, from our understanding, except for PRS configuration itself, the consistency should be ensured by the same PRS transmission cells. Therefore, the target UE should send the area where the interested PRS is applicable to LMF. With this priori knowledge, LMF is able to attach area and PRS configurations to specific NW-side additional condition. Furthermore, LMF may request the RAN-node near the UE and match with the condition(s) to perform DL positioning accordingly.

- Include the cell information in on-demand PRS request to ensure consistency during training and inference.

## Signaling related to performance monitoring/functionality management

At the previous RAN1 meetings, the following agreements/conclusions were made regarding the performance monitoring of AI/ML positioning Case 1.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Summarized from the RAN1 agreement, the performance monitoring and functionality management can be realized in the following options for positioning use case 1:

- Option 1: UE performs monitoring metric calculation and functionality management

- Option 2: LMF performs monitoring metric calculation and UE performs functionality management

- Option 3: UE performs monitoring metric calculation and LMF performs functionality management

- Option 4: LMF performs monitoring metric calculation and functionality management

For Option 1, with no LMF perception of which AI/ML functionality is enabled, UE autonomously decides to enable AI/ML based positioning and performs functionality management, UE may report AI/ML output of location prediction as the legacy location estimate. However, since RAN2 has agreed that “’UE-autonomous, UE’s decision is not reported to the network’ is not considered for Rel-19”, no further discussion on Option 1 is needed in WI phase.

As to Option 2, for the LMF-involved positioning (i.e., not UE autonomous positioning) procedure, we tend to believe that UE cannot perform AI functionality management under LMF decision on positioning method, where AI functionality is designed to be a positioning method on the trend. That is, if the target UE receives the request of location information attached to positioning method correspond to a specific functionality (FFS the granularity), it should consider the functionality is activated, while those not requested are deactivated. Therefore, Option 2 is excluded naturally.

In this sense, functionality management can only be performed by LMF in terms of R19 scope and positioning system layout.

- LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

For Option 3, functionality management is realized by LMF requesting UE with AI-enabled location information just as illustrated in Option 2.

- Functionality management (e.g., activation/deactivation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

Furthermore, based on RAN1 input, LMF may provide some assistance information to UE to achieve performance monitoring metric calculation at UE, e.g., ground truth label of the target UE (Option A-1), position calculation assistance data (Option A-2), PRU measurement or PRU location (Option A-3). UE can also obtain label from PRU in 3GPP-transparent way (Option A-4), which has no specification impact and is outside RAN2 scope. Logically speaking, if LMF configures UE to perform performance monitoring, or UE itself has intention to perform performance monitoring, the information can be provided via LPP Assistance Data Transfer procedure in both proactive/reactive way.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring metric calculation at target UE, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data.

For Option 4, based on RAN1 input, to achieve performance monitoring metric calculation at LMF, the model output based on either target UE’s measurement (Option B-1) or PRU’s measurement (Optionb B-2) is needed from UE.

Since the model output is location estimate, which is already the location information request by LMF as in the legacy positioning session, this can directly reuse LPP Provide Location Information to transmit. And for the PRU measurement part, LPP Provide Assistance Data can be reused.

- To enable performance monitoring metric calculation at LMF, target UE should provide model inference output based on the measurement of target UE or PRU.

- To enable performance monitoring metric calculation at LMF, LMF obtains model inference output from target UE via LPP Request/Provide Location Information.

On the basis of the above analysis, LMF or UE is able to perform performance monitoring metric calculation. LMF can be of long-term decision while UE can make real-time to decide whether AI/ML positioning accuracy is trustworthy. However, RAN1 has been parallel discussing entities to perform performance monitoring, we would like RAN2 to wait for RAN1 progress on whether both options should be introduced in specification.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring metric calculation.

## Reply on LMF-based AI/ML Positioning to SA2

SA2 has sent LS [3] about LMF-involved AI/ML positioning Case 2b, asking about RAN involvement and future plan as follows:

SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference.

Regarding the inquiry by SA2 to RAN1 and RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. RAN2 view can be summarized as follows:

- Current progress: As per WID RP-243244, LMF-based AI/ML Positioning for Case 2b is second priority use case, RAN2 has not make any discussion or agreement on Case 2b so far.

- RAN2 future work plan: Since Case 2b is the use case of second priority, RAN2 does not plan to work on such case until the use cases of the first priority works are done. Besides, whether to support the second priority use cases is also dependent on RAN1 discussion, so RAN2 may have to wait for RAN1 discussion progress before starting any potential work on case 2b.

In this context, a reply LS based on the above analysis to inform SA2 about our progress and work schedule on Case 2b is composed and to be approved.

- Adopt the Draft Reply LS to SA2 as attached in Annex, with the following RAN2 status on LMF-involved AI/ML positioning Case 2b:

- RAN2 has not make any discussion or agreement on Case 2b which is the use case of the second priority;

- RAN2 would wait for RAN1 discussion progress before starting any potential work on case 2b.

- Conclusion

In summary, we make the following proposals on functionality-based LCM for AI/ML enhanced positioning:

Consistency for training and inference

- On-demand PRS scheme can be utilized to ensure consistency of NW-side configuration between training and inference.

- Include the cell information in on-demand PRS request to ensure consistency during training and inference.

Signaling of performance monitoring/functionality management

- LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

- Functionality management (e.g., activation/deactivation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring metric calculation at target UE, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data.

- To enable performance monitoring metric calculation at LMF, target UE should provide model inference output based on the measurement of target UE or PRU.

- To enable performance monitoring metric calculation at LMF, LMF obtains model inference output from target UE via LPP Request/Provide Location Information.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring metric calculation.

Reply on LMF-based AI/ML Positioning to SA2

- Adopt the Draft Reply LS to SA2 as attached in Annex, with the following RAN2 status on LMF-involved AI/ML positioning Case 2b:

- RAN2 has not make any discussion or agreement on Case 2b which is the use case of the second priority;

- RAN2 would wait for RAN1 discussion progress before starting any potential work on case 2b.

- Reference

- RAN2#128 meeting Chair’s Notes

- RAN1#118bis meeting Chair’s Notes

- R2-2500062	LS on LMF-based AI/ML Positioning

# Annex: Draft Reply LS to SA2

3GPP TSG-RAN WG2 Meeting#129	R2-25xxxxx

Athens, Greece, 17th – 21st Feb. 2025

Title:	[DRAFT] Reply LS on LMF-based AI/ML Positioning for Case 2b

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	vivo [to be RAN WG2]

To:	SA WG2

Cc:	RAN WG1

Contact Person:

Name:	KIMBA DIT ADAMOU, Boubacar

E-mail Address:	kimba@vivo.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		-

1. Overall Description:

RAN2 thanks SA2 for the liason in R2-2500062 [S2-2501133] to inform about the progress on LMF-involved AI/ML positioning case 2b.

Regarding the inquiry by SA2 to RAN1 and RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b, RAN2 would like to provide the following feedback:

- Current progress: As per WID RP-243244, LMF-based AI/ML Positioning for Case 2b is a second priority use case, RAN2 has not made any discussion or agreement on Case 2b so far.

- RAN2 future work plan: Whether to support the second priority use cases is dependent on RAN1, RAN2 would firstly wait for RAN1 discussion before starting any potential work on Case 2b.

2. Actions:

To SA WG1

ACTION: RAN2 kindly asks SA2 to take the above feedback for AI/ML positioning use cases into account.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #129bis	7 - 11 April 2024					TBD, CN

TSG RAN2 Meeting #130		19 - 23 May 2024					Malta, EU