3GPP TSG-RAN WG2 Meeting #129                              	  R2-2500209

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Discussion on conditional LTM

Document for:	Discussion and Decision

Introduction

In latest meeting, RAN2 initially discussed conditional LTM and the following agreements were reached:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we discuss the open issues for supporting conditional LTM.

Discussion

Conditional LTM is introduced in Rel-19 which aims to achieve high robustness by enabling the procedure to be executed without explicit signalling exchange with source cell beforehand. As indicated in WID, conditional LTM is limited to intra-CU scenario, which is same as Rel-18 LTM. In the following sections, we will analyse the open issues for supporting CLTM.

Early sync

In previous meeting, it has agreed that PDCCH ordered early TA acquisition is supported for conditional LTM. And early sync with RA response is supported for CLTM as there is no LTM cell switch MAC CE delivered to UE. Regarding the RA response format, it has agreed the network can inform the candidate cell’s TA information to UE via new MAC CE. Besides TA value, the candidate configuration index should be also present to differentiate the candidates in case the early sync procedure has been triggered to multiple candidates.

The content of early sync response MAC CE includes:

- candidate configuration index

- TA value

In legacy RACH procedure, RAR window is used for RAR monitoring, i.e. upon preamble transmission, UE start RAR window and monitoring PDCCH with RAR window. While for Rel-18 PDCCH ordered early sync, UE assumes the Random Access procedure is completed upon transmission of the preamble as there is no RA response expected. For Rel-19 early sync with RAR, it is questionable whether the RAR reception relies on a RAR window like timer.

|  | Pros | Cons |
| --- | --- | --- |
| With RAR window like timer | Legacy RAR window can be reused. | - Spec impact to differentiate R18 and R19 early sync procedure is required. - RACH to serving/other candidate may be postponed due to RAR window. |
| Without RAR window like timer | - The Rel-18 early sync procedure can be reused without spec impact. - RACH to serving/other candidate can be triggered without waiting RAR window. | TA maintenance may not be aligned between UE and NW. |

RAN2 decide whether to rely on RAR window for early sync response MAC CE reception.

Besides PDCCH ordered early sync, there is another alternative for triggering the early sync procedure, i.e. event triggered early sync. UE can evaluate the candidate cell/RSs and trigger early sync to a fulfilled cell/beam to acquire the TA value. In R19 inter-CU LTM, companies also reach consensus that the event configuration can be used for early sync scenario since UE have the real-time L1 measurement of the candidate cells/RSs. UE can trigger the early sync to candidate cell(s) as long as the candidate fulfils the condition.

Event triggered early sync to candidate cell(s) is supported for CLTM.

Another open issue is that how the TA maintenance for candidate cell is supported. For serving cells, maintenance of uplink time alignment relies on a TA timer that is associated with a TAG. UE starts/restarts the corresponding TA timer if TA is indicated by NW. For early sync procedure, it is agreed the candidate cell TA is maintained by a new timer. And if early sync response MAC CE including TA information is provided to UE, the TA maintenance associated to candidate cell should be studied.

One straightforward way is to reuse TAG for TA indication and maintenance. Similar as serving cell TA maintenance, UE starts or restarts the timeAlignmentTimer associated with this TAG when a timing advance command is received in a random access response message for a candidate cell belonging to a TAG. In that way, TAG ID should be unique at least among serving cell(s) and candidate cells. And preparation on the TAG ID assignment/coordination is also required during HO preparation period.

Another way is to maintain the TA of candidate cell in the granularity of cell, i.e. the TA timer is maintained per candidate cell. And the TA timer is started upon reception of the early sync response MAC CE. Upon CLTM execution or other HO execution, UE shall stop all running TA timers which maintained for the CLTM candidate cells and start the corresponding TA timer associated with PTAG of the target cell.

For CLTM candidates, the early sync TA timer is maintained per candidate cell.

The TA timer is started upon reception of the early sync response MAC CE.

The TA timer(s) is stopped upon cell switch execution.

The TA timer(s) is stopped upon the corresponding candidate cell is released.

Execution condition configuration

In [POST128][107][MOB] LTM and CLTM RRC running CR, there are few FFSs regarding the CLTM execution configuration. The first issue is whether L1 and L3 execution conditions can be configured simultaneously or not for a candidate. In our understanding, L3 is a complementary solution in case UE capability does not support L1 measurement/event-triggered reporting, or NW expect a robust CLTM execution. Neither of the two cases provide strong motivation to support both L1 and L3 execution conditions to be configured simultaneously. And support both L1 and L3 execution conditions will lead more measurement overhead for UE. Single execution condition type is sufficient for CLTM execution evaluation for a candidate.

Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

For a candidate, single execution condition type (either L1 or L3 execution condition) is configured to UE.

Another FFS related to the CLTM execution is that whether we re-use events specified for the event-triggered L1 measurements or we specify events specifically for CLTM.

Editor’s Note: FFS whether we re-use events specified for the event-triggered L1 measurements or we specify events specifically for CLTM.

In Rel-16, CHO execution condition reuses the principle of the L3 event triggered measurement reporting, and a new set of event IEs are introduced with prefix “cond” to assist UE for differentiation. For CLTM, we can follow same way as in CHO since the situation is same. For example, reuse events specified for the event-triggered L1 measurements for CLTM execution condition.

Reuse events specified for the event-triggered L1 measurements for CLTM execution condition.

RACH-less CLTM execution

As in NW triggered intra-CU LTM, both RACH-based and RACH-less cell switch can be supported for CLTM execution. For RACH-less CLTM, as the cell switch execution is triggered by UE itself, NW may not be able to schedule a dynamic grant to UE immediately upon cell switch execution. In earlier meeting, CG based RACH-less CLTM is supported, and there is one FFS on whether the DG based RACH-less CLTM should also be supported. To support DG based RACH-less CLTM, one essential issue is to align the beam info between UE and NW, that is, NW have to know whether the UE is accessing the cell and provide the dynamic grant on the correct beam that UE is monitoring on. The beam info alignment can be resolved by introducing additional indication info between UE and NW, while one may argue that the additional beam indication is against with the motivation of supporting CLTM.

To be simple, we prefer to only support CG based RACH-less for CLTM.

Only CG based RACH-less CLTM is supported if TA is available. DG based RACH-less is not supported for CLTM.

And for CG resource selection for RACH-less CLTM, UE can follow the way in RACH-less HO in NTN and mIAB. UE selects a configured uplink grant for initial uplink transmission according to the configured threshold.

Threshold based TCI state selection is used for RACH-less CLTM.

The CG resources associated with the selected TCI state is applied for first UL transmission.

LTM, CLTM and L3 HO coexistence

The issue on coexistence between different types of HO mechanism should also be studied. As shown below, Rel-18 LTM support the coexistence with all other L3 HO mechanisms except DAPS.

| Coexistence(Y/N) | Normal HO | DAPS | CHO | CPAC | SCPAC |
| --- | --- | --- | --- | --- | --- |
| LTM | Y | N | Y | Y | Y |

For CLTM, we can follow similar agreements made in Rel-18, CLTM can be coexisted with other L3 HO mechanism.

CLTM, LTM and L3 HO coexistence are supported.

CLTM and DAPS coexistence is not supported.

- In TS38.300, how UE performs the condition evaluation for CHO is defined once the L3 HO is triggered.

#### 9.2.3.4	Conditional Handover

##### 9.2.3.4.1	General

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed.

For CLTM, the start/stop condition for execution condition evaluation should be same as CHO, which is that UE starts evaluating the execution condition(s) upon receiving the CLTM configuration, and stops evaluating the execution condition(s) once a handover is executed.

Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration) or LTM cell switch command MAC CE, the UE executes the HO procedure as described in clause 9.2.3.2 or LTM cell switch procedure as described in clause 9.2.3.5, regardless of any previously received CHO configuration.

And if any HO command/ LTM cell switch command is received before any CLTM execution condition is satisfied, UE shall execute the received HO command/LTM cell switch command, regardless of any previously received CLTM configuration.

UE starts evaluating the execution condition(s) upon receiving the CLTM configuration, and stops evaluating the execution condition(s) once a handover is executed.

Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

CLTM and mTRP coexistence

In R18, LTM support the coexistence with MIMO 2TA. UE determines the TA group of the accessed TRP according to the pointer associated with the indicated TCI state. For CLTM, we understand there is no need to prohibit a candidate cell configuring mTRP.

Currently we have agreed NW triggered early sync procedure and early TCI state activation/deactivation are supported for CLTM, while CLTM execution is triggered by UE based on pre-configured execution conditions. It should further study on how to align the beam of early sync, TCI state and CLTM execution since UE and NW may not be synced on the selected TRP/TAG.

Support the coexistence of MIMO two TA and CLTM.

Further study the beam alignment issue for CTM execution, early sync, TCI activation

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	The content of early sync response MAC CE includes:

	candidate configuration index

	TA value

Proposal 2	RAN2 decide whether to rely on RAR window for early sync response MAC CE reception.

Proposal 3	Event triggered early sync to candidate cell(s) is supported for CLTM.

Proposal 4	For CLTM candidates, the early sync TA timer is maintained per candidate cell.

Proposal 5	The TA timer is started upon reception of the early sync response MAC CE.

Proposal 6	The TA timer(s) is stopped upon cell switch execution.

Proposal 7	The TA timer(s) is stopped upon the corresponding candidate cell is released.

Proposal 8	For a candidate, single execution condition type (either L1 or L3 execution condition) is configured to UE.

Proposal 9	Reuse events specified for the event-triggered L1 measurements for CLTM execution condition.

Proposal 10	Only CG based RACH-less CLTM is supported if TA is available. DG based RACH-less is not supported for CLTM.

Proposal 11	Threshold based TCI state selection is used for RACH-less CLTM.

Proposal 12	The CG resources associated with the selected TCI state is applied for first UL transmission.

Proposal 13	CLTM, LTM and L3 HO coexistence are supported.

Proposal 14	CLTM and DAPS coexistence is not supported.

In TS38.300, how UE performs the condition evaluation for CHO is defined once the L3 HO is triggered.

Proposal 15	UE starts evaluating the execution condition(s) upon receiving the CLTM configuration, and stops evaluating the execution condition(s) once a handover is executed.

Proposal 16	Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Proposal 17	Support the coexistence of MIMO two TA and CLTM.

Proposal 18	Further study the beam alignment issue for CTM execution, early sync, TCI activation

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx