3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500250

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on UE-initiated Beam Reporting and CSI Enhancement

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- 1. Introduction

For Rel-19 MIMO, the following subjects about UE-initiated beam reporting and CSI enhancement were outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we will further discuss the detail spec impacts for UE-initiated beam reporting and CSI enhancement.

- 2. UE-initiated beam reporting

RAN1 has made following progress and decisions on the UE-initiated beam reporting.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

RAN1 have already discussed for several meetings on the details of event evaluation/triggering, including whether to capture the UE initiated beam reporting in MAC or PHY layer. However, RAN1 has not concluded if the triggering event determination is handled at PHY or MAC layer.

During RAN2#128 meeting, RAN2 made some discussion on whether the UE initiated beam reporting is handled in MAC or PHY layer, but there was no conclusion. From our perspective, the whole procedure of UE initiated beam reporting is in PHY layer including triggering event, report procedure and so on, which is different with RLF procedure. The measurement of BFR is reported via MAC CE and the counting is operated in RAN1, while for the case of UEIBM, the RS for current beam is implicitly derived from indicated TCI state and the report is via UCI. Moreover, the detail procedure and spec impacts are still under discussion in RAN1. Thus, we suggest waiting more RAN1 progress on the UE initiated beam reporting.

Observation 1: The whole procedure of UE initiated beam reporting is in PHY layer including triggering event, report procedure and so on, which is different with RLF procedure.

Proposal 1: Suggest RAN2 to wait more RAN1 progress on the UE initiated beam reporting.

- 3. CSI enhancement

During RAN1#119 meeting, RAN1 sent an LS[2] with a revised list of RRC parameters. Among these RRC parameters, there are some parameters which need to be discussed in RAN2 as following.

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Value range | Comment |
| --- | --- | --- | --- | --- | --- |
| CSI-CJTC | CSI-AperiodicTriggerState | delayOffsetCompensation | New | {enabled} | It is up to RAN2 whether this paremeter is located inside or outside "CSI-AssociatedReportConfigInfo" |
| CSI-CJTC | CSI-AperiodicTriggerState | triggeringScheme | New | {separate, joint} | Depending on whether parameter "delayOffsetCompensation" is located inside or outside "CSI-AssociatedReportConfigInfo" within a trigger state IE (up to RAN2), this parameter may or may not be needed |

The following agreement for the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting was made in RAN1#119 meeting.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, when a single CSI trigger state associated with >1 Rel-18 Type-II CJT CSI reports, the 1-bit indicator per CSI trigger state applies to all the Rel-18 Type-II CJT CSI reports linked with the CJTC Dd report(s).

According to the above RAN1 agreement, the 1-bit indicator, i.e., delayOffsetCompensation, applies to ALL the Rel-18 Type-II CJT CSI reports linked with the CJTC Dd report(s), when a single CSI trigger state associated with >1 Rel-18 Type-II CJT CSI reports. Thus, we suggest this parameter, i.e., delayOffsetCompensation, is located under CSI-AperiodicTriggerState and outside of "CSI-AssociatedReportConfigInfo", so that it can apply to all the Rel-18 Type-II CJT CSI reports, which is simple and saving more ASN.1 coding bits.

Proposal 2: The parameter, i.e., delayOffsetCompensation, is located under CSI-AperiodicTriggerState and outside of "CSI-AssociatedReportConfigInfo".

Regarding the triggeringScheme, our suggestion is this parameter is not needed. Based on the above RAN1 agreement, the delayOffsetCompensation is only applied for the separate trigger, which includes two trigger states. Each trigger state actives one CSI-AssociatedReportConfigInfo, for example, trigger state1 is CJTC and trigger state2 is CJT. The delayOffsetCompensation is under the trigger state2 and linkedCJTCReport is also configured in the AssociatedReportConfigInfo of trigger state2 to link CJTC report when codebook type is set to eTypeII-CJT as following.

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Value range | Description |
| --- | --- | --- | --- | --- | --- |
| CSI-CJTC | CSI-ReportConfig | linkedCJTCReport | New | CSI-ReportConfigID | Linked CJTC report when codebook type is set to eTypeII-CJT |

For the joint trigger, one trigger state is to active two CSI-AssociatedReportConfigInfo(s) corresponding to CJTC and CJT, separately. According to RAN1 agreement, UE should perform delay offset compensation based on the latest linked CJTC Dd report in the same slot on PUSCH, which delayOffsetCompensation is absent. Thus, from RAN2 point of view, it’s not necessary to use one RRC parameter to explicitly indicate the two triggering schemes, i.e., separate trigger and joint trigger. It’s clear to differentiate the two triggering schemes using some other parameters implicitly.

Proposal 3: The parameter, i.e., triggeringScheme, is not needed.

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Value range | Comment |
| --- | --- | --- | --- | --- | --- |
| CSI-CRI | CSI-ReportConfig | reportQuantity-r19 | New | ‘cri-RI-PMI-CQI’, ‘cri-RI-LI-PMI-CQI’ | It is up to RAN2 whether the legacy reportQuantity is reused for this or not. If reused, this parameter is not needed |

Regarding the above RRC parameter reportQuantity-r19, we prefer to reuse the legacy parameter reportQuantity. The legacy parameter reportQuantity is as following.

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

Since the value range of legacy reportQuantity also includes ‘cri-RI-PMI-CQI’ and ‘cri-RI-LI-PMI-CQI’ as above, the legacy reportQuantity can be reused for Rel-19. Thus, the parameter reportQuantity-r19 is not needed.

Proposal 4: The legacy parameter reportQuantity can be reused for Rel-19 MIMO. The parameter reportQuantity-r19 in the list of RRC parameters is not needed.

- 4. Conclusion

This paper discusses the detail spec impacts for UE-initiated beam reporting and CSI enhancement. The contribution of this paper is summarized as follows.

Observation 1: The whole procedure of UE initiated beam reporting is in PHY layer including triggering event, report procedure and so on, which is different with RLF procedure.

Proposal 1: Suggest RAN2 to wait more RAN1 progress on the UE initiated beam reporting.

Proposal 2: The parameter, i.e., delayOffsetCompensation, is located under CSI-AperiodicTriggerState and outside of "CSI-AssociatedReportConfigInfo".

Proposal 3: The parameter, i.e., triggeringScheme, is not needed.

Proposal 4: The legacy parameter reportQuantity can be reused for Rel-19 MIMO. The parameter reportQuantity-r19 in the list of RRC parameters is not needed.

- 5. Reference

- RP-242394 WID revision: NR MIMO Phase 5

- R2-2500008, LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 (R1-2410758; contact: Samsung)