# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500256
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Discussion on on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1], the WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

OD-SSB was discussed in RAN2#127 [2] and RAN2#127b [3]. And below agreements were made.

Agreements on OD-SSB in RAN2#127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell in RAN2#127b

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we continue to discuss RAN2 impacts of OD-SSB based on above agreements.

# 2 Discussion

RAN2#127 agreed to first study Scenario 2/2A and wait RAN1 conclusion on Scenario 3A/3B [2].

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

We provide Figure.1 to illustrate Scenario 2/2A for ease of followed discussion.

Figure.1 Illustration of considered scenarios (scenario 2/2A) of OD-SSB

Meanwhile, RAN1 also agreed to support Case 1 and Case 2, which are illustrated in Figrue.2 for ease of followed discussion.

- Case 1: No always-on SSB on the SCell.

- Case 2: Always-on SSB is also periodically transmitted on the SCell besides OD-SSB.

Figure. 2 Illustration of Case 1 and Case 2 of OD-SSB

## 2.1 Further details of OD-SSB MAC CE

In RAN2#127b [3], below high-level agreements on OD-SSB MAC-CE format were made:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Some detailed design options on MAC-CE format were discussed during RAN2#127b [3]. However, as RAN1 is still discussing what information needs to be included in OD-SSB MAC-CE, no conclusion was made. The related RAN1 progress includes below agreements:

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Although RAN1 design is not finished, we think some high level principle of OD-SSB MAC-CE design can be discussed for progress.

First, we would like to discuss how to indicate the SCell(s) with OD-SSB activation. In RAN2#127b [3], there are mainly three kinds of solutions:

- Alt-1: Introduce a fixed sized MAC-CE with a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE)

- Alt-2: Introduce a variable sized MAC-CE with explicit Serving Cell ID(s) with activated OD-SSB

- Alt-3: Introduce a fixed sized MAC-CE with only one explicit Serving Cell ID with activated OD-SSB (i.e. multiple MAC-CEs are needed if more than one serving cells have activated OD-SSB)

We prefer Alt-1 due to below reason:

- Alt-3 is precluded by below RAN2#127b agreement:

NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- Alt-2 has benefit of less overhead only when the number of SCells with activated OD-SSB is small. When the SCell number is large, its overhead is higher than Alt-1. Meanwhile, MAC-CE with variable size will bring extra complexity in UE processing.

Thus, we propose:

Proposal 1: The OD-SSB MAC-CE has a fixed size and includes a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

Please note that legacy SCell A/D MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells, according to TS 38.321 [7]. Similarly, the new OD-SSB MAC-CE should also support the two formats.

Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Then, we discuss how to include other information (e.g. periodicity of OD-SSB) in the new MAC-CE. As can be observed, RAN1 agreed to include periodicity of OD-SSB in MAC-CE, but other parameters were not precluded. In RAN2#127b [3], there are mainly two kinds of solutions to include periodicity of OD-SSB in MAC-CE:

- Alt-1: Include actual values of OD-SSB periodicities (5/10/20/40/80/160ms) in MAC-CE

- Alt-2: Include configuration indices of candidate SSB periodicities in RRC in MAC-CE

We prefer Alt-2 due to below reasons:

- OD-SSB periodicity is expected to be much shorter than always-on SSB. Thus, supporting all possible SSB periodicity values are not necessary.

- Alt-2 is more forward compatible: other agreed OD-SSB information can also be included in RRC together with periodicity under the same configuration index.

Proposal 3: Considering forward compatibility, configuration indices of candidate SSB periodicities in RRC (rather than actual values of SSB periodicities) are included in the OD-SSB MAC-CE.

One example of OD-SSB MAC-CE with 7 SCells and up to 2 candidate values of OD-SSB periodicity can be found in Figure.1, where Bi (i=1-7) indicates the activation status of OD-SSB in SCell with SCellIndex i and Ci (i=1-7) indicates whether 1st or 2nd candidate value of OD-SSB periodicity is applied in SCell with SCellIndex i.

Figure.1 Example of OD-SSB MAC-CE with 7 SCells and up to 2 candidate values of OD-SSB periodicity

## 2.2 L3 measurement on OD-SSB

L3 measurement on OD-SSB was discussed in RAN2#127b [3], but the discussion is controversial. In this section, we first discuss case 2 (i.e. whether always-on SSB and/or OD-SSB are measured), and then discuss L3 measurement framework. Finally, we discuss spec impact on how to support case 1.

### 2.2.1 How to support L3 measurement in case 2

In RAN2#127b [3], the following two options were identified, but no conclusion can be made due to divergent views.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Meanwhile, we noticed that RAN4#112b also discussed this issue and identified the following 3 scenarios for study [4]:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

We think whether always-on SSB and/or OD-SSB are measured can be left to RAN4 because RAN4 need to first decide whether to specify new measurement requirements in OD-SSB case 2.

Proposal 4: RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

### 2.2.2 L3 measurement framework

In RAN2#127b [3], the following two options were identified, but no conclusion can be made due to divergent views.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

We think the key issue is whether to follow legacy SMTC based L3 RRM framework:

- Option 1 keeps the legacy SMTC based L3 RRM framework.

  - Whether / how different measurement configurations (e.g. SMTC or MO) can be adapted when OD-SSB is transmitted can be further discussed as next step, which is a kind of stage 3 issue.

- Option 2 challenges the legacy SMTC based L3 RRM framework, i.e. based on OD-SSB transmission pattern rather than SMTC configuration

Thus, we first discuss whether to keep legacy SMTC based L3 RRM framework, and then discussed required specification changes.

#### 2.2.2.1 Whether follow legacy SMTC based L3 RRM framework

We prefer Option 1 because Option 2 will bring significant cross-WG specification impacts:

- RAN2 impacts: the UE autonomous OD-SSB measurement outside of SMTC will challenge the per-frequency L3 RRM framework and introduce significant RAN2 specification impacts.

  - Option 2 needs to at least modify below highlighted specification in Section 5.5.2.10 of TS 38.331 [5].

5.5.2.10	Reference signal measurement timing configuration

…omit unrelated part...

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

  - Option 2 essentially intends to change L3 RRM framework from per-frequency configuration to per-cell configuration. It will introduce at least the following spec changes in TS 38.331.

    - As Section 5.5 of 38.331 is drafted with assumption on per-frequency L3 measurement configuration, the loop structure is used to independently process measurements for each measId. Option 2 will need spec changes to support measurement with per-cell configuration.

    - In existing L3 framework, one MeasId links one per-frequency measurement configuration (i.e. MO) with one report configuration (i.e. ReportConfigNR). Then, it is not clear how to link per-cell measurement configuration with report configuration. Further spec changes are needed.

- RAN4 impacts: the UE autonomous OD-SSB measurement outside of SMTC will lead to significant RAN4 specification changes in TS 38.133 [7] because all L3 measurement requirements are specified with SMTC periodicity (e.g. SCell activation latency).

  - Please note that RAN4 has clearly indicated it in their LS to RAN2 (R4-2416911) [8].

However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

From section 5.1.1 of TS 38.215:

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. …omit the following part... |
| --- | --- |

RAN1 impacts: the UE autonomous OD-SSB measurement outside of SMTC will also result in multiple RAN1 specification changes in TS 38.215 [9]. One example of required spec change can be found highlighted:

- Misaligned understanding with NW: the UE autonomous OD-SSB measurement outside of SMTC may lead to misalignment on actually measured SSB between UE and NW, especially when OD-SSB is partially overlapped or has small timing gap with SMTC.

- UE power saving: it mandates the UE to perform L3 measurement on OD-SSB outside of SMTC, which is bad for UE power saving.

Based on above analysis, we can summarize them as Observation 1-Observation 4.

Observation 1: To support UE autonomous OD-SSB measurement outside of SMTC, it essentially challenges the per-frequency L3 RRM framework and will introduce significant RAN2 specification impacts:

  - Allow UE to perform L3 measurement based on OD-SSB outside SMTC in Section 5.5.2.10 of TS 38.331.

  - As existing section 5.5 of 38.331 is drafted with assumption on per-frequency L3 measurement configuration, spec changes are needed for per-cell measurement configuration.

  - Spec changes are needed to link per-cell measurement configuration with one report configuration.

Observation 2: The UE autonomous OD-SSB measurement outside of SMTC will lead to significant RAN4 specification changes because all L3 measurement requirements are specified with SMTC configuration.

Observation 3: The UE autonomous OD-SSB measurement outside of SMTC will also result in multiple RAN1 specification changes in TS 38.215.

Observation 4: The UE autonomous OD-SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when OD-SSB is partially overlapped or has small timing gap with SMTC.

Thus, we propose to keep legacy SMTC based L3 RRM framework.

Proposal 5: Keep SMTC based L3 RRM framework for OD-SSB case 1 and case 2 (i.e. not allow UE autonomous SSB measurement outside of SMTC).

#### Whether multiple configurations are needed

In legacy L3 RRM based on SSB, the NW need to configure UE SMTC in MeasObjectiveNR. Furthermore, according to Section 5.5.2.10 of TS 38.331 [5], the UE shall not consider SSB outside the SMTC occasion for RRM measurements. SMTC is a periodical window because it is assumed that SSB is always transmitted in periodical way. However, one single SMTC may not cover all the required SSB(s) to perform L3 measurements because the assumption of periodic SSB transmission is NOT valid anymore for OD-SSB in case 2.

We provide an example illustrated in Figure.2, where the UE is configured with one SMTC whose window covers the always-on SSB. However, when OD-SSB is triggered after T1, the configured SMTC window can’t cover OD-SSB. Thus, although the UE may know when OD-SSB starts transmission via activation MAC-CE, it can’t perform L3 measurements towards OD-SSB because OD-SSB is not covered within its configured SMTC. One may argue that NW implementation may resolve this issue (e.g., via proper configuration of SMTC by considering OD-SSB pattern). However, please note that SMTC is configured per frequency rather than per cell, which makes NW difficulty to configure one single SMTC window to cover all SSBs without causing negative impacts to legacy UE. For example, NW may configure a SMTC window with short periodicity to cover both always-on SSB and OD-SSB, but it will lead to UE extra power consumption, especially for the legacy UEs.

Figure.2 Illustration of issue in case 2: one single SMTC can only cover always-on SSB but can’t cover OD-SSB. So, UE can’t perform L3 measurements towards OD-SSB, although it knows when OD-SSB is triggered.

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover both OD-SSB and always-on SSB without causing negative impacts to legacy UE (e.g. extra power consumption if NW configures a dense SMTC to cover both types of SSBs).

In legacy, SMTC can only be reconfigured by RRC signaling. Because OD-SSB can be activated / deactivated via MAC-CE, we think one single SMTC can’t quickly align with the dynamic change of OD-SSB periodicity. Thus, it is necessary to allow the UE to dynamically adapt between different SMTC configurations depending on whether OD-SSB is activated / deactivated.

On detailed RRC signaling specification change, we think the following two solutions can be further discussed:

- Alt-1: OD-SSB dedicated SMTC configuration is included in the same MO of always-on SSB

- Alt-2: OD-SSB dedicated SMTC configuration is included in an OD-SSB dedicated MO.

Proposal 6: For case 2, introduce dynamic adaptation between different SMTC configurations depending on whether OD-SSB is activated / deactivated. FFS how to configure OD-SSB dedicated SMTC configuration (e.g. in the same MO of always-on SSB or in an OD-SSB dedicated MO).

During online discussion of RAN2#127b [3], it was raised the question if adaptation of SMTC will lead to RAN4 requirement changes. We want to clarify two aspects:

- There seems misunderstanding that SMTC adaptation needs to activate multiple SMTCs simultaneously. But we understand that only one SMTC is activated at one time (e.g., either legacy SMTC when only always-on SSB is transmitted, or OD-SSB dedicated SMTC when OD-SSB is activated).

- RAN2 had already introduced different features with multiple SMTCs (e.g. smtc2, smtc2-LP, smtc3list and smtc4list) as highlighted in below copy of Section 5.5.2.10 of TS 38.331 [5]. And RAN4 had specified their requirements in TS 38.133 [7]. So, we think it is “business as usual”.

5.5.2.10	Reference signal measurement timing configuration

…omit unrelated part...

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the IAB-MT shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

Observation 6: Only one SMTC is activated at one time (e.g., either legacy SMTC when only always-on SSB is transmitted, or OD-SSB dedicated SMTC when OD-SSB is activated).

Observation 7: RAN2 had already introduced different features with multiple SMTCs (e.g. smtc2, smtc2-LP, smtc3list and smtc4list). And RAN4 had specified their requirements in TS 38.133. Thus, it is “business as usual”.

### 2.2.3 How to support L3 measurement in case 1

In RAN2#127 [2], it was agreed that the UE doesn’t expect to measure SSB when OD-SSB is deactivated, i.e. the UE measures SSB only when on-demand SSB is activated.

Agreements on OD-SSB in RAN2#127

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

From Section 5.3.3.1 of TS 38.331

omit unrelated part

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

omit unrelated part

From L3 measurement perspective, one key difference of case 1 from case 2 is to allow the UE not to perform measurement for servingCellMO when OD-SSB transmission is deactivated. According to Section 5.3.3.1 of TS 38.331 [5], the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO):

This is a basic principle ever since from LTE L3 RRM which means that the dynamic activation/deactivation for serving cell measurement is not allowed. It seems to be conflicted with above RAN2#127 agreement.

Observation 8: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

To resolve this issue, we think the same solution of case 2 in Proposal 6 can be reused. Please note that the UE doesn’t perform L3 measurements based on SSB outside SMTC according to TS 38.331 [5]. Thus, RAN2#127 agreement can be implemented by keeping absence of the SMTC configuration for legacy always-on SSB because the UE will regard all SSBs are outside SMTC window and thereby not perform L3 measurement. Thus, we propose:

Proposal 7: For case 1, reuse the same SMTC configuration adaptation solution of case 2 (e.g. the SMTC configuration for legacy always-on SSB is absent).

Figure.3 Illustration of unified solution for case 1 and case 2

We believe that this is the unified solution with the least specification impacts, and we also provide an example in Figure. 3 to help illustration.

# 3 Conclusion

In this contribution, we further discuss RAN2 specification impacts of on-demand SSB for SCell. Our observations are:

Observation 1: To support UE autonomous OD-SSB measurement outside of SMTC, it essentially challenges the per-frequency L3 RRM framework and will introduce significant RAN2 specification impacts:

  - Allow UE to perform L3 measurement based on OD-SSB outside SMTC in Section 5.5.2.10 of TS 38.331.

  - As existing section 5.5 of 38.331 is drafted with assumption on per-frequency L3 measurement configuration, spec changes are needed for per-cell measurement configuration.

  - Spec changes are needed to link per-cell measurement configuration with one report configuration.

Observation 2: The UE autonomous OD-SSB measurement outside of SMTC will lead to significant RAN4 specification changes because all L3 measurement requirements are specified with SMTC configuration.

Observation 3: The UE autonomous OD-SSB measurement outside of SMTC will also result in multiple RAN1 specification changes in TS 38.215.

Observation 4: The UE autonomous OD-SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when OD-SSB is partially overlapped or has small timing gap with SMTC.

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover both OD-SSB and always-on SSB without causing negative impacts to legacy UE (e.g. extra power consumption if NW configures a dense SMTC to cover both types of SSBs).

Observation 6: Only one SMTC is activated at one time (e.g., either legacy SMTC when only always-on SSB is transmitted, or OD-SSB dedicated SMTC when OD-SSB is activated).

Observation 7: RAN2 had already introduced different features with multiple SMTCs (e.g. smtc2, smtc2-LP, smtc3list and smtc4list). And RAN4 had specified their requirements in TS 38.133. Thus, it is “business as usual”.

Observation 8: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

Based on above observations, our proposals are:

MAC-CE format

Proposal 1: The OD-SSB MAC-CE has a fixed size and includes a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 3: Considering forward compatibility, configuration indices of candidate SSB periodicities in RRC (rather than actual values of SSB periodicities) are included in the OD-SSB MAC-CE.

L3 measurement in NES case 2

Proposal 4: RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

L3 RRM framework

Proposal 5: Keep SMTC based L3 RRM framework for OD-SSB case 1 and case 2 (i.e. not allow UE autonomous SSB measurement outside of SMTC).

Proposal 6: For case 2, introduce dynamic adaptation between different SMTC configurations depending on whether OD-SSB is activated / deactivated. FFS how to configure OD-SSB dedicated SMTC configuration (e.g. in the same MO of always-on SSB or in an OD-SSB dedicated MO).

L3 measurement in NES case 1

Proposal 7: For case 1, reuse the same SMTC configuration adaptation solution of case 2 (e.g. the SMTC configuration for legacy always-on SSB is absent).

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#127, Chair Notes.

[3] RAN2#127b, Chair Notes.

[4] R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson.

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[7] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

[8] TS 38.215-v18.4.0, Physical layer measurements, 2024-1.

[9] R4-2416911, LS on SSB adaptation.