3GPP TSG-RAN WG2 Meeting #129                                    R2-2500305

Athens, Greece, Feb. 17th- 21st, 2025

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on A-IoT paging

Document for:	Discussion & Decision

- 1 Introduction

In RAN#106, the WI of A-IoT was approved with the following objective on AIoT paging.

A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In this contribution, we will share our views on the Paging design based on agreements in SI stage.

- 2 Discussion

Issue 1: identification on devices

The information about the identificaiton of devices are used to indicate which device(s) needs response to the AIoT paging message. According to the study during SI stage, the following information is considered:

- Identifier of a single AIoT device

- Group ID of multiple devices

- No identifier is included, which indicates all AIoT devices receiving the paging message should response

- Multiple identifers of multiple AIoT devices

As indicated by WID, the AIoT paging message can support single/multiple device IDs and no device ID. However, the detailed design relies on the further clarification from SA2:

- ID type: the CN can provide a list of device IDs (covering a single ID and multiple ID) or no ID. The remaining issue is whether the CN will provide the group ID or not. If group ID is provided, A-IoT paging message may need an ID type field.

- ID length: this determines the length of ID field in the AIoT paging message. In general, a single device ID should have a fixed length. The value of length needs SA2’s input. However, regardless of the length value, the length of ID field in AIoT paging message should be variable since it may contain zero/one/multiple IDs. Thus, the AIoT paging message should have a field of ID length. In addition, if the group ID can be provided to reader, whether the group ID and device ID have the same length or not also needs SA2’s inputs.

Proposal 1-1: RAN2 is kindly asked to agree that the Paging message can include zero/one/multiple device IDs, FFS on group ID

Proposal 1-2: RAN2 is kindly asked to agree that the A-IoT paging message includes the ID length field and the ID field, where the ID field includes zero/one/multiple device IDs. FFS on ID type field.

Proposal 1-3: ID information design needs SA2’s clarificaiton on the following aspects:

- Whehter the group ID is provided to the reader or not

- The length of a single device ID, and the length of group ID (if provided)

Issue 2: Information to avoid duplicate response from the device

To avoid the duplicated responses from the same device, some information can be included in the paging message. If such information is not changed, the device can skip the response as long as it has responded to the reader. Such information can be linked to the service request from the CN, e.g., once the CN triggers a new inventory service, the reader can send the (subsequent) AIoT paging message(s) to include the information linked to such service. We can call such information as service related information (name is FFS). All AIoT paging messages associated with the same service from the CN will include the same service related information. Thus, the design of such information depends on how the service request from CN is operated:

- Option 1: one service at a time

In this option, the reader will perform one service over A-IoT Uu interface at a time. After one service is completed, the reader can start another one. This option can simplify the A-IoT Uu interface, e.g., the reader needn’t handle the parallel service requests over A-IoT Uu interface, the device needn’t maintain multiple services. Meanwhile, such option allows a short service-related information in A-IoT paging message.

- Option 2: multiple services at a time

In this option, the reader may trigger A-IoT operations over A-IoT Uu interface for multiple services. Thus, different A-IoT paging messages may contain different service related information. If the A-IoT device is running multiple services at the same time, it can response different A-IoT paging messages containing different service related information. Such option allows the flexible A-IoT service operations. However, it may require more bits for the service-related information.

Apparently, Option 1 is by default supported. For the A-IoT paging message design, the impact of the support of Option 2 is mainly on the length of service-related information. However, whether to support option 2 or not is SA2’s decision. So, in the first meeting of this WI, RAN2 can take option 1 as the starting point for the service-related information design. The length of such information is FFS depending on the SA2’s inputs.

Proposal 2-1: RAN2 is kindly asked to agree that the A-IoT Paging message contains the service-related information to avoid the duplicated response from the device.

Proposal 2-2: RAN2 is kindly asked to agree that the design of the service-related information is started from the case that one service is operated at a time, and FFS on the length of such information (depends on SA2 on the support of multiple services at a time).

Proposal 2-3: RAN2 is kindly asked to consult SA2 on whether to support multiple services at a time.

Issue 3: Resource configuration information

The resource configuration is used by the device(s) to determine the resource used to transmit the Msg1 of random access procedure. Such resource can be an access occasion or a set of resource occasions. Each access occasion represents a time-frequency resource. The assignment of access occasion depends on the random access type triggered by A-IoT paging message:

- Type 1: Contention-free random access

In case that the A-IoT paging message includes a single device ID, the reader can assign one access occasion to such device so that it can perform the connection-free random access.

- Type 2: Content-based random access

In case that the A-IoT paging message does not include any device ID (i.e., trigger multiple devices to access), the reader can assign multiple access occasions to device(s) so that those devices can perform contention-based random access.

To support the above two types of random access, the A-IoT Paging message can include the number of access occasions.

Proposal 3-1: RAN2 is kindly asked to agree that the A-IoT paging message can include the number of access occasions.

As indicated in WID, the A-IoT Paging message should be designed to allow including multiple device IDs. In this case, either contention-free or contention-based random access can be configured. For contention-free random access, each device should be assigned a dedicated access occasion. RAN2 can discuss whether and how to support the resource configuration for contention-free random access of multiple devices.

Proposal 3-2: RAN2 is kindly asked to discuss whether and how to support the dedicated access occasion assignment for multiple devices in one A-IoT Paging message with intention of configuring the contention-free random access.

Issue 4: Paging message update

Among the AIoT paging messages associated with the same CN request, the configuration infomration in each message may be changed. Fig. 1 shows one example on the resource configuration change for different AIoT paging messages. Specifically, the CN request is to inventory 100 devices. At the beginning, the AIoT reader may need assign a long period (i.e., period 1) to collect responses since large number of devices cause more collisions. After Period 1, 80 devices have been inventoried. Thus, the AIoT devices can assign a short period (i.e., period 2) for inventory considering the small number of remaining devices.

Fig. 1 Configuration information change among AIoT paging messages

Proposal 4-1: for the same service, RAN2 is kindly asked to agree that the A-IoT paging message may contain different configuration information (e.g., resource configuration).

- 3 Conclusion

Based on the discussion above, we have the following observations and proposals:

Identification on devices

Proposal 1-1: RAN2 is kindly asked to agree that the Paging message can include zero/one/multiple device IDs, FFS on group ID

Proposal 1-2: RAN2 is kindly asked to agree that the A-IoT paging message includes the ID length field and the ID field, where the ID field includes zero/one/multiple device IDs. FFS on ID type field.

Proposal 1-3: ID information design needs SA2’s clarificaiton on the following aspects:

- Whehter the group ID is provided to the reader or not

- The length of a single device ID, and the length of group ID (if provided)

Information to avoid duplicate response from the device

Proposal 2-1: RAN2 is kindly asked to agree that the A-IoT Paging message contains the service-related information to avoid the duplicated response from the device.

Proposal 2-2: RAN2 is kindly asked to agree that the design of the service-related information is started from the case that one service is operated at a time, and FFS on the length of such information (depends on SA2 on the support of multiple services at a time).

Proposal 2-3: RAN2 is kindly asked to consult SA2 on whether to support multiple services at a time.

Resource configuration information

Proposal 3-1: RAN2 is kindly asked to agree that the A-IoT paging message can include the number of access occasions.

Proposal 3-2: RAN2 is kindly asked to discuss whether and how to support the dedicated access occasion assignment for multiple devices in one A-IoT Paging message with intention of configuring the contention-free random access.

Paging message update

Proposal 4-1: for the same service, RAN2 is kindly asked to agree that the A-IoT paging message may contain different configuration information (e.g., resource configuration).