3GPP TSG-RAN WG2 Meeting #129	R2-2500333

Athens, Greece 17th – 21st of February 2025

Agenda item:	7.2

Source:	Nokia

Title:	On the Remaining Rel-18 LTM Aspects and Corrections

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are addressing several remaining issues for Release 18 LTM, some of which have been already briefly discussed in 2024 without any resolution. We also address the topics raised in RAN4 liaison statements.

# 2	Discussion

## 2.1 	LTM PDCP data recovery

The topic has been explained in our previous paper [1] and discussed at RAN2#128 [2]:

R2-2410442 Miscellaneous Rel-18 LTM Aspects and Corrections Nokia discussion Rel-18 NR_Mob_enh2-Core Proposal 2: Adopt the TP in Annex B. - Noted. [MediaTek]: Intention is ok, but not happy with wording. Instead of “same RLC entity”, it would be better to associate with master key or secondary key. [LG]: In the current PDCP spec, it is specified that PDCP recovery is applied to the released DRB or re-established DRB, so thinks it can be already handled by PDCP. [Nokia]: PDCP does not know this situation, RRC should inform PDCP. [Samsung]: Understand with this proposal, it is to avoid redundant PDCP re-establishment. We still survive w/o this change. Do we really need this optimization in the late phase? [Huawei]: Agree with Samsung. For DC case, we do not handle all cases for optimization. [Xiaomi]: Agree with LG.

As can be seen in the aforementioned excerpt from the Chairman notes [2], the changes proposed by us have been found to be correct (at least the reason behind suggesting such correction). However, companies were not convinced these changes are necessary after Release 18 has been “finalized” and frozen. Among the others, it has been underlined that without our changes proposed in [1], the UE will perform additional PDCP re-establishment which is perhaps pointless, but nothing serious is broken due to that.

Observation 1: At RAN2#128 it has been noticed that without changes proposed in Annex B of R2-2410442, the UE will perform additional PDCP re-establishment. However, according to the companies, there is nothing seriously wrong with current UE behaviour.

Nevertheless, we believe the changes we suggest are quite simple and the benefits are clear. Let us explain the issue and resolution once more. 

For the LTM execution procedure, if the “ltm-NoResetID” is not the same as the “VarLTM-ServingCellNoResetID”.  UE resets the RLC bearers for the cell group that triggered the cell change. For the MCG LTM, only the MCG RLC bearers will be reset. However, UE triggers PDCP data recovery for all the DRBs that is part of the current UE configuration, as such this would include trigger PDCP recovery for all bearers including MN terminated SCG bearers or SN terminated MCG bearers [6].

1> if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID: 2> for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered: 3> if servedRadioBearer is set to drb-Identity: 4> after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config; 2> for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered: 3> after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config; 2> for each drb-Identity value that is part of the current UE configuration: 3> if this DRB is an AM DRB: 4> after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

Observation 2: UE triggers PDCP data recovery for all the DRBs that are part of the current UE configuration. This would include triggering PDCP recovery for all bearers including MN terminated SCG bearers and SN terminated MCG bearers.

Triggering PDCP data recovery for MN terminated SCG bearers is redundant. This is already captured in S-CPAC specification [6] by triggering the PDCP data recovery only for the bearers that are associated with a re-established RLC entity. Similar behaviour should be adopted for LTM procedure.

##### 5.3.5.13.8 Subsequent CPAC execution … 4> if there is an associated SCG RLC bearer in the selected subsequent CPAC candidate configuration that is part of the current UE configuration: 5> re-establish the SCG RLC entity as specified in TS 38.322 [4]; 4> if the RLC entity of the associated RLC bearer(s) is re-established; or 4> if an associated RLC bearer is released in the selected subsequent CPAC candidate configuration: 5> if the bearer is an AM DRB: 6> trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];

Proposal 1: Adopt the TP in Annex A.

## 2.2 RAN4 LS on fast RRC processing for LTM

In November 2024 RAN4 has responded to RAN2 LS in [3]. RAN4 has replied to three questions asked by RAN2 earlier in 2024 in the following way:

RAN2 Question 1: As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? RAN4: Yes. RAN2 Question 2: Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? RAN4: No. RAN4 has defined a specific rule for the feature of ltm-FastProcessingConfig-r18 to ensure that both UE and NW can be in sync regarding the early processing of the target LTM configuration. That said, the group finds it beneficial to provide such auxiliary information for efficient UE processing. RAN2 Question 3: Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells? RAN4: Yes. RAN4 also agreed that early TCI state activation command or PDCCH-order can be used to trigger early RRC decoding, as specified in the description of component TLTM_RRC-processing in section 6.3.1.3 of TS 38.133

As can be noticed in RAN4 responses, apart from the answer to Q1, the remaining two responses are a bit vague. Question 1 and corresponding answer is straightforward. Question 2 is actually the gist of the original LS – does the UE need to know the number of SpCells and SCells within LTM configurations before early ASN.1 decoding? RAN4 has responded “No”, but then also stated such auxiliary information is found to be beneficial.

Observation 3: As per RAN4 response LS in R4-2420218, the UE does not need to know the number of SpCells and SCells within LTM configurations before early ASN.1 decoding.

Observation 4: RAN4 finds it beneficial if such auxiliary information is provided.

Nevertheless, it has to be noted the intention here is to add a Release 18 functionality, while this 3GPP release has been frozen a while ago. Furthermore, as observed in RAN4 LS [3], this is not an essential correction but just an “auxiliary information” which one of the 3GPP working groups finds to be beneficial.

Observation 5: Release 18 has been frozen a while ago while the early ASN.1 decoding feature is fully functional even if the UE is not informed explicitly how many cells there are in each LTM configuration.

Thus, we suggest the following:

Proposal 2: RAN2 does not consider any further the addition of an explicit information regarding the number of SpCells and SCells within the LTM candidate configuration.

## 2.3 RAN4 LS on Feature Group 39-6 prerequisites

In November 2024 RAN4 has sent another LS to RAN2 [4], also with respect to ltm-FastProcessingConfig-r18. [4] is brief and simple. It indicates the prerequisite for ltm-FastProcessingConfig-r18 shall be removed:

RAN4 discussed the FG 39-6 “Fast processing of LTM candidate cell RRC configuration” further and agreed that the prerequisite of the feature group should be removed.

Observation 6: RAN4 in R4-2420125 informs RAN2 that the prerequisite for FG 39-6 should be removed.

In [5] the capability is defined as follows:

ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133 [5]. A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. UE shall set the capability values for maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18 consistently for all bands. These capability values represent the maximum number across all the supported bands. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5].

It is clearly stated that ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18 are the prerequisites of this capability.

Observation 7: As per TS 38.306, ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18 are the prerequisites of ltm-FastProcessingConfig-r18.

It is therefore suggested to agree changes shown in Annex B for TS 38.306.

Proposal 3: RAN2 adopts the changes shown in Annex B to implement what RAN4 suggested in R4-2420125.

## 2.4 On LTM recovery and the use of UE-based TA

A procedure in section 5.3.5.18.6 of  describes how the UE checks if the UE-based TA value is to be applied during LTM execution. It covers both the regular LTM execution (a text excerpt prior to “or” in the first 1> subclause) and LTM execution as a part of LTM recovery (i.e. without cell switching MAC CE, text excerpt after “or” in the first 1> subclause, highlighted in green):

[…] 1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID: 2> if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 3> replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID; 3> for each LTM-Candidate IE in ltm-Config: 4> if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 5> inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate; 4> else: 5> inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate; […]

It has been decided during Release 18 LTM design that LTM Recovery can be only CBRA-based. Thus, the use of ltm-UE-MeasuredTA-ID is not allowed if second part of the condition in 1> subclause quoted above is fulfilled.

Observation 8: The use of ltm-UE-MeasuredTA in the 5.3.5.18.6 shall not be allowed when LTM is executed due to the LTM candidate selection in accordance with 5.3.7.3.

Thus, we suggest updating 5> subclause of 5.3.5.18.6 in the way depicted in Annex B.

Proposal 4: Adopt the TP in Annex C to correct the ltm-UE-MeasuredTA issue identified in 38.331, section 5.3.5.18.6.

## 2.5 On DC LTM configured simultaneously in MCG and SCG

For Rel. 18 LTM, we have agreed that MCG and SCG LTM can be configured, but not triggered simultaneously.

RAN2#124 LTM for simultaneous PCell and PSCell change is not supported in Rel 18 RAN2#127bis RAN2 understands that if UE supports all ltm-MCG-NRDC-r18, ltm-MCG-IntraFreq-r18 and ltm-SCG-IntraFreq-r18, the UE is also able to support the configuration of LTM for MCG and LTM for SCG simultaneously.

In Section 5.3.5.18.1 in 38.331 [6] we have captured the following:

In NR-DC, the UE may receive two independent ltm-Config: - an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and - an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1. In case the UE receives two independent ltm-Config: - the UE maintains two independent ltm-Config; - the UE maintains two independent VarLTM-ServingCellNoResetID, one associated with each ltm-Config; - the UE maintains two independent VarLTM-ServingCellUE-MeasuredTA-ID, one associated with each ltm-Config; - the UE independently performs all the procedures in clause 5.3.5.18 for each ltm-Config and the associated VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID unless explicitly stated otherwise.

Moreover, in Section 7.3 in 37.340 we have captured the following:

For LTM operation, the MN indicates the maximum number of LTM candidate configurations the SN is allowed to configure for SCG LTM, to ensure that UE capabilities are not exceeded. The SN can also request the MN for a new value of the maximum number of allowed LTM candidate configurations to configure for SCG LTM, and it is up to the MN whether to accommodate the SN request. If the SN receives from the MN a new value for the maximum number of LTM candidate configurations, it is SN responsibility to ensure that its configured LTM candidate configurations to comply with the new limit. For the LTM related UE capabilities about L1 measurements requiring coordination between the MN and the SN, it is up to the MN to decide on how to resolve the dependency between MN and SN configurations and ensure that the UE capabilities are not exceeded. The MN then provides the resulting UE capabilities about L1 measurements usable for SCG configuration to the SN. The SN can also request a new value for such UE capabilities for L1 measurements, and it is up to the MN whether to accommodate the SN request. If the SN receives from the MN a new value for such UE capabilities for L1 measurements, it is SN responsibility to ensure that its configured L1 measurement configurations to comply with the new limit.

In 38.331, the above text is reflected in the following IE:

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

-- Text Omitted

[[

maxNumberLTM-CandidatesSCG-r18   INTEGER(0..maxNrofLTM-Configs-r18)                               OPTIONAL

]],

[[

allowedL1-MeasConfigNRDC-r18     L1-MeasConfigNRDC-r18                                            OPTIONAL

]]

-- Text Omitted

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

Looking at the above agreements and specifications text, it seems that simultaneous configuration for MCG and SCG LTM is possible. As long as both MCG and SCG LTM are configured at the same time for a UE, either MN or SN can trigger RACH-based LTM.

Observation 9: Rel. 18 supports simultaneous configuration, but not simultaneous triggering of RACH-based MCG and SCG LTM.

For Rel. 18 LTM, we have defined several enhancements, which are signaled from the UE to the NW through the UE capabilities, such as:

- Early DL synchronization (e.g., number of TCI states that can be activated for a UE)

- Early UL synchronization (e.g., number of cells for which early TA acquisition or UE-based TA acquisition can be performed)

- Early configuration decoding

In 38.306 examples of such capabilities are described as follows:

| rach-EarlyTA-Measurement-r18 Indicates the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE. Power ramping for PRACH retransmission based on PDCCH order indication. UE also supports dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s). A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ue-TA-Measurement-r18 Indicates whether the UE supports UE-based TA measurement by indicating the maximum number of candidate cells that the UE maintains the TA for. A UE supporting this feature shall also indicate the support of at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. | Band | No | N/A | N/A |

Considering the excerpts from the specification text, only LTM related capabilities about L1 measurements are exchanged between MN and SN. Thus, if both MCG and SCG LTM are simultaneously configured, and UE supports capabilities such as early UL/DL synchronization, early decoding, etc., neither MN nor SN can trigger additional procedures (e.g., early UL/DL synchronization). The reason for this is because MN does not know which UE capabilities are currently being configured from SN. For instance, consider the early TA measurements. Based on the capabilities highlighted in the above table, UE reports the maximum number of cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving LTM MAC CSC. In this case, MN does not know how the number of cells for which SN has triggered early TA acquisition, so it does not know if it can trigger procedure for cells configured as MCG LTM candidates.

Observation 10: Based on the current specification, if LTM is configured in both MCG and SCG, then it is not possible for MN or SN to trigger e.g., early synchronization since they are not aware of what the other node has triggered.

Considering that the true gains of LTM come from UEs supporting such capabilities (even if these are not mandatory features), we think that 3GPP should provide support for the cases when both MCG and SCG LTM are configured towards the UE.

Proposal 5: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells for which early synchronization procedures can be performed. Adopt the TP in Annex D in TS 38.331.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: At RAN2#128 it has been noticed that without changes proposed in Annex B of R2-2410442, the UE will perform additional PDCP re-establishment. However, according to the companies, there is nothing seriously wrong with current UE behaviour.

Observation 2: UE triggers PDCP data recovery for all the DRBs that are part of the current UE configuration. This would include triggering PDCP recovery for all bearers including MN terminated SCG bearers and SN terminated MCG bearers.

Observation 3: As per RAN4 response LS in R4-2420218, the UE does not need to know the number of SpCells and SCells within LTM configurations before early ASN.1 decoding.

Observation 4: RAN4 finds it beneficial if such auxiliary information is provided.

Observation 5: Release 18 has been frozen a while ago while the early ASN.1 decoding feature is fully functional even if the UE is not informed explicitly how many cells there are in each LTM configuration.

Observation 6: RAN4 in R4-2420125 informs RAN2 that the prerequisite for FG 39-6 should be removed.

Observation 7: As per TS 38.306, ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18 are the prerequisites of ltm-FastProcessingConfig-r18.

Observation 8: The use of ltm-UE-MeasuredTA in the 5.3.5.18.6 shall not be allowed when LTM is executed due to the LTM candidate selection in accordance with 5.3.7.3.

Observation 9: Rel. 18 supports simultaneous configuration, but not simultaneous triggering of RACH-based MCG and SCG LTM.

Observation 10: Based on the current specification, if LTM is configured in both MCG and SCG, then it is not possible for MN or SN to trigger e.g., early synchronization since they are not aware of what the other node has triggered.

Proposal 1: Adopt the TP in Annex A.

Proposal 2: RAN2 does not consider any further the addition of an explicit information regarding the number of SpCells and SCells within the LTM candidate configuration.

Proposal 3: RAN2 adopts the changes shown in Annex B to implement what RAN4 suggested in R4-2420125.

Proposal 4: Adopt the TP in Annex C to correct the ltm-UE-MeasuredTA issue identified in 38.331, section 5.3.5.18.6.

Proposal 5: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells for which early synchronization procedures can be performed. Adopt the TP in Annex D in TS 38.331.

# References

- R2-2410442 Miscellaneous Rel-18 LTM Aspects and Corrections 3GPP TSG-RAN WG2 Meeting #128	Orlando, USA, November 18th – 22nd, 2024

- R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #128	Orlando, USA, November 18th – 22nd, 2024

- R4-2420218 Reply LS on the fast RRC processing for LTM 3GPP TSG RAN WG2 Meeting #113 Orlando, US, 18th - 22nd  November, 2024

- R4-2420125 LS on UE capability for fast RRC processing for LTM 3GPP TSG RAN WG2 Meeting #113 Orlando, US, 18th - 22nd  November, 2024

- 3GPP TS 38.306 NR UE capabilities, V18.4.0 (2024-12)

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification, V18.4.0 (2024-12)

# Annex A – TP for PDCP data recovery

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

[…]

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

3>	for each drb-Identity value that is part of the same RLC entity that is being reestablished:

4>	if this DRB is an AM DRB:

5>	at the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

[…]

# Annex B – changes to 38.306 related to RAN4 LS (R4-2420125)

| ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133 [5]. UE shall set the capability values for maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18 consistently for all bands. These capability values represent the maximum number across all the supported bands. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

# Annex C – TP for TS 38.331 (section 5.3.5.18.6) – use of ltm-UE-MeasuredTA-ID for LTM Recovery

##### 5.3.5.18.6	LTM cell switch execution

…

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID and the LTM cell switch is not due to the selected cell in accordance with 5.3.7.3:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

…

# Annex D – TP for TS 38.331 – Early Synchronization for MCG and SCG LTM simultaneously configured

It is sufficient to update CG-ConfigInfo. For simplicity, in the sequel we show an example of adding the IEs related to early TA acquisition. Depending on the group discussion, additional capabilities can also be considered.

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

-- Text omitted

[[

maxNumberLTM-CandidatesSCG-r18   INTEGER(0..maxNrofLTM-Configs-r18)                               OPTIONAL

]],

[[

allowedL1-MeasConfigNRDC-r18     L1-MeasConfigNRDC-r18                                            OPTIONAL

]],

[[

rach-EarlyTA-Measurement-r18   INTEGER(0..maxNrofLTM-Configs-r18)                               OPTIONAL,

ue-TA-Measurement-r18   INTEGER (0..maxNrofLTM-Configs-r18)                               OPTIONAL

]],

}

-- Text omitted

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP