3GPP TSG-RAN WG2 Meeting #129		R2-2500355

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on UE-initiated/event-driven beam management

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

According to the previous RAN1 meetings, some progress on UE-initiated/event-driven beam management were made in RAN1 [1][2][3], e.g..

Agreement in RAN1#116bis Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Agreement in RAN1#118bis Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed. Agreement in RAN1#119 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

In this contribution, we will discuss the RAN2 aspects related to UE-initiated/event-driven beam management, including MAC and RRC impacts.

- Discussion

  - UE-initiated/event-driven beam report - MAC impacts

In RAN1#116bis meeting, two report modes were agreed for UE-initiated/event-driven beam report and the beam report carried as UCI was also agreed. Furthermore, in RAN1#119 meeting, the first PUCCH indication was agreed as a new UCI type for both modes, rather than SR. Hence, the beam report transmission procedure is a total physical operation. Considering measurement is also performed in the physical layer, it is more reasonable to leave the issue about the evaluation of trigger event for beam report to RAN1 to discuss and decide.

Proposal 1: It is up to RAN1 to discuss and decide how to perform the evaluation of trigger event for beam report, and how to capture the corresponding procedure.

Regarding the measurement RS determination of new beam(s), RAN1 has agreed that the measurement RS(s) for new beam(s) are explicitly configured in one RS resource set associated with a CSI reporting configuration in RAN1#118 meeting [2].  But whether the RS in the RS resource set can be updated by MAC-CE is still being discussed in RAN1.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

In our view, new beam(s) changes as the current beam changes, so it is necessary to update the measurement RS(s) of new beam(s). In addition to dynamic update by MAC CE, RRC reconfiguration is another approach. However, as RRC reconfiguration is a long-time operation, it is not suitable for UE-initiated/event-driven beam reporting, which is intended to reduce TCI switch delay. To avoid the update of measurement RS(s) of new beam(s), all of the current beam(s) applied for transmission and reception, the activated TCI states other than the indicated one, and inactive TCI states should be included in the measurement resource set. However, it may result in a large measurement overhead and increase the UE computation complexity. In summary, the dynamic update of measurement RS within the new beam resource set by MAC CE is more reasonable. But anyway, the final decision should be up to RAN1 discussion.

Proposal 2: RAN2 assume to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

  - Configuration – RRC impacts

In RAN1#116 bis meeting[1], RAN1 has agreed to introduce Event-2 as the trigger event for beam report.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

In our understanding, the trigger event should be configured by RRC for beam report. Thus, the corresponding configuration as well as the definition of the trigger event should be captured in RRC specification. If there is configuration for trigger event in RRC, the corresponding measurement report should be triggered by the configured event; otherwise, it will follow the legacy periodic report.

In addition to the trigger Event 2 defined as above, RAN1 has working assumption on Event 1 and Event 7 in RAN1#118 meeting [2] as below, which has not been final decided:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise The additionally supported events will be lower priority compared to event 2.

If these events are agreeable, it should be up to RRC to configure which trigger event(s) should be applied for beam report.

Proposal 3: It is up to RRC to configure which/whether trigger event(s) should be applied for beam report.

Proposal 4: The corresponding RRC configuration as well as the definition of trigger events should be captured in RRC specification.

- Conclusion

In this contribution, we discuss the RAN2 aspects on UE-initiated/event-driven beam management, including RRC and MAC impacts. Based on the discussion, we have the following observations and proposals.

MAC impacts:

Proposal 1: It is up to RAN1 to discuss and decide how to perform the evaluation of trigger event for beam report, and how to capture the corresponding procedure.

Proposal 2: RAN2 assume to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

RRC impacts:

Proposal 3: It is up to RRC to configure which/whether trigger event(s) should be applied for beam report.

Proposal 4: The corresponding RRC configuration as well as the definition of trigger events should be captured in RRC specification.

- References

- Draft_Minutes_report_RAN1#116bis

- Draft_Minutes_report_RAN1#118

- Draft_Minutes_report_RAN1#119