3GPP TSG-RAN WG2 Meeting #129												R2-2500378

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.4.1

Source: 	Sharp

Title:  	Issues on Additional LCH Priority and Prioritization

Document for: 	Discussion

# Introduction

In RAN2#127bis meeting, RAN2 made the following agreements on LCP prioritization:

Agreements on LCP enhancements - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

In RAN2#128 meeting, RAN2 made the following agreements:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

This document discusses leftover issues on additional LCH priority and prioritizations.

# Discussion

## Configuration of Additional LCH Priority

The additional LCH priority is used for the LCH priority-adjusted data for each logical channel. The additional priority can be another logical channel priority value specific to a logical channel. Thus, from configuration perspective, it is natural that logicalChannelConfig includes the additional LCH priority. We do not see any necessity of separate logical channel configuration with different logical channel identity.

Additional LCH priority is configured by logicalChannelConfig in an RRC message.

It may be possible that multiple additional LCH priority values are configured for different conditions. However, we see that it brings an unnecessary complexity without much gain. For simplicity, it would be desirable to have at most one additional LCH priority for a logical channel.

At most one additional LCH priority can be configured for each logical channel.

It may be possible that LCH with LCH priority-adjusted data prioritizes data from SRB when the additional priority is higher than that of SRB. In this case, SRB transmission can be delayed until other DRBs finish all pending transmissions.

However, it is not a big issue or can be resolved by proper priority configuration. The default priority of SRB1 is 1 which is the highest priority. Even if an additional priority for other logical channel is configured with a very high priority value, it is not higher priority than that of SRB1. Thus, SRB1 will not experience actual transmission delay. Other SRBs could experience some delayed transmission. But it is possible even without additional priority that a logical channel for a DRB is configured with a very high priority value. If the network thinks it has a big problem, the situation can be avoided by not configuring high additional priority.

The impact of additional priority to SRB can be handled by network implementation.

## Impact to Intra-UE Prioritization

The additional LCH priority was discussed as an LCP enhancement. However, LCH priority is used for intra-UE prioritization configured with Rel-16 lch-basedPrioritization. In Rel-16 lch-basedPrioritization, priority of an uplink grant is determined by using the LCH priority. Also, subclause 5.4.4 on SR uses priority of an SR i.e., the priority of the logical channel that triggered SR, for resource overlap among PUSCH and SR in Rel-16 lch-basedPrioritization. All those legacy intra-UE prioritization rules assume a single LCH priority. RAN2 needs to discuss how to determine those priorities when we have both legacy priority and additional priority.

Figure 1. Priority issue in LCH-based prioritization

Figure 1 shows the issue on whether the additional LCH priority is used. The example assumes LCH1 has priority value of 10 as the normal priority and priority value of 3 as the additional LCH priority. When there is LCH priority-adjusted data whose discard timer expiry time is below the threshold, LCP is performed with priority value of 3. Since there is a time delay until the actual transmission, it is not clear which priority value is used for priority of the uplink grant. From data usage point of view, additional priority which was used during LCP procedure needs to be used to prioritize the uplink grant carrying LCH priority-adjusted data. However, from LCP point of view, all LCH priority-adjusted data has been included in MAC PDUs, the priority may fall back to the normal priority.

RAN2 to discuss how to determine the priority of an uplink grant and priority of a logical channel which triggered an SR in lch-basedPrioritization (intra-UE prioritization).

## Priority to Trigger a Regular BSR

- In the current BSR procedure, a regular BSR is triggered if UL data for a logical channel which belongs to an LCG becomes available to the MAC entity and this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG. If an additional priority is used, it is not clear which priority is used for triggering a regular BSR.

- We have two options on the table.

- Option 1: Normal priority is always used.

- Option 2: Additional priority is used if an LCH has priority-adjusted data.

- Suppose both LCH1 and LCH2 are configured with delay-critical priority. For example, priority {1,2} is configured for LCH1 and priority {2,3} is configured for LCH2. Note that the former one is a normal priority whereas the latter one is a delay-critical priority. At the time that UL data for a LCH1 becomes available to the MAC entity, the UL data is normal data, so it is reasonable to use the normal priority (i.e., priority 2) for the LCH1. But LCH2 may have LCH priority-adjusted data. If normal priority (i.e., priority 3) is used for LCH2, BSR is triggered for LCH1. But if delay-critical priority (i.e., priority 2) is used, BSR for LCH1 will not be triggered. So, depending on the using of normal priority or delay-critical priority to the logical channel, the determination about whether BSR should be triggered may be different. If normal priority is used for the buffered data, BSR is triggered. But if delay-critical priority is used for the buffered data, BSR is not triggered. For this case, configuration of delay-critical priority for a LCH with low priority may delay the notification of the available of the UL data to gNB for a LCH with high priority because of the temporal adjusted priority of the LCH with low priority. But if normal priority is always used for logical channel that the buffered data belong to, the BSR for a LCH with high priority can be triggered as in legacy.

In Regular BSR triggering, normal priority is always used for logical channel that the buffered data belonging to.

## Independent Configurations of DSR and Delay-Critical Priority

- According to the agreement in RAN2#127bis, RAN2 will not introduce any setting restrictions of the new remaining time threshold for delay-critical priority with relation to DSR triggering threshold remainingTimeThreshold. But there was no discussion/conclusion on whether there is any restriction on the configuration of delay-critical priority and remainingTimeThreshold for DSR. In other words, it is not clear whether delay-critical priority and remainingTimeThreshold for DSR can be configured independently or shall be jointed configured.

RAN2 to choose one option:

- Option 1. delay-critical priority is configured only if remainingTimeThreshold for DSR is configured.

- Option 2. delay-critical priority can be configured regardless of configuration of remainingTimeThreshold for DSR.

# Conclusions

RAN2 is requested to discuss and agree the following proposals:

- Additional LCH priority is configured by logicalChannelConfig in an RRC message.

- At most one additional LCH priority can be configured for each logical channel.

- The impact of additional priority to SRB can be handled by network implementation.

- RAN2 to discuss how to determine the priority of an uplink grant and priority of a logical channel which triggered an SR in lch-basedPrioritization (intra-UE prioritization).

- In Regular BSR triggering, normal priority is always used for logical channel that the buffered data belonging to.

RAN2 to choose one option:

- Option 1. delay-critical priority is configured only if remainingTimeThreshold for DSR is configured.

- Option 2. delay-critical priority can be configured regardless of configuration of remainingTimeThreshold for DSR.