3GPP TSG-RAN WG2 Meeting #129	 R2-2500449

Athens, GR, February 17-21, 2025

Source:	KDDI Corporation

Title:	Discussion on On-demand SIB1

Tdoc Type:	Discussion

Agenda Item:	8.5.3	On-demand SIB1

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#128[1], on-demand SIB1 was discussed and below agreements were made.

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, we discuss the points that need further consideration in RAN2.

2	Discussion

2.1 RAR procedure

In the legacy SI request using CFRA procedure, as specified in TS 38.321[2], if a valid downlink assignment (as defined in TS 38.213 [6]) is received on the PDCCH for the RA-RNTI, and the received TB is successfully decoded, UE performs RAR reception according to the received PDCCH information. Upon receiving RAR, it is considered that the SI request (RA preamble) has been successfully received, and UE tries to acquire SI message. Additionally, the unsuccessful pattern of the SI request is also specified in TS 38.321.

From the RAN1 agreement below,

RAN1 assumes that the UE is expected to receive the RAR in response to the preamble transmission for the Msg1-based on-demand SIB1 procedure as the baseline.

It can be inferred that the UE receives a PDCCH indicating the scheduling information for the RAR. Upon successfully receiving the RAR, the UE can proceed the SIB acquisition.

Therefore, it is straightforward to reuse the existing procedure for the Msg1-based the on-demand SI.

Proposal1: RAN2 agree that On-demand SIB1 uses RAR procedure for On-demand SI.

The following is specifications that complements the explanation of Proposal 1.

From Section 5.2.2.3.3 of TS 38.331[3]: 1> else: 2> if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met; or 2> if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met: 3> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 3> if acknowledgement for SI request is received from lower layers: 4> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

From Section 5.1.4 of TS 38.321: 1> else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded: 2> if the Random Access Response contains a MAC subPDU with Backoff Indicator: 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. 2> else: 3> set the PREAMBLE_BACKOFF to 0 ms. 2> if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3): 3> consider this Random Access Response reception successful. 2> if the Random Access Response reception is considered successful: 3> if the Random Access Response includes a MAC subPDU with RAPID only: 4> consider this Random Access procedure successfully completed; 4> indicate the reception of an acknowledgement for SI request to upper layers. 1> if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received: 2> consider the Random Access Response reception not successful; 2> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> if the Random Access Preamble is transmitted on the SpCell: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request: 5> consider the Random Access procedure unsuccessfully completed.

2.2 SIB1 monitoring window

In RAN2 #127[4] meeting, it was discussed that the decision regarding how long UE should wait to receive SIB1 could be left to the specific implementation of UE. Consequently, it was decided to wait for RAN1 progress this issue.

P6 in R2-2408461 (Google) Proposal 5: If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred. Wait for more RAN1 progress.

However, it would be beneficial to clarify the necessity of SIB1 monitoring window from the perspective of RAN2. The purpose of SIB1 monitoring window is not only to determine whether UE has acquired SIB1 but also to be applicable in deciding whether to retransmit WUS.

Firstly, in the case of on-demand SIB1 with RAR, the system considers on-demand SIB1 based on the reception of RAR, and the decision to retransmit WUS is based on whether RAR is received or not. However, in scenarios without RAR, there is no acknowledgment from the network, so a substitute method is needed to decide on the retransmission of WUS. One solution to address this issue is to introduce SIB1 monitoring window, which would enable the decision on the necessity of WUS retransmission.

On the other hand, in the RAN2 discussions, it was agreed to base on RAR, and for the reasons mentioned above, there are doubts regarding the introduction of SIB1 monitoring window in the RAR scenario. Next, although the decision of how long to wait for SIB1 reception for legacy gNB was left to UE implementation, there is a need for UE implementations corresponding to the SIB1 monitoring window for NES Cells. This leads to concerns that UE will have to implement two patterns to perform SIB1 reception, which complicates the implementation.

For these reasons, it is considered reasonable not to introduce the SIB1 monitoring window in Rel-19.

Proposal2: RAN2 agree not to introduce SIB1 monitoring window in Rel-19.

2.3 Automatic Neighbour Relation

gNB manages the NCR(neighbor cell relation) information to facilitate handover (HO) between gNBs. There are two main methods for creating the NCR: one is manual creation by the operator, and the other is automatic creation through ANR (Automatic Neighbor Relations).

For the automatic creation of NCR through ANR, UE needs to obtain the NCGI information of the target cell and report it to the source cell. In the on-demand SIB1 scenario, there is no periodic transmission of SIB1 from NES cell, so UE cannot acquire SIB1 for NES cell.

As a result, ANR cannot function properly, leading to the issue of being unable to automatically create NCR. Manually creating all NCRs would significantly increase operational costs, so it is necessary to consider the issues in the on-demand SIB1 scenario.

Proposal3: RAN2 discuss ANR issue in the on-demand SIB1 scenario.

To achieve the coexistence of on-demand SIB1 and ANR, it is necessary to discuss specific methods. For example, One approach is that when gNB detects ANR activity, it could coordinate with other gNB or higher-level network equipment to stop the SIB1-less mode of NES cells through network interface and have NES Cell transmit SIB1 periodically at all times. Another approach could be for UE to obtain SIB1 information of the NES cell in advance before executing ANR operations. In the latter case, if the UE is to acquire the necessary SIB1 information for ANR in advance, it must first receive and store the required information internally, which would lead to a significant burden on UE's implementation. Therefore, it is considered more straightforward to first consider solution like stopping the SIB1-less mode on the network side in RAN3, rather than considering solution in RAN2.

Proposal 4: If RAN2 discuss the coexistence of ANR and on-demand SIB1, RAN2 agree that the solution should be considered within RAN3

3	Conclusion

We make the following observations and proposals:

Proposal 1: RAN2 agree that On-demand SIB1 uses RAR procedure for On-demand SI.

Proposal 2: RAN2 agree not to introduce SIB1 monitoring window in Rel-19.

Proposal 3: RAN2 discuss ANR issue in the on-demand SIB1 scenario.

Proposal 4: If RAN2 discuss the coexistence of ANR and on-demand SIB1, RAN2 agree that the solution should be considered within RAN3

3	　Reference

[1]　RAN2#128 chairnote

[2]　3GPP TS 38.321 V18.1.0

[3]　3GPP TS 38.331 V18.1.0

[4]  RAN2#127 chairnote