3GPP TSG-RAN WG2 Meeting #129	R2-2500488

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	Rapporteur Inputs

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This document contains the following rapporteur’s input to this meeting:

-	A list of XR agreements reached in RAN2 so far;

-	An updated Workplan factoring the RAN agreements in;

-	A short overview of the status from other groups, focusing on matters impacting RAN2.

# 2	List of Agreements

## 2.1	RAN2#125bis

Regarding the organisation of the work:

-	RAN2 tries to identify impacts on other WGs as soon as possible, e.g. before study phase ends.

Regarding multi-modality (study phase):

-	For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information;

-	FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc.

-	RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE;

-	RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge;

-	Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas;

-	RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows;

-	For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed;

-	Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs.

Regarding Scheduling Enhancements (study phase):

-	RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied:

-	Alternative 1: Enhance LCP restrictions/LCH selection;

-	Alternative 2: Enhance LCH prioritization;

-	RAN2 should consider potential impact on traffic from SRBs.

-	RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance - FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

Regarding RLC Enhancements:

-	We focus on RLC AM;

-	RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR;

-	RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets).

## 2.2	RAN2#126

Regarding multi-modality (study phase):

-	Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

Regarding scheduling enhancements (study phase):

-	For LCP enhancements, LCP Prioritisation:

-	Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR;

-	The solution should consider impact on UE complexity (as already indicated in SI objective description);

-	For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

-	Use additional priority configured to LCHs in case of these LCHs with delay-critical data.

-	FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH.

-	For LCP enhancements, LCH Restrictions:

-	We try to avoid RAN1 impacts;

-	RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way;

-	For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial;

-	The solutions should not disallow non-delay critical data from using an UL grant.

-	For LCP enhancements, Granularity:

-	LCP prioritization within a logical channel will not be considered in RAN2 discussions;

-	FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

-	For DSR enhancements:

-	Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG;

-	FFS whether DSR triggering is impacted;

-	FFS whether PDU set importance needs to be included.

Regarding RLC enhancements:

-	For avoiding unnecessary retransmissions:

-	For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

1.	Rx initiated approach

2.	Tx initiated approach

-	RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

-	For Tx initiated approach:

-	The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs;

-	Tx side stops retransmit obsolete SDUs;

-	Rx side updates state variables according to the information from Tx side.

-	For Rx initiated approach:

-	For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter;

-	Tx side just processes the status report as in legacy;

-	FFS how Rx side determines that an SDU should be abandoned.

-	For autonomous retransmissions:

-	To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

-	Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered);

-	Retransmission based on enhanced status report;

-	Retransmission based on enhanced polling;

-	FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism.

-	Impact on capacity should be considered;

-	RAN2 focuses on the enhancements for UL traffic.

In addition, a few LSs were sent to SA2:

-	R2-2405779 Reply LS to SA2 on FS_XRM PH2

-	R2-2405781 Reply LS on Application-Layer FEC Awareness at RAN

-	R2-2405782 LS on multi-modality awareness at RAN

## 2.3	RAN2#127

Regarding an incoming LS from SA2 [R2-2406433], RAN2 agreed:

-	From RAN2 periodicity can already be provided to gNB via TSCAI and/or UAI, which is sufficient for infrequent periodicity changes;

-	RAN2 does not have consensus on whether additional indication for dynamic changes of the periodicity are needed;

-	RAN2 thinks TTNB may be useful for the NW scheduling for DL, provided it is provided in advance and is reliable and accurate at RAN.

Then, RAN2 agreed the following regarding multi-modality (study phase):

-	Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

-	FFS whether this can be applied to DL

-	RAN2 considers that based on multi-modal information:

-	The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

-	The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

-	For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns.

-	For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

-	For PDU set discard enhancements:

-	RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

-	RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

-	For DRX enhancements:

-	Not support multiple active DRX configurations

Regarding scheduling enhancements (also study phase), RAN2 agreed:

-	RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR

-	Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

-	For enhanced DSR:

-	There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

-	FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

-	FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

-	FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

Regarding RLC enhancements:

-	For Unnecessary retransmissions:

-	Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

-	In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.

-	In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where

-	Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach

-	Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

## 2.4	RAN2#127bis

Agreements for RRM measurement gap skipping:

1.	RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

2.	No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features

3.	RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear.

4.	RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4.

5.	RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution.

Agreements on LCP enhancements:

1.	As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

2.	FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation.

3.	Introduce an independent per-LCH remaining time threshold for applying delay-critical priority.

4.	We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

Agreements on DSR enhancements:

1.	We do not change the definition of delay-critical data

2.	For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s)

3.	Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc.

4.	For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0:

•	Buffer size of data volume in each portion

•	Shortest remaining time among PDCP SDUs buffered in each portion.

5.	There is no need to include PSI in the enhanced DSR MAC CE.

6.	A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

7.	FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Agreements on RLC timely retransmissions:

1.	RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

2.	Exclude enhanced status reporting.

3.	Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

Agreements on avoiding unnecessary retransmissions:

1.	RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does.

2.	RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e.

•	TX side stops transmissions of an outdated SDU

•	RX side abandons the SDU based on a local timer

•	Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

•	FFS if some C-PDU handling is needed to avoid C-PDU discard

•	FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

Agreements on XR rate control

1.	FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

2.	RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications:

-	Extend the Bit Rate field

-	Define a new bit rate table to provide sufficient granularity for XR traffic

-	Introduce new values for the bitRateMultiplier

3.	Send LS to SA4 asking about range/granularity which is required

## 2.4	RAN2#128

Agreement for the reply to SA2 on PDU set information: RAN2 confirms that it can be useful for gNB to have PDU Set Information marking without PDU Set QoS parameters.

Agreements on RRM measurement gaps impacts:

1.	No MG-specific enhancements is needed on DSR operation.

2.	RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped

3.	No MG-specific enhancements is needed for DRX operation.

Agreements on LCP prioritization:

1.	As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round.

2.	As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

Agreements on DSR enhancements:

-	Let the network configure the triggering and reporting thresholds without constraints.

-	RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR

-	The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

-	The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

Agreements on AL-FEC (related to LS from SA2):

-	There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM.

-	RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications

Agreements on unnecessary RLC retransmissions:

-	There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer

-	Unless critical issue is identified, no Tx to Rx indication will be introduced

-	Special handling to avoid PDCP control PDU discard is not needed.

-	A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.

-	The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Agreements on timely RLC retransmissions:

-	Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

Agreements on XR rate control

1.	RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level.

2.	gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

3.	Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

4.	RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP.

5.	RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE

6.	FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

# 3	Workplan

The WID was updated at RAN#109 and now contains the following objectives [RP-243318]:

-	Specify support for multimodality in RAN for UL and DL [RAN3]:

NOTE:	This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance  information-based approach as an alternative. RAN#107 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE.

NOTE:	Review in RAN#107 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q2/25 onwards.

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

-	Specify the following user plane enhancements [RAN2]:

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

-	Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]:

-	PDU set based handling;

-	ECN marking;

-	Burst Arrival Time reporting, if needed;

-	PSI Discard coordination, if needed;

NOTE:	No RAN2 impact from above items.

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

-	Specify Core requirements related to the above objectives as necessary [RAN4].

The corresponding update to the workplan is given below.

Table 3-1: Workplan for the XR WI

| Date | RAN1 | RAN2 | RAN3 | RAN4 | RAN |
| --- | --- | --- | --- | --- | --- |
| 2024-02 | RAN1#116 Start discussion on a design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. |  |  |  |  |
| 2024-03 |  |  |  |  | RAN#103 Status Report (target of 2% corresponding to 0.5 TU spent over an initial allocation of 24.8) |
| 2024-04 | RAN1#116bis Continue the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Start agreeing on solution(s). | RAN2#125bis Endorse workplan Start the work on all 4 objectives except measurement gaps, for which RAN1 conclusions first need to be reached. For multi-modality and scheduling enhancements, start the study phase. | RAN3#123bis Start discussion on RAN3 related objectives on NR-DC, e.g. PDU set based handling, ECN marking, etc. Agree the DC bearer types to be supported. Designate responsible companies for BL Stage-2 & 3 CRs. |  |  |
| 2024-05 | RAN1#117 Continue developing the solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#126 Continue the work on all 3 objectives. | RAN3#124 Finalize the solution on required enhancements. Finalize Stage-2 CR(s) Agree the skeleton of Stage-3 CR(s). |  |  |
| 2024-06 |  |  |  |  | RAN#104 Status Report (target of 18% corresponding to 4.5 TUs spent over an initial allocation of 24.8) Take into account outcome of SA2 study if available. |
| 2024-08 | RAN1#118 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#127 Continue the work on all 3 objectives. For multi-modality and scheduling enhancements, conclude the study phase and convey the outcome to RAN#105. | RAN3#125 Agree Stage-2 CR(s) and Stage-3 CR(s) | RAN4#112 Approval of workplan Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |  |
| 2024-09 |  |  |  |  | RAN#105 Status Report (target of 30% corresponding to 7.5 TUs spent over an initial allocation of 24.8) Checkpoints: 1. Multi-modality (RAN2) 2. Scheduling enhancements (RAN2) |
| 2024-10 | RAN1#118bis Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Check if RAN1 involvement related to objective on multi-modality is needed. | RAN2#127bis Check with RAN1 the progress on measurement gaps and start discussing in RAN2. Continue the work on UL scheduling enhancements according to the refined objective. Start the work on uplink congestion signalling. | RAN3#125bis Discuss multi-modality when handling incoming LS (no additional TU allocated to the new objective). | RAN4#112bis Conclusions on scenarios for measurement skipping based on RAN1 conclusions. Discussion on the need for UE assistance information regarding measurements occasions needed. |  |
| 2024-11 | RAN1#119 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#128 Narrow down possible solutions. First version of Stage 2 CR | RAN3#126 Discuss LS on PDU Set Information Marking Support without QoS parameters, and usage of multi-modality information (no additional TU allocated to the new objective). | RAN4#113 Discussion on RRM requirements with measurement skipping. Discussion on the necessary UE assistance information. |  |
| 2024-12 |  |  |  |  | RAN#106 Status Report (target of 55% corresponding to 13.5 TUs spent over an initial allocation of 24.8) Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) |
| 2025-02 | RAN1#120 Finalize, if not already, the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Provide RAN1 input to RAN2 on RRC configuration parameters, UE capabilities, and Stage 2 TPs. | RAN2#129 Continue the work on the details of the agreed solutions. First versions of Stage 2 & 3 CR. | RAN3#127 Discuss LS on UL bit rate control, available data rate reporting and multi-modality awareness (no additional TU allocated to the new objective). | RAN4#114 Conclusion on RRM requirements with measurement skipping. Discussion on the need for UE assistance information regarding measurement skipping. Agreement of CR work split. |  |
| 2025-03 |  |  |  |  | RAN#107 Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) Status Report (target of 67% corresponding to 16.5 TUs spent over an initial allocation of 24.8) |
| 2025-04 | RAN1#120bis | RAN2#129bis Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. |  | RAN4#114bis Conclusion on the necessary UE assistance information. First Draft CRs for RRM core requirements (draft CRs for 38.133 expected) First big Draft CR. |  |
| 2025-05 | RAN1#121 | RAN2#130 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. |  | RAN4#115 Discussion on remaining open issues for RRM core. |  |
| 2025-06 |  |  |  |  | RAN#108 Status Report (target of 91% corresponding to 22.5 TUs spent over an initial allocation of 24.8) |
| 2025-08 |  | RAN2#131 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs |  | RAN4#116 Discussion on remaining open issues for RRM core. Revision of CRs for RRM core requirements. Planning of RRM performance. Initial discussion on RRM test cases. |  |
| 2025-09 |  |  |  |  | RAN#109 Status Report (target of 100% corresponding to 24.8 TUs spent over an initial allocation of 24.8) CRs received for approval. |

# 4	Status from other groups

## 4.1	RAN1

During RAN1#119, the following was agreed:

-	For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”.

-	FFS: Interpretation of bit value equal to “0”.

- A component of an FG can have Option 1 as a first value and Option 2 as a second value:

-	Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

-	Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

-	Note: UE indicates only one value of this component.

- For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

-	Explicit indication can be configured for each DCI format individually by higher layer.

-	The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

-	Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

## 4.2	RAN3

RAN3 discussed the SA2 LS on PDU Set Information Marking Support without QoS parameters. RAN3 agreed It is feasible to re-use the existing PDU Set Based Handling Support Indication to indicate SMF that PDU Set based handling is supported by NG-RAN when receiving the DL PDU Set Marking indication without PDU Set QoS Parameters. Some companies believe it is beneficial to support dynamic PSI activation/deactivation from RAN to 5GC, but other companies have different view. There is no agreement on supporting dynamic PSI activation/deactivation from RAN to 5GC. RAN3 expects SA2 to further discuss this topic based on RAN3 feedback. Reply LS to SA2 agreed in R3-247875.

RAN3 discussed the RAN2 LS on usage of multi-modality information. RAN3 would like to confirm the usage of MMSID as concluded by RAN2, while traffic synchronization can be further checked in RAN3 based on progress in RAN2, SA2 and SA4. Reply LS to RAN2 agreed in R3-247874.

## 4.2	RAN4

LS Response to RAN1 in R4-2420198.

Other agreements captured in R4-2420103.

Deployment scenarios:

-	EN-DC and NE-DC for DCI-based solution:

-	Is NOT supported for measurement gap skipping by the WID.

The following measurement types are considered for XR measurement skipping:

-	Intra-frequency;

-	Inter-frequency;

-	LTE-inter- RAT (NR-EUTRAN FDD/TDD);

-	The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

General approach for defining requirements for measurements with gaps	:

-	For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

How to capture impact of measurement skipping:

-	Reuse existing measurement accuracy requirements;

-	RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping:

-	For the delay requirements, FFS whether and how to extend the delay.

Time offset requirement for measurement gap skipping:

-	Baseline feature 5 ms;

-	Optional capability 3 ms, FFS if any condition is needed.

## 4.3	SA2

For reference, the table below describes the impacts the SA2 key issues are expected to have on RAN.

Table 4.3-1: SA2 Key Issues

| KI | Description | RAN Impacts |
| --- | --- | --- |
| 1 | Support of PDU set based QoS handling enhancement: study PDU set based QoS handling enhancements considering both control plane and user plane perspectives. In particular, this KI addresses whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced | PDU Set handling RAN3 Alternative QoS profiles RAN3 QoS notification RAN3 FEC Awareness RAN2(?), RAN3 |
| 2 | Support PDU Set information identification for end-to-end encrypted XRM traffic: study the enhancement of PDU Set information Identification for encrypted XRM in 5G networks | No |
| 3 | Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network: study whether, how, and what PDU Set QoS information can be used for DSCP marking on the outer header of downlink packets of the PDU Set over N3/N9 in the transport network | No |
| 4 | Traffic detection and QoS flow mapping for multiplexed data flows: study whether, how, and what PDU Set QoS information can be used for DSCP marking on the outer header of downlink packets of the PDU Set over N3/N9 in the transport network | No |
| 5 | QoS Handling when Traffic Characteristics Change Dynamically: study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN | Burst Size RAN2, RAN3 TTNB RAN2, RAN3 |
| 6 | L4S for non-3GPP access networks and intermediate 5GS nodes: extend the L4S mechanism to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes | No |
| 7 | Support for PDU Set in non-3GPP access: support PDU set based QoS Handling to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes | No |
| 8 | Enhancement for UE with the tethered devices: study whether and how to identify traffic flows from the tethered devices behind the UE from the uplink traffic (e.g. traffic from different tethered devices may be mapped to different QoS Flows to enable QoS differentiation) | No |
| 9 | Enhancement for XR related network information exposure: to study how to enhance network exposure mechanism to better support the network information/capability exposed to the application layer | Data Rate Exposure RAN3 |

The SA2 status report for the work on Extended Reality and Media Service (XRM) Phase 2​ was provided in SP-241385, with a completion rate of 65%. Focusing on matters of interest to RAN2, for KI#5, the following was agreed:

-	 Time to Next Burst [23.501]:

-	The time to next data burst may be provided to the NG-RAN by the UPF to assist NG-RAN's behaviour in downlink. As described in clause 6.1.3.27.5 of TS 23.503 [45] the PCF may include a Time to Next Burst Marking Indication within a PCC Rule to request the UPF to identify and mark the time to next data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF.

-	Based on the Time to Next Burst indication in a PCC Rule, the SMF instructs the UPF to identify and mark the time to next data burst in downlink. The SMF may provide the UPF the Protocol Description used by the service data flow received in the PCC Rule.

-	According to the Time to Next Burst indication and DL Protocol Description from the SMF, the UPF identifies the time to next data burst carried in an N6 RTP Header Extension/transport protocol header as defined in TS 26.522 [179] and sends the identified time to next data burst to NG-RAN in a downlink GTP-U header.

-	Time to next data burst marking is only used when the deployment is such that the time to next data burst is sufficiently accurate i.e. the jitter (e.g. the N6 jitter) is sufficiently limited without impact to the accuracy of the time to next data burst.

-	The time to next data burst was then clarified as the interval between the transmission of the last PDU in the current data burst and the first PDU of the next data burst [S2-2501110]

-	Burst Size [23.501]:

-	The Data Burst Size may be provided to the NG-RAN in the downlink GTP-U header by the UPF in order to assist radio resource management. As described in clause 6.1.3.27.5 of TS 23.503 [45] the PCF may include a Data Burst Size Marking Indication within a PCC Rule to request the UPF to identify and mark the Data Burst Size of the data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF.

-	Based on the Data Burst Size Marking Indication in a PCC Rule, the SMF instructs the UPF to identify and mark the Data Burst Size of the data burst in downlink for the corresponding PDR. The SMF may provide the UPF the Protocol Description received in the PCC Rule.

-	According to the Data Burst Size Marking Indication and DL Protocol Description from the SMF, the UPF identifies in the first DL packets of the data burst the Data Burst Size carried in an N6 RTP Header Extension/ transport protocol header as defined in TS 26.522 [179]. The UPF sends the identified Data Burst size to NG-RAN in the downlink GTP-U header of the first PDUs of the data burst.

NOTE:	Still no reply to the RAN2 LS on multi-modality awareness, neither from SA2#166 [S2-2411300], nor from the January Adhoc [S2-2500014].

# 5	Conclusion

This contribution has provided:

-	The list of XR agreements reached in RAN2 so far;

-	An update to the workplan;

-	A brief overview of the status in RAN1, RAN3, RAN4 and SA2.