3GPP TSG-RAN WG2 #129	R2-2500502

Athens, Greece, 17 – 21 February 2025

Agenda item:		8.5.2	On-demand SSB SCell operation

Title:		Further discussion on On-demand SSB for SCell

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#127 and #127bis, RAN2 had some agreements on the On-demand SSB operator for SCell [1][2].

Agreements on OD-SSB @ #127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell @ #127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we discuss some details of network signalling and RRM measurements.

# 2. Discussion

2.1	MAC signalling for OD-SSB (de)activation

For the OD-SSB activation/deactivation, RAN2 agreed to introduce the new MAC CE which can indicate for multiple SCells simultaneously. The new MAC CE can be used for both the scenario 2 (OD-SSB activation before SCell activation) and 2A (OD-SSB activation at the same time as SCell activation).

We assume that from the network perspective, these two scenarios can co-exist. For example, the network configures two SCells with deactivated state initially for a UE. At some point in time, the network sends the new MAC CE indicating OD-SSB activation on the SCell#1 and #2 with the legacy SCell activation/deactivation MAC CE indicating activation of SCell#1. Later, the network sends the legacy SCell activation/deactivation MAC CE indicating activation of SCell#2. For the SCell#1, it corresponds to the scenario 2A, while for the SCell#2 it corresponds to the scenario 2. We assume this handling is allowed and it is up to the network implementation.

Proposal 1: RAN2 to confirm that it is up to network implementation how the new MAC CE and the legacy SCell activation/deactivation MAC CE are used in combination.

Fig.1 Example combination of new MAC CE and legacy MAC CE

2.2	RRC configuration and new MAC CE contents

We discuss details of new MAC CE activating the OD-SSB. In RAN1#119, there were many agreements [6] and some of them are closely related to RAN2 discussions, for example:

Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Observation 1: RAN1 agreed to configure time domain location of OD-SSB per OD-SSB periodicity by RRC for both Case #1 and Case #2.

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Observation 2: RAN1 agreed to support configuration or indication of the number N of OD-SSB bursts to be transmitted after OD-SSB is indicated, while detail (e.g., RRC or MAC) seems still FFS.

Considering the agreements in RAN2 and RAN2 so far, we assume that the RRC can configure multiple OD-SSB patterns, i.e. combinations of OD-SSB periodicity and time domain location of the corresponding OD-SSB. To make the function more useful, the new MAC CE can indicate one of the preconfigured OD-SSB patterns. At this moment, we consider that the new MAC CE includes the information to indicate the target SCell(s) and the indication of one preconfigured OD-SSB pattern per SCell. For the deactivation based on the number N, RAN2 should wait for RAN1 progress.

Proposal 2: RAN2 to agree that the RRC can configure multiple OD-SSB patterns (i.e. combination of OD-SSB periodicity and time domain location of the corresponding OD-SSB) and the new MAC CE (de)activating OD-SSB can indicate one of the preconfigured OD-SSB patterns.

Proposal 3: The new MAC CE (de)activating OD-SSB includes at least:

- Information to indicate the target SCell(s)

- Information to indicate the preconfigured OD-SSB pattern per SCell.

2.3	RRM measurements

In RAN2#127bis, RRM measurements for the case #2 (Always-on SSB is periodically transmitted on the cell) was discussed with two options on the table [3] but postponed.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

On this issue, there were some questions, for example whether the OD-SSB and always-on SSB are on the same frequency or different frequency. Depending on the exact scenarios or network configurations, the option to be selected can be different, or there may be benefits in both options. For example, in the Option 1 the UE power consumption can be saved compared with the Option 2, if both SSBs are transmitted on the same frequency. If both SSBs are transmitted on the different frequencies, the Option 2 may be necessary depending on the reason why both are on the different frequencies. So, firstly clarification on expected scenarios would be necessary.

For RRM measurements on OD-SSB, RAN4 has also discussed the possible scenarios for the Case #2 and made some agreements including [4]:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

Then, RAN4 had sent the LS to RAN1 (Cc to RAN2) in [5], which is related to the RAN2 discussions above. RAN1 discussed the RAN4 LS and replied to RAN4 (Cc to RAN2) in [7].

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

However, RAN1 is still discussing some details for which a clarification is needed for further RAN2 discussion above. From RAN2 perspective, it is better to wait for RAN1 progress for one more meeting.

Proposal 4: For the issue of RRM measurements on OD-SSB, RAN2 to wait for further RAN1 progress on RAN4 LS (R4-2416913) related to expected scenarios.

# 3. Conclusion

In this contribution we discussed some details of network signalling and RRM measurements and made the following proposals.

Proposal 1: RAN2 to confirm that it is up to network implementation how the new MAC CE and the legacy SCell activation/deactivation MAC CE are used in combination.

Proposal 2: RAN2 to agree that the RRC can configure multiple OD-SSB patterns (i.e. combination of OD-SSB periodicity and time domain location of the corresponding OD-SSB) and the new MAC CE (de)activating OD-SSB can indicate one of the preconfigured OD-SSB patterns.

Proposal 3: The new MAC CE (de)activating OD-SSB includes at least:

- Information to indicate the target SCell(s)

- Information to indicate the preconfigured OD-SSB pattern per SCell.

Proposal 4: For the issue of RRM measurements on OD-SSB, RAN2 to wait for further RAN1 progress on RAN4 LS (R4-2416913) related to expected scenarios.

# References

[1] R2-2407571, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R2-2409211, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Vice Chairman (Samsung)

[3] R2-2408295, On-demand SSB SCell Operation, Samsung Electronics Co., Ltd

[4] R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

[5] R2-2409521=R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN4 (To RAN1, Cc RAN2)

[6] Draft Report of 3GPP TSG RAN WG1 #119 v0.3.0

[7] R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, RAN1 (To RAN4, Cc RAN2)

# Annex for reference [4]

### Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

Agreement:

- In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement.

  - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

- Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS.

- RAN4 will inform RAN1 if any further agreement.

Issue 1-3-2: Deactivated SCell measurement before OD-SSB transmission activation

Agreement:

- The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) before UE receiving OD-SSB transmission indication.

Issue 1-3-3: Deactivated SCell measurement after OD-SSB transmission deactivation

Agreement:

- The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) after the OD-SSB transmission deactivation.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

  - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)