3GPP TSG-RAN WG2 Meeting #129	R2-2500629

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.7.5

Source:		Lenovo

Title:		AM RLC enhancement

Document for:		Discussion and Decision

- Introduction

In this contribution we discuss RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

- Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2#127bis, the following agreements have been achieved:

RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In RAN2#128, the following agreements have been achieved:

RLC timely retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. Avoiding unnecessary retransmissions - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

2.1 Avoiding Unnecessary Retransmissions

RAN2 has agreed that a new timer is introduced at the RLC receiver (e.g., t-Receive) to abandon obsolete RLC SDUs. The timer starts when a gap is detected at the RLC receiver. That is, the receiver RLC can start t-Receive if it is not already running when the RLC Rx entity receives a RLC SDU from its peer entity and RX_Next_Highest >= RX_Next + 1. Additionally, the RLC Rx can maintain a new state variable that holds the value of the SN following the SN of the RLC PDU that triggered t-Receive. The new timer, t-Receive is stopped when the RLC SDU that started the timer is correctly received by the RLC Rx entity. That is, when a RLC SDU is received and RX_Next is updated to a value equal to or greater than that of the new state variable, the t-Receive timer is stopped if it was already running.

Upon expiry of t-Receive, the receiver RLC entity updates RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which the RLC SDU has not been abandoned, and for which not all bytes have been received. Additionally, RAN2 has agreed that the abandoned SDUs determined by t-Receive is positively acknowledged in the status report. Upon submission of a status report to its peer entity, the RLC receiver starts the t-StatusProhibit as per legacy behaviour. Upon reception of the status report, the RLC Tx entity updates its transmission window, state variables etc. as per legacy procedure.

Furthermore, t-Reassembly is started when the RLC Rx entity receives a RLC SDU either partly or out of sequence. If the new timer, t-Receive was started and runs to expiry while t-Reassembly is still running, it may be possible that the RLC reception window is updated beyond the RLC SDU that started the t-Reassembly timer, i.e., the RLC SDU that started the t-Reassembly falls outside the new reception window. In such a case, it makes sense that the t-Reassembly is stopped when the SDU that started the timer was abandoned by the RLC receiver.

Proposal 1. A receiver RLC can start a new timer if it is not running when the RLC entity receives a RLC SDU corresponding to a PDCP SDU and RX_Next_Highest >= RX_Next + 1.

Proposal 2. RAN2 to discuss whether to define a new state variable that holds the value of the SN following the SN of the RLC PDU that triggered the new timer (t-Receive).

Proposal 3. RAN2 to further discuss when a RLC SDU is received and RX_Next is updated to a value equal to or greater than that of the new state variable, the new timer (t-Receive) is stopped if it was already running.

Proposal 4. Upon expiry of the new timer (t-Receive), the receiver RLC entity updates the RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which the RLC SDU has not been abandoned, and for which not all bytes have been received.

Proposal 5. If the reception window is updated due to the expiry of t-Receive and t-Reassembly was still running for a RLC SDU that now falls outside the updated reception window, the t-Reassembly timer is stopped.

2.2 Timely Retransmissions

RLC-AM feedback based on current polling parameters are not well adapted for short packet delay budgets applicable to XR traffic. The data volume of PDU set of XR traffic is variable, i.e., the number of PDUs or the size of the PDUs of a PDU set is variable. One possibility, the polling parameter and t-PollRetransmit is set to small value for triggering more frequent polling to the RLC receiver, and gNB frequently transmits status report, which can cause higher overhead.

In legacy, a polling can be triggered if both the transmission buffer and the retransmission buffer become empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU, or if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling). However, if there are multiple PDU sets in transmission or retransmission buffer, the corresponding polling may not be triggered in a timely manner.

Hence, the RLC retransmission based on current RLC-AM feedback mechanism may not be timely triggered, which causes the packets to arrive too late at the receiver, thus impacting the user experience.

Observation 1. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 2. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered in a timely manner.

To avoid the late RLC retransmission, RLC transmitter may trigger early RLC retransmission for data based on the delay status, provided the transmitting entity did not receive a status report, assuming polling has been transmitted to the peer RLC entity. For example, the PDCP entity delivers a delay-critical indication to the corresponding AM RLC entity, and the delay critical data may be determined based on a separately configured threshold, different than R18 value. Additionally, another condition that can be considered to enable autonomous retransmissions is the PSI value. For example, only those packets with PSI value greater than a preconfigured threshold are eligible for autonomous retransmissions. Lastly, RAN2 can also look into scheduling enhancements (e.g., LCP enhancements) for packets that are autonomously retransmitted to avoid any delays in transmission of such data before the discardTimer expires.

Proposal 6. To achieve autonomous retransmissions on RLC layer, RAN2 to agree the following:

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

RAN2 also agreed that the enhancement of polling mechanism will be considered for the early RLC retransmission. For example, if there is more than one PDU set in transmission buffer after transmission of a PDU set, the transmitter RLC entity can trigger a new polling based on the end of PDU set, and if the remaining discard time of the data in the PDU set is below a threshold. In which case, the AM RLC transmitter entity may poll its peer AM RLC receiver entity in order to trigger STATUS reporting. And if gNB receives the polling and a t-StatusProhibit is running, it may ignore the running timer and send a status report to the peer AM RLC receiver entity at least when the status report is triggered by this new polling condition, and if there is a NACK status report to transmit.

Proposal 7. RAN2 to agree RLC transmitter triggers enhanced polling based on PDU set for data with remaining delay below a threhold.

Furthermore, if both autonomous retransmission and enhanced polling are configured by the network, it makes sense to have separate triggering conditions for autonomous retransmission and enhanced polling respectively. For example, the delay criticality threshold for enhanced polling may be slightly earlier than that for autonomous retransmission. The intention here is that autonomous retransmission is only triggered when the discard timer of a packet is close to expiry and the remaining time is not sufficient to wait for a status report (either triggered by legacy mechanism or by enhanced polling).

Proposal 8. When network configures both autonomous retransmission and enhanced polling for a single RLC entity, the two features are separately configured with different triggering thresholds.

Additionally, RAN2 should also consider mechanisms to cancel either enhanced polling or autonomous retransmission or both for a packet to avoid overloading of the link. For example, if a RLC entity is configured with both features and a legacy status report is received for a packet with a negative acknowledgement before the enhanced poll or autonomous retransmission were triggered, the enhanced poll and autonomous retransmission of that packet can be cancelled since a legacy retransmission would already be triggered in response to the NACK. Or, the cancellation of an enhanced poll or autonomous retransmission can be considered when such was already triggered and an ACK (e.g., HARQ ACK or a RLC status-report containing an ACK for the corresponding PDU) is received before the enhanced poll or autonomous retransmission is submitted to lower layers.

Proposal 9. RAN2 to consider having cancellation conditions for enhanced polling and autonomous retransmission based on the reception of status report(s).

Autonomous retransmissions can congest the link if configured poorly, due to excessive transmissions of redundant RLC PDUs. Hence, autonomous retransmission can be network controlled to provide additional control when this function is used, e.g., via RRC configuration for each RLC entity. We think both enhancements can be used together without it conflicting the other, each may use a different threshold. The new polling trigger may avoid/reduce the number of early RLC retransmission.

Proposal 10. The enhanced option(s) to achieve timely retransmissions on RLC layer is configured per RLC entiy by RRC.

- Conclusion

In this contribution, we discuss the RLC AM enhancement for XR traffic with small delay budget. We have the following observation and proposals:

Observation 1. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 2. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered in a timely manner.

Proposal 1. A receiver RLC can start a new timer if it is not running when the RLC entity receives a RLC SDU corresponding to a PDCP SDU and RX_Next_Highest >= RX_Next + 1.

Proposal 2. RAN2 to discuss whether to define a new state variable that holds the value of the SN following the SN of the RLC PDU that triggered the new timer (t-Receive).

Proposal 3. RAN2 to further discuss when a RLC SDU is received and RX_Next is updated to a value equal to or greater than that of the new state variable, the new timer (t-Receive) is stopped if it was already running.

Proposal 4. Upon expiry of the new timer (t-Receive), the receiver RLC entity updates the RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which the RLC SDU has not been abandoned, and for which not all bytes have been received.

Proposal 5. If the reception window is updated due to the expiry of t-Receive and t-Reassembly was still running for a RLC SDU that now falls outside the updated reception window, the t-Reassembly timer is stopped.

Proposal 6. To achieve autonomous retransmissions on RLC layer, RAN2 to agree the following:

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Proposal 7. RAN2 to agree RLC transmitter triggers enhanced polling based on PDU set for data with remaining delay below a threhold.

Proposal 8. When network configures both autonomous retransmission and enhanced polling for a single RLC entity, the two features are separately configured with different triggering thresholds.

Proposal 9. RAN2 to consider having cancellation conditions for enhanced polling and autonomous retransmission based on the reception of status report(s).

Proposal 10. The enhanced option(s) to achieve timely retransmissions on RLC layer is configured per RLC entiy by RRC.

- References

TS 38.322 v 18.0.0

R2-2410287, AM RLC enhancement, Lenovo