3GPP RAN WG2 Meeting #129	R2-2500659

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.3

Source:	HONOR

Title:	Discussion on UL capacity enhancement

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID, the following objectives were achieved:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#128 meeting, the following agreements were made:

1. Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) 2. Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels 3. The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) 4. There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) 5. The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific 6. There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) 7. For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time 8. For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) 9. The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) 10. Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) 11. Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. 12. RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this contribution, we discuss the remaining issues on CB-Msg3 mechanism.

2. Discussion

In the previous meeting, it was agreed that there will be a RSRP threshold that determines whether CB-Msg3 can be used. If the existing RSRP thresholds to determine CE level are lower than the RSRP threshold that determines whether CB-Msg3 can be used, there will be only one CE level can be used, which will not fully utilize the coverage enhancement feature. Therefore, we suggest introducing a new set of RSRP threshold to determine CE level for CB-MSG3 mechanism.

Proposal 1: Introduce a new set of RSRP threshold to determine CE level.

The CB-MSG3 will be triggered if the configured condition of TBS is met. Based on the mechanism of CE, when the UE determines one CE level to trigger CB-MSG3, if the number of CB-MSG3 transmissions reaches maximum number of transmissions of this CE level, it will transfer to the next level to continue the transmission. However, if TBS is configured per CE level, the condition of TBS for the next level may not be met. If so, the mechanism of CB-MSG3 will no longer be able to continue executing, and the UE will not be able to obtain gain from it. In view of this, a common maximum TBS is appropriate.

Proposal 2：A common maximum TBS should be configured for different CE levels.

When DSA is applied for CB-MSG3 transmission, one issue to be discussed is whether the UE needs to monitor one or multiple PDCCH monitoring windows. Two options are shown as follows:

Option 1: multiple/parallel PDCCH monitoring windows

For multiple PDCCH monitoring windows, one PDCCH monitoring window will be started/restarted after sending every replica. In detail, one PDCCH monitoring window will be restarted after sending the next replica, and the previous window can be stopped. Alternatively, parallel PDCCH monitoring window can be started after every replica for the reception of the corresponding response while the previous window is still running. That is, the UE might manage up to 4 windows at the same time, which will be the UE complexity. Therefore, we should carefully consider parallel PDCCH monitoring window.

Option 2: one PDCCH monitoring window

We have noticed an issue that, if the one PDCCH monitoring window is started after the last replica, the UE might not receive any response in the duration of the window, since the NW has already provided the response after receiving the previous replica and mistakenly thinking this is the last replica. To solve this issue, there are two solutions as follows:

One solution is to start PDCCH monitoring window after the end of DSA resource window. However, if the UE selects the earlier occasion DSA resource within this window time range, the monitoring window has to wait for the end of DSA time window, which might have a slight impact on UL capacity.

Another solution is to start PDCCH monitoring window, after completing the sending of the first replica. Compared with multiple PDCCH monitoring windows, if the interval between the selected two replica sending occasions is greater than the length of the PDCCH monitoring window, unnecessary PDCCH monitoring would be introduced.

Based on the analysis on the two options, we have come to the following proposal:

Proposal 3: It is preferred to have multiple PDCCH monitoring windows.

In the previous meeting, one FFS has been left as that one common RNTI or multiple RNTIs shall be used for monitoring respond message. If multiple RNTIs are used, after the NW sends the response towards UE in a short time duration (i.e., UE-eNB RTT), it may receive a new replica from the same UE. Since the NW doesn't know if this replica comes from the same UE as the previous one, the NW has to respond to this replica, which may result in unnecessary signaling overhead. Moreover, due to that the respond message is used for contention resolution, if the winner of contention resolution is same as that of the previous replica, it will have a negative impact on UL capability enhancements. Therefore, to solve this issue, one RNTI should be used.

Proposal 4: It is suggested to use one common RNTI.

In two-step RACH, each MSGA payload resource is associated with the resource of transmitting preamble. Similar to two-step RACH, the time window for DSA CB-Msg3 occasion selection can be configured corresponding to each CB-MSG3 resource. Based on the relationship between DSA resource and CB-MSG3 resource, the RNTI for DSA replicas can be calculated based on the associated CB-MSG3 resource.

Proposal 5: The DSA CB-Msg3 time window can be mapped to the CB-MSG3 resource.

3. Conclusions

In this contribution, we discussed the issues related to CB-Msg3 mechanism. Based on the discussion, the following proposals are concluded:

Proposal 1: Introduce a new set of RSRP threshold to determine CE level.

Proposal 2：A common maximum TBS should be configured for different CE levels.

Proposal 3: It is preferred to have multiple PDCCH monitoring windows.

Proposal 4: It is suggested to use one common RNTI.

Proposal 5: The DSA CB-Msg3 time window can be mapped to the CB-MSG3 resource.

4. References

[1]	RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

[3]	RP-221547, Summary for NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)

[4]	TS36.300, Overall description