3GPP TSG-RAN WG2 Meeting # 129	R2-2500728

# Athens, Greece, 17th – 21st February 2025

Agenda item:	7.2

Source:	Google

Title:	Discussion on LTM cell switch with CFRA

Document for:	Discussion & Decision

- Discussion

When the UE  receives a LTM Cell Switch Command, the UE performs section 5.18.35 in TS 38.321 as shown below. That is, the MAC entity indicates to the upper layers (i.e., RRC layer) that the LTM cell switch procedure is triggered and wait for the upper layers to reset the MAC entity.

### 5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with the SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with the SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

After receiving the indication of the LTM cell switch procedure triggered, the RRC of the UE performs section 5.3.5.18.6 in TS 38.331 as shown below.

##### 5.3.5.18.6 LTM cell switch execution Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall: <unrelated part omitted> 1> if the LTM cell switch is triggered by an indication from lower layers: 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

In accordance with section 5.3.5.18.6 in TS 38.331 [2], the UE performs actions specificed in section 5.3.5.3 in TS 38.331 [2] as shown above and below.

#### 5.3.5.3 Reception of an RRCReconfiguration by the UE The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch: <unrelated part omitted> 1> if the RRCReconfiguration includes the masterCellGroup: 2> perform the cell group configuration for the received masterCellGroup according to 5.3.5.5; <unrelated part omitted> 1> if the RRCReconfiguration includes the secondaryCellGroup: 2> perform the cell group configuration for the SCG according to 5.3.5.5; <unrelated part omitted> #### 5.3.5.5 Cell Group configuration ##### 5.3.5.5.1 General The network configures the UE with Master Cell Group (MCG), and zero or one Secondary Cell Group (SCG). In (NG)EN-DC, the MCG is configured as specified in TS 36.331 [10], and for NE-DC, the SCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE. The UE performs the following actions based on a received CellGroupConfig IE: 1> if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync: 2> perform Reconfiguration with sync according to 5.3.5.5.2;

The LTM candidate configuration (i.e., the RRCReconfiguration) includes a reconfigurationWithSync. As specified in section 5.3.5.5.1 in TS 38.331, the UE performs the Reconfiguration with sync according to section 5.3.5.5.2 in TS 38.331. In section 5.3.5.5.2 in TS 38.331, the RRC layer resets the MAC entity as shown below.

##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. <unrelated part omitted> 1> else (sl-PathSwitchConfig is not included): <unrelated part omitted> 2> else: 3> reset the MAC entity of this cell group;

Upon resetting the MAC entity, the UE performs section 5.12 in TS 38.321 as shown below.

## 5.12 MAC Reset If a reset of the MAC entity is requested by upper layers upon receiving RRCResume or RRCSetup, the MAC entity shall: <unrelated part omitted> Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall: <unrelated part omitted> 1> discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

After resetting the MAC entity, the RRC submits the RRCReconfigurationComplete message to the MAC entity as specified in TS 38.331. The submission of the RRCReconfigurationComplete message triggers the MAC entity to perform a random acces procedure.

As describe in 5.12 in TS 38.321, the UE discards CFRA resources signalled in the LTM Cell Switch Command upon resetting the MAC entity. Therefore, the UE does not have CFRA resoruces to perform CFRA to transmit the RRCReconfigurationComplete message for the LTM cell switch. As a result, the UE performs CBRA to transmit the RRCReconfigurationComplete message for the LTM cell switch.

Observation: 	When the UE receives a LTM Cell Switch Command that configures CFRA resource, the UE discards the CFRA resources in the MAC reset triggerred by the LTM cell switch and therefore, the UE only performs CBRA for the LTM cell switch.

To fix the problem described in the observation, we propose that the UE does not discard the CFRA resources explicitly signalled in the LTM Cell Switch Command in the MAC reset. Instead, the UE discards the CFRA resources only when the UE successfully completes the LTM cell switch.

Proposal: 	The UE does not discard the CFRA resources explicitly signalled in the LTM Cell Switch Command in the MAC reset, and discards the CFRA resources only when the UE successfully completes the LTM cell switch.

# Conclusion

RAN2 is requested to discuss the problem and proposal presented in this paper. The proposed text for TS 38.321 is provided in the annex.

Observation: 	When the UE receives a LTM Cell Switch Command that configures CFRA resource, the UE discards the CFRA resources in the MAC reset triggerred by the LTM cell switch and therefore, the UE only performs CBRA for the LTM cell switch.

Proposal: 	The UE does not discard the CFRA resources explicitly signalled in the LTM Cell Switch Command in the MAC reset, and discards the CFRA resources only when the UE successfully completes the LTM cell switch.

# Annex: Text Proposal

## 5.12	MAC Reset

If a reset of the MAC entity is requested by upper layers upon receiving RRCResume or RRCSetup, the MAC entity shall:

1>	stop the MBS multicast DRX timers;

1>	flush the soft buffers for all DL HARQ processes used for MBS multicast;

1>	for each DL HARQ process used for MBS multicast, consider the next received transmission for a TB as the very first transmission.

Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall:

1>	if the MAC reset is not due to SCG deactivation:

2>	initialize Bj for each logical channel to zero;

1>	initialize SBj for each logical channel to zero if Sidelink resource allocation mode 1 is configured by RRC;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is configured for the deactivated SCG:

2>	stop (if running) all timers except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers.

1>	else:

2>	stop (if running) all timers, except MBS broadcast DRX timers;

2>	consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	sets the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1;

1>	stop, if any, ongoing Random Access procedure;

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any, except the contention-free Random Access Resources signalled in the LTM Cell Switch Command;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Delay Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered consistent LBT failure;

1>	cancel, if any, triggered Sidelink consistent LBT failure;

1>	cancel, if any, triggered BFR;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure;

1>	cancel, if any, triggered Pre-emptive Buffer Status Reporting procedure;

1>	cancel, if any, triggered Timing Advance Reporting procedure;

1>	cancel, if any, triggered Recommended bit rate query procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	cancel, if any, triggered configured sidelink grant confirmation;

1>	clear, if any, configured sidelink grants;

1>	cancel, if any, triggered Desired Guard Symbol query;

1>	cancel, if any, triggered Positioning Measurement Gap Activation/Deactivation Request procedure;

1>	cancel, if any, triggered SDT procedure;

1>	cancel, if any, triggered IAB-MT Recommended Beam Indication query;

1>	cancel, if any, triggered Desired DL TX Power Adjustment query;

1>	cancel, if any, triggered Desired IAB-MT PSD range query;

1>	cancel, if any, triggered Case-6 Timing Request query;

1>	cancel, if any, triggered SL-PRS resource request;

1>	flush the soft buffers for all DL HARQ processes, except for the DL HARQ process being used for MBS broadcast;

1>	for each DL HARQ process, except for the DL HARQ process being used for MBS broadcast, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	clear, if any, Differential Koffset;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is not configured; or

1>	if the MAC reset is not due to SCG deactivation:

2>	reset all BFI_COUNTERs;

1>	reset all LBT_COUNTERs.

If a Sidelink specific reset of the MAC entity is requested for a PC5-RRC connection by upper layers, the MAC entity shall:

1>	flush the soft buffers for all Sidelink processes for all TB(s) associated to the PC5-RRC connection;

1>	consider all Sidelink processes for all TB(s) associated to the PC5-RRC connection as unoccupied;

1>	cancel, if any, triggered Scheduling Request procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink CSI Reporting procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink DRX Command MAC CE associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Request transmission procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Information Reporting procedure associated to the PC5-RRC connection;

1>	stop (if running) all timers associated to the PC5-RRC connection;

1>	reset the numConsecutiveDTX associated to the PC5-RRC connection;

1>	initialize SBj for each logical channel associated to the PC5-RRC connection to zero.

### 5.1.6	Completion of the Random Access procedure

Upon completion of the Random Access procedure, the MAC entity shall:

1>	discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any;

1>	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer and the MSGA buffer.

Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall:

1>	indicate the successful completion of the Random Access procedure to the upper layers.

Upon successful completion of the Random Access procedure initiated for LTM cell switch, the MAC entity shall:

1>	indicate the successful completion of the LTM cell switch to upper layers;

1 > release the contention-free Random Access Resources if signalled in the LTM Cell Switch Command triggering the LTM cell switch.