3GPP TSG-RAN WG2 #129	R2-2500776

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.18

Source: 	Ericsson, T-Mobile US, Deutsche Telekom

Title:  	Addressing Bandwidth Restrictions for MT-SDT

Document for:	Discussion and Decision

# Introduction

During RAN2#128 the unbalanced transmission possibilities between UL and DL data in RRC_INACTIVE with the use of Small Data Transmissions was raised in [1]. An agreement was taken to address the issue.

RAN2#128 Agreement RAN2 will address this issue. FFS the exact solution

This contribution brings up the main motivation for change, as well as alternatives discussed during RAN2#128, with our view for a preferred solution.

# Discussion

## Imbalance in UL and DL data transmission in RRC_INACTIVE

Rel-17 agreements for SDT restrict small data transmissions to the initial Bandwidth part (BWP); the reason being to render SDT procedure as close to legacy initial access procedure as possible.

For UL transmissions initiated as part of MO-SDT, frequency resources across the entire initial UL BWP are available. However, for DL transmission, initial BWP operation restricts the frequency domain resource allocation to the size of CORESET 0. This results in an imbalance between the UL and DL on how much data that can be transmitted in parallel over SDT. Further details of this are available in [1].

It could be emphasized that the goal of addressing this imbalance is not to achieve equal data transfer for the UL and DL, but rather to increase the DL capacity to not force multiple subsequent transmissions in the DL. Earlier simulations and estimations in this area have already been discussed in RAN2 and as shown in [2] and Figure 1, if the data to be sent exceeds approx. 1200 Bytes then the UE would be better served in RRC_CONNECTED instead [2]. Thus, the goal should be to increase the downlink capacity for DL buffers beneath that.

Figure 1; Number of transmissions versus data volume

## Solutions to address the imbalance

During RAN2#128 discussions within the TEI19 agenda item were held and apart from the solution provided in reference [1] some alternatives where proposed. These alternatives could mainly be categorized in two groups, where one tries to utilize the existing resources to the UE (CQI Report in Msg3, discussed in 2.2.2) and one tries to allocates more resources to the UE (Common Frequency Resources, CFR, and moving out of initial DL BWP, discussed in 2.2.1).

### Increase network bandwidth by allocating new resources

Blindly allocating more resources to a UE for the purpose of delivering more data seems like a straightforward solution but is something generally reserved for a UE in RRC_CONNECTED mode and comes with a number of disadvantages as described below.

- Complex UE operation for small transfers – A UE that has to switch to a different bandwidth part for a small amount of data to be transmitted forces unnecessary actions to the UE, causing energy impact and defeats the purpose of RRC_INACTIVE transfers. The UE might as well go to CONNECTED for the transmission.

- The resource utilization for those transfers suffers from the same disadvantages as in legacy SDT, i.e. since no indication of the channel quality, except that the UE has passed the initial RSRP threshold check, with the data must be encoded for the cell edge/worst possible scenarios which is not efficient in many cases.

- Updating the specifications to move the UE out of the initial BWP enforces some broad specification impact.

- RedCap UEs which are operating at 20 MHz have no possibility to utilize a wider BWP. UEs capable of both RedCap and SDT is an expected combination for many use cases. Here despite UL and DL BWPs being comparable, the performance is still impacted without introducing quality reporting, as the UE must still be assumed to be in cell-edge

- Configuring the new BWP that the UE should move to needs to be performed as a separate configuration (e.g. in RRCRelease or a new SIB). Additionally, new signaling might have to be introduced to instruct the UE to move to the new BWP which can introduce additional signalling and significant standard changes.

Moving out of the initial bandwidth part for a transmission characterized as small data creates unnecessary complexity and energy impact.

RAN2 agrees to keep the user on the initial DL bandwidth part while in RRC_INACTIVE.

The proposal of using CFR during RAN2#128 was also discussed. After some investigations we draw the following conclusions.

CFR is used for downlink oriented traffic within the MBS framework.

Using CFR would create dependencies between features and suffers from similar characteristics as in Observation 1.

### Increase utilization of existing resources

As opposed to increasing the amount of frequency resources, this approach instead tries to increase the utilization of existing resources. This approach has been taken for similar functionalities in LTE where 36.321 chapter 6.1.3.19 specifies a MAC CE with a Quality Report that can be sent together with RRCResumeRequest in Msg3.

#### 6.1.3.19 Downlink Channel Quality Report and AS RAI MAC Control Element DCQR and AS RAI MAC control element is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2. A MAC PDU shall contain at most one DCQR and AS RAI MAC control element. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.19-1): - R: Reserved bit, set to "0"; - AS RAI: The field corresponds to Access Stratum Release Assistance Indication as shown in Table 6.1.3.19-1. The length of the field is 2 bits; - Quality Report: For a NB-IoT UE, if npdsch-16QAM-Config is not configured, the report mapping is defined in Table 9.1.22.15-1 in TS 36.133 [9] and if npdsch-16QAM-Config is configured the report mapping is defined in Table 9.1.22.17-1 in TS 36.133 [9]. For a BL UE or UE in CE, the field corresponds to DL channel quality report as defined in TS 36.133 [9]. The length of the field is 4 bits. Figure 6.1.3.19-1: DCQR and AS RAI MAC control element

In LTE, a channel quality indicator in the form of a MAC CE was introduced to enable link adaptation from the network before more detailed information about the channel is available.

This Msg3 Quality Report indicates the downlink quality in order for the network to use proper link adaptation for data transmission in Msg4. However, the SINR range referred to for this quality report is based on LTE SINR ranges which differs versus NR, and thus RAN4 would need to run simulations and agree on a new table which we don’t find to be a suitable way forward.

Sending an LS to RAN4 and requesting simulations and specification work from them to define a new SINR range for a DCQR MAC CE seems not suitable.

Another solution is to report a different measurement, e.g. SS-RSRQ, which is possible for the UE to measure for an SSB for all RRC states. RSRQ is defined in 3GPP 38.215 chapter 5.1.3 as:

Secondary synchronization signal reference signal received quality (SS-RSRQ) is defined as the ratio of N×SS-RSRP / NR carrier RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks.

This means that SS-RSRQ tells us about the ratio for the carrier signal over the complete received signal including interference and noise. With this information the network could draw important conclusions for link adaptation.

RSRQ might not fill the same purpose as SINR, but for this specific purpose the aim is only to have a rough indication for transfers in RRC_INACTIVE mode and not perform a link adaptation like the one for RRC_CONNECTED.

Table 1 below describes the mapping between the reported RSRQ and reported value between 0 and 127.

Table 1; SS-RSRQ mapping in 3GPP 38.133 (Table 10.1.11.1-1)

This could easily fit in a MAC CE with the size of just one octet, with a spare bit available. Further optimizations could be discussed, e.g. to map the RSRQ to fewer bits in a similar fashion as the Quality report from LTE (4 bits), but to limit the specification impact it is suggested to report the whole SS-RSRQ.

Whether or not the UE should acquire the SS-RSRQ at the time of MT-SDT paging, or if it should rely on earlier measurements for RSRQ for reporting while in SDT mode could be left for UE implementation.

RAN2 introduce a Downlink Channel Quality Report MAC CE for NR for the purpose of providing the network with early indication of the radio quality in form of the SS-RSRQ, which has no impact to other WGs.

## Analysis of solution

Based on the discussion in 2.2.1 and 2.2.2 we propose to move forward with the approach of introducing a new MAC CE that reports SS-RSRQ and we have provided technical proposal for this purpose in chapter 5.

Introduce the necessary changes according to TP in Annex.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Moving out of the initial bandwidth part for a transmission characterized as small data creates unnecessary complexity and energy impact.

Observation 2	CFR is used for downlink oriented traffic within the MBS framework.

Observation 3	Using CFR would create dependencies between features and suffers from similar characteristics as in Observation 1.

Observation 4	In LTE, a channel quality indicator in the form of a MAC CE was introduced to enable link adaptation from the network before more detailed information about the channel is available.

Observation 5	Sending an LS to RAN4 and requesting simulations and specification work from them to define a new SINR range for a DCQR MAC CE seems not suitable.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 agrees to keep the user on the initial DL bandwidth part while in RRC_INACTIVE.

Proposal 2	RAN2 introduce a Downlink Channel Quality Report MAC CE for NR for the purpose of providing the network with early indication of the radio quality in form of the SS-RSRQ, which has no impact to other WGs.

Proposal 3	Introduce the necessary changes according to TP in Annex.

# References

3GPP R2-2410542. “Quality Indication in Msg3 for SDT”, Ericsson

3GPP, R2-2307934, ‘’ Handling BWP restrictions in MT-SDT’’, Ericsson, T-Mobile USA, Deutsche Telekom

# ANNEX – Technical Proposal

### 6.1.3	MAC Control Elements (CEs)

<Unnecessary chapters omitted>

#### 6.1.3.X	Downlink Channel Quality Report MAC CE

The Downlink Channel Quality Report MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2b.

It has a fixed size and consists of one octet defined as follows (Figure 6.1.3.X-1):

-	R: Reserved bit, set to 0;

-	SS-RSRQ: This field indicates the SS-RSRQ as measured by the UE (according to TS 38.215). The length of the field is 7 bits.

NOTE 1:	When the UE collects the SS-RSRQ it is up to UE implementation whether a new value is collected or if the UE maintains an SS-RSRQ for the duration of the SDT procedure.

| R | SS-RSRQ |  |  |  |  |  |  |  |  |  | Oct 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 6.1.3.X-1: Downlink Channel Quality Report MAC CE

Table 6.2.1-2b: Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 217 | 64 to 281 | Reserved |
| 218 | 282 | Downlink Channel Quality Report |
| 219 | 283 | Enhanced Multiple Entry PHR for multiple TRP STx2P (four octets Ci) |
| 220 | 284 | Enhanced Multiple Entry PHR for multiple TRP STx2P (one octets Ci) |
| 221 | 285 | Enhanced Single Entry PHR for multiple TRP STx2P |
| 222 | 286 | SL LBT Failure |
| 223 | 287 | Multiple Entry PHR with assumed PUSCH (four octets Ci) |
| 224 | 288 | Multiple Entry PHR with assumed PUSCH (one octets Ci) |
| 225 | 289 | Single Entry PHR with assumed PUSCH |
| 226 | 290 | SL-PRS Resource Request |
| 227 | 291 | Refined Long BSR |
| 228 | 292 | Delay Status Report |
| 229 | 293 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
| 230 | 294 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
| 231 | 295 | Enhanced Single Entry PHR for multiple TRP |
| 232 | 296 | Enhanced Multiple Entry PHR (four octets Ci) |
| 233 | 297 | Enhanced Multiple Entry PHR (one octets Ci) |
| 234 | 298 | Enhanced Single Entry PHR |
| 235 | 299 | Enhanced BFR (one octet Ci) |
| 236 | 300 | Enhanced BFR (four octet Ci) |
| 237 | 301 | Truncated Enhanced BFR (four octet Ci) |
| 238 | 302 | Positioning Measurement Gap Activation/Deactivation Request |
| 239 | 303 | IAB-MT Recommended Beam Indication |
| 240 | 304 | Desired IAB-MT PSD range |
| 241 | 305 | Desired DL Tx Power Adjustment |
| 242 | 306 | Case-6 Timing Request |
| 243 | 307 | Desired Guard Symbols for Case 6 timing |
| 244 | 308 | Desired Guard Symbols for Case 7 timing |
| 245 | 309 | Extended Short Truncated BSR |
| 246 | 310 | Extended Long Truncated BSR |
| 247 | 311 | Extended Short BSR |
| 248 | 312 | Extended Long BSR |
| 249 | 313 | Extended Pre-emptive BSR |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for Downlink Channel Quality Report, when Downlink Channel Quality Report is to be included in Msg3;

-	MAC CE for SL-PRS Resource Request;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

#### 5.4.3.2	Multiplexing of MAC Control Elements and MAC SDUs

The MAC entity shall multiplex MAC CEs and MAC SDUs in a MAC PDU according to clauses 5.4.3.1 and 6.1.2.

NOTE:	Content of a MAC PDU does not change after being built for transmission on a dynamic uplink grant, regardless of LBT outcome.

## 5.27	Small Data Transmission

### 5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT cannot be performed by RA-SDT (i.e., RA-SDT is not applicable as specified in clause 5.1.1b), but can be performed either by Random Access procedure (i.e., with 2-step RA type or 4-step RA type) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT;

-	cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT;

-	sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

The following UE variable is used for the SDT procedure:

-	MAX_DURATION_TO_NEXT_CG_OCCASION;

-	RSRP_THRESHOLD.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

### 5.27.2	TA Validation for CG-SDT

RRC configures the following parameters for TA validation for CG-SDT:

-	cg-SDT-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation.

The MAC entity shall, upon the reception of CG-SDT configuration:

1>	store the current RSRP of the downlink pathloss reference for TA validation as defined in TS 38.331 [5] clause 5.7.17.

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled:

1>	The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and

1>	Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and

1>	cg-SDT-TimeAlignmentTimer is running.

### 5.27.X	Transmission of Downlink Channel Quality Report

The MAC entity of a UE with an SDT configuration may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported.

If the UE has been instructed by higher layers to report DL channel quality in Msg3:

1>	collect SS-RSRQ according to TS 38.215.

NOTE 1:	When the UE collects the SS-RSRQ it is up to UE implementation whether a new value is collected or if the UE maintains a fresh SS-RSRQ for the duration of the SDT procedure.

1> if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI:

2>	if the allocated resources can accommodate a Downlink Channel Quality Report MAC control element plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate a Downlink Channel Quality Report MAC control element as defined in clause 6.1.3.X;

2>	else:

3>	cancel the triggered "Msg3 Downlink Channel Quality Report";