3GPP TSG-RAN2 Meeting #129	R2-2500899

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			ZTE Corporation

Title: 	Discussion on RRM measurement prediction

Agenda item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

In the RAN2 126 meeting, the simulation scenario and the corresponding prioritization for RRM measurement prediction are defined [1], as follows:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this document, we’d like to provide our simulation results and the corresponding analysis for temporal domain prediction.

- Discussion

## 2.1 General

In the RAN2 127bis meeting, we have agreed that companies are free to consider non-AI and simple AI. In this document, we provide our simulation results with non-AI approach for temporal domain prediction to study how much the gain that AI can provide. The general idea of non-AI approach is ‘sample and hold’, but details (e.g. which result to be held) are different with different RRM sub use cases.

Regarding non-AI approach for RRM sub use case 1 (L1 beam result -> L1 beam result), the L1 measurement results within prediction window reuses the L1 measurement result from last actual measurement, and then L3 filtering is performed as legacy. Take Figure 1 as example, the predicted result with non-AI approach at T3 is calculated by the formula: (1-L3 filtering parameter) * L3 filtered results at T2 + L3 filtering parameter * L1 filtered results at T2.

Regarding non-AI approach for RRM sub use case 2 (L3 filtered cell result  L3 filtered cell result), the L3 filtered measurement results within prediction window reuses the L3 filtered measurement results from last actual measurement. Take Figure 1 as example, the predicted results with non-AI approach at T3 = L3 filtered results at T2 = (1-filtering parameter) *L3 filtered result at T1 + L3 filtering parameter * L1 filtered results at T2. So, in our simulation, the simulation results with non-AI approach are different with different RRM sub use cases in the same condition.

Figure 1: An example for temporal domain prediction

## 2.2 FR1 to FR1 intra-freq temporal domain case B

FR1 to FR1 intra frequency temporal domain case B is done by predicting sub set measurement instances in temporal domain of the same cell. The tables below show some important simulation assumptions in our simulation and the corresponding simulation results.

Table 2.2.1: Simulation temple table for FR1 to FR1 temporal domain measurement prediction case B

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in [4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | Whether LOSsoft is modeled or not | Not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.2.2 |

Table 2.2.2: Simulation results for temporal domain prediction case B

| MRRT | [observation window, prediction window] and detailed skipping pattern | RRM sub use case |  | ARSRP difference with UE speed = 30km/h | ARSRP difference with UE speed = 90km/h |
| --- | --- | --- | --- | --- | --- |
| 50% | [600ms, 200ms] skipping pattern: 1:1 | RRM sub use case 1 | AI | 0.502 | 0.693 |
|  |  |  | Non-AI | 0.538 | 0.722 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.522 | 0.673 |
|  |  |  | Non-AI | 0.636 | 0.775 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.521 | 0.701 |
|  |  |  | Non-AI | 0.542 | 0.904 |
|  | [1000ms,200ms] skipping pattern: 1:1 | RRM sub use case 1 | AI | 0.498 | 0.679 |
|  |  |  | Non-AI | 0.538 | 0.722 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.508 | 0.668 |
|  |  |  | Non-AI | 0.636 | 0.775 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.508 | 0.669 |
|  |  |  | Non-AI | 0.542 | 0.904 |
|  | [400ms,400ms] skipping pattern: 2:2 | RRM sub use case 1 | AI | 0.452, 0.592 | 0.618, 0.930 |
|  |  |  | Non-AI | 0.513, 0.667 | 0.668, 1.015 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.469, 0.611 | 0.607, 0.912 |
|  |  |  | Non-AI | 0.474, 0.615 | 0.630, 0.959 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.482, 0.644 | 0.658, 0.974 |
|  |  |  | Non-AI | 0.486, 0.648 | 0.776, 1.151 |
|  | [1200ms,400ms] skipping pattern: 2:2 | RRM sub use case 1 | AI | 0.449, 0.579 | 0.595, 0.895 |
|  |  |  | Non-AI | 0.513, 0.667 | 0.668 1.015 |
|  |  | RRM sub use case 2+ filtering option 2 | AI | 0.453, 0.609 | 0.599, 0.891 |
|  |  |  | Non-AI | 0.474, 0.615 | 0.630, 0.959 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.452, 0.598 | 0.598, 0.877 |
|  |  |  | Non-AI | 0.486, 0.648 | 0.776, 1.151 |
| 60% | [400ms,600ms] skipping pattern: 2:3 | RRM sub use case1 | AI | 0.465, 0.612, 0.722 | 0.625, 0.917, 1.203 |
|  |  |  | Non-AI | 0.523, 0.682, 0.801 | 0.681, 1.015, 1.354 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.474, 0.623 0.742 | 0.601, 0.896, 1.195 |
|  |  |  | Non-AI | 0.480, 0.627, 0.747 | 0.626, 0.945, 1.274 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.499, 0.663, 0.788 | 0.666, 0.960, 1.234 |
|  |  |  | Non-AI | 0.511, 0.675, 0.808 | 0.839, 1.192, 1.543 |
|  | [1400ms,600ms] Skipping pattern: 2:3 | RRM sub use case 1 | AI | 0.463, 0.589, 0.695 | 0.598, 0.844, 1.075 |
|  |  |  | Non-AI | 0.523, 0.682, 0.801 | 0.681, 1.015, 1.354 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.461, 0.613, 0.739 | 0.592, 0.879, 1.141 |
|  |  |  | Non-AI | 0.480, 0.627, 0.747 | 0.626, 0.945, 1.274 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.458, 0.603 0.712 | 0.590, 0.887, 1.125 |
|  |  |  | Non-AI | 0.511, 0.675, 0.808 | 0.839, 1.192, 1.543 |
| 66.7% | [800ms, 400ms] Skipping pattern: 1:2 | RRM sub use case 1 | AI | 0.544, 0.674 | 0.779, 1.050 |
|  |  |  | Non-AI | 0.557, 0.711 | 0.801, 1.141 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.544, 0.692 | 0.700, 0.987 |
|  |  |  | Non-AI | 0.627, 0.753 | 0.767, 1.008 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.553, 0.709 | 0.725, 0.994 |
|  |  |  | Non-AI | 0.627, 0.780 | 1.131, 1.478 |
|  | [400ms, 800ms] skipping pattern: 2:4 | RRM sub use case1 | AI | 0.464, 0.599, 0.708, 0.812 | 0.650, 0.953 1.231, 1.517 |
|  |  |  | Non-AI | 0.510, 0.657, 0.775, 0.894 | 0.710, 1.046, 1.366, 1.693 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.470, 0.613 0.727, 0.840 | 0.598, 0.908 1.208, 1.493 |
|  |  |  | Non-AI | 0.476, 0.622, 0.739, 0.848 | 0.618, 0.948, 1.269, 1.600 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.497, 0.648 0.771, 0.890 | 0.691, 0.983 1.256, 1.516 |
|  |  |  | Non-AI | 0.514, 0.671, 0.795, 0.921 | 0.894, 1.246, 1.576, 1.914 |
|  | [1600ms,800ms] skipping pattern: 2:4 | RRM sub use case 1 | AI | 0.458, 0.583, 0.677, 0.772 | 0.616, 0.871, 1.105, 1.356 |
|  |  |  | Non-AI | 0.511, 0.658, 0.776, 0.895 | 0.711, 1.047, 1.368, 1.694 |
|  |  | RRM sub use case 2 + filtering option 2 | AI | 0.471, 0.619, 0.733, 0.835 | 0.593, 0.887, 1.156, 1.407 |
|  |  |  | Non-AI | 0.476, 0.622, 0.739, 0.848 | 0.618, 0.948, 1.269, 1.600 |
|  |  | RRM sub use case 2 + filtering option 3 | AI | 0.459, 0.604, 0.719, 0.826 | 0.592, 0.878, 1.130, 1.377 |
|  |  |  | Non-AI | 0.516, 0.673, 0.797, 0.923 | 0.898, 1.251, 1.580, 1.918 |

- The impact of filtering option

In last meeting, we have agreed that companies can report which filtering options are used for the input L3 RSRP of sub-use case 2. In this part, we study the impact of filtering option on the prediction performance. In our simulation, filtering option 2 and option 3 are adopted. The simulation results show that the prediction with two filtering options provide similar prediction accuracy, the maximum RSRP difference between the average prediction error with two filtering options is less than 0.062dB in the same condition, and the minimum RSRP difference is only

Observation 1: Regarding temporal domain prediction case B, filtering option 2 and filtering option 3 provide similar prediction performance.

- AI based approach vs Non-AI based approach

In addition, we also observe that in high speed scenario, the AI approach can provide more gain compared with non-AI. For example, with UE speed = 30km/h, the best gain that AI can provide is 20.1%; while with UE speed = 90km/h, the best gain that AI can provide is 32.7%. The reason is: the channel condition in high speed scenario changes more rapidly, so the sample and hold cannot work well.

Observation 2: Regarding temporal domain prediction case B, the gain that AI can provide is more obvious in high speed scenario.

Proposal 1: Capture the observation 1~2 for temporal domain prediction case B in the TR.

## 2.3 FR2 to FR2 intra-freq temporal domain case A

For intra-frequency temporal domain case A, continuous historical measurement results are used to predict continuous future measurement results, and no measurement is reduced. The table below show some important simulation assumption in our simulation and the corresponding simulation results.

Table 2.3.1: Simulation temple table for FR2 to FR2 temporal domain measurement prediction case A

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | Whether LOSsoft is modeled or not | Not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.3.2 |

Table 2.3.2: simulation results for temporal domain case A

| RRM sub use case | Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |  |
| --- | --- | --- | --- | --- |
| RRM sub use case 1 | 60 | [800ms, 1200ms] | AI | 0.795, 1.361, 2.042 |
|  |  |  | Non-AI | 0.799, 1.869, 2.959 |
|  |  | [800ms, 800ms] | AI | 0.810, 1.391 |
|  |  |  | Non-AI | 0.829, 1.955 |
|  |  | [800ms, 400ms] | AI | 0.758 |
|  |  |  | Non-AI | 0.831 |
|  |  | [1200ms,400ms] | AI | 0.739 |
|  |  |  | Non-AI | 0.831 |
|  |  | [1600ms,400ms] | AI | 0.746 |
|  |  |  | Non-AI | 0.831 |
|  | 120 | [800ms, 1200ms] | AI | 1.109, 1.987, 2.920 |
|  |  |  | Non-AI | 1.213, 2.909, 4.546 |
|  |  | [800ms, 800ms] | AI | 1.087, 1.945 |
|  |  |  | Non-AI | 1.203, 2.932 |
|  |  | [800ms, 400ms] | AI | 0.992 |
|  |  |  | Non-AI | 1.206 |
|  |  | [1200ms,400ms] | AI | 0.961 |
|  |  |  | Non-AI | 1.206 |
|  |  | [1600ms,400ms] | AI | 0.963 |
|  |  |  | Non-AI | 1.206 |
| RRM sub use case 2 | 60 | [800ms, 1200ms] | AI | 0.665, 1.394, 2.178 |
|  |  |  | Non-AI | 1.138, 2.213, 3.265 |
|  |  | [800ms, 800ms] | AI | 0.687, 1.400 |
|  |  |  | Non-AI | 1.208, 2.340 |
|  |  | [800ms, 400ms] | AI | 0.680 |
|  |  |  | Non-AI | 1.248 |
|  |  | [1200ms,400ms] | AI | 0.672 |
|  |  |  | Non-AI | 1.248 |
|  |  | [1600ms,400ms] | AI | 0.666 |
|  |  |  | Non-AI | 1.248 |
|  | 120 | [800ms, 1200ms] | AI | 0.936, 2.059, 3.193 |
|  |  |  | Non-AI | 1.773, 3.453, 5.072 |
|  |  | [800ms, 800ms] | AI | 0.941, 2.015 |
|  |  |  | Non-AI | 1.844, 3.574 |
|  |  | [800ms, 400ms] | AI | 0.946 |
|  |  |  | Non-AI | 1.886 |
|  |  | [1200ms,400ms] | AI | 0.916 |
|  |  |  | Non-AI | 1.886 |
|  |  | [1600ms,400ms] | AI | 0.914 |
|  |  |  | Non-AI | 1.886 |

- AI based approach vs Non-AI based approach

Similar as in temporal domain case B, the gain that AI model can provide increases with the increase of prediction window length. For example, if prediction window length is 400ms, the best gain that AI can provide is only 0.972dB, while if the prediction window is 1200ms, the best gain can achieve 1.879dB.

Observation 3: Regarding the temporal domain prediction case A, the gain that AI can provide increases with the increase of prediction window length.

Proposal 2: Capture the observation 3 for temporal domain prediction case A in the TR.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

- FR1 to FR1 intra-freq temporal domain case B

Observation 1: Regarding temporal domain prediction case B, filtering option 2 and filtering option 3 provide similar prediction performance.

Observation 2: Regarding temporal domain prediction case B, the gain that AI can provide is more obvious in high speed scenario.

- FR2 to FR2 intra-freq temporal domain case A

Observation 3: Regarding the temporal domain prediction case A, the gain that AI can provide increases with the increase of prediction window length.

Proposal 1: Capture the observation 1~2 for temporal domain prediction case B in the TR.

Proposal 2: Capture the observation 3 for temporal domain prediction case A in the TR.

- Reference

- Report of RAN2 126 meeting.