3GPP TSG-RAN WG2 #129	R2-2501004

Athens, Greece, Feb. 17th – 21st 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2	Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#128[2], the following agreements are made for S&F operation:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

This paper attempts to investigate about consideration on S&F operation based on the above objectives and agreements.

- Discussion

2.1 Remaining time of current operation mode

In RAN2 #129, it was agreed that the remaining time of current operation mode can be informed via System Information broadcast. As mentioned in the agreement, Rel-19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed. Besides the delay of NAS procedures, we think this information could be useful for cell reselection to search a cell which has longer remaining time in real-time mode (or shorter remaining time in S&F mode). In this case, the UE has to know current operation mode and remaining time for neighbor satellites. Therefore, we would like to propose that S&F related information for neighbor satellites (current operation mode, remaining time) could be informed to UE.

Observation 1: Remaining time of current operation mode could be useful during cell reselection procedure to select a cell which has longer remaining time of real-time mode.

Proposal 1: S&F related information for neighbor satellites (current operation mode, remaining time) could be informed to UE.

Based on the chair notes of RAN2 #129, which SIB is used for the remaining time information is still FFS. We think SIB31 is suitable rather than SIB1, since the information is not necessary for the UEs not supporting S&F operation, and the UEs supporting S&F operation might be required anyway to monitor SIB31 for Satellite ID List.

Proposal 2: Remaining time of current operation mode could be broadcasted via SIB31.

2.2 Existing RRC connection at the mode switch

It has been already agreed that UE not supporting S&F capability should be barred on “S&F mode”. In the case where the UE not supporting S&F already has established RRC connection before switching to “S&F mode”, we think the RRC connection should be released when the satellite is going to “S&F mode” from “real-time mode”, since the UE not supporting S&F most likely does not expect the significant delay on S&F operation.

Proposal 3: RRC connection of UE not supporting S&F should be released with RRCConnectionRelease message when the satellite is going to “S&F mode” from “real-time mode”.

Furthermore, during the last meeting, it is still FFS whether a new release cause will be introduced for the case where the network wants to release a Rel-19 UE because the feeder link becomes unavailable. We think the new release cause is not necessary although it could be used to avoid re-connection during mode switching. Instead of the new release cause, waitTime in RRCConnectionRelease could be used to avoid re-connection until the completion of mode switching. The network may not be able to estimate the exact timing when the feeder link becomes unavailable actually, however, the network could change the S&F indication in SIB1 to “S&F mode” in advance and set waitTime to the time duration until next the SIB1 with new S&F indication.

Observation 2: To avoid re-connection during mode switching, waitTime could work by setting the time duration until the next SIB1 with new S&F mode indication.

Proposal 4: The new release cause would not be necessary to avoid re-connection during mode switching.

2.3 UE requirement indication

It could be expected that UE’s UL data may have period of validity even though S&F UE should be delay-tolerant, since the delay on S&F operation could be significantly longer than the delay expected on the legacy delay-tolerant communication. Based on this assumption, storing & forwarding the UL data would be wasteful after the actual delay exceeds UE’s tolerable delay. Therefore, we would like to propose to introduce a mechanism to indicate UE’s requirement for UL data transmission (e.g., level of delay-tolerance, data expiration time) from UE to satellite, so that satellite can discard the data for storing & forwarding when the actual delay exceeds the requirement.

Proposal 5: RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to satellite

- Summary and proposal

This paper discussed about consideration on S&F operation and proposed the following:

Observation 1: Remaining time of current operation mode could be useful during cell reselection procedure to select a cell which has longer remaining time of real-time mode.

Proposal 1: S&F related information for neighbor satellites (current operation mode, remaining time) could be informed to UE.

Proposal 2: Remaining time of current operation mode could be broadcasted via SIB31.

Proposal 3: RRC connection of UE not supporting S&F should be released with RRCConnectionRelease message when the satellite is going to “S&F mode” from “real-time mode”.

Observation 2: To avoid re-connection during mode switching, waitTime could work by setting the time duration until the next SIB1 with new S&F mode indication.

Proposal 4: The new release cause would not be necessary to avoid re-connection during mode switching.

Proposal 5: RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to satellite

- References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2410913, “Report from Break-out session on NR-NTN and IoT-NTN”