3GPP TSG-RAN WG2 Meeting #129	R2-2501045

Athens, GR, 17 – 21 Feb, 2025

Agenda Item:	8.10.5.3

Source:	CMCC, CATT, Ericsson, Huawei, Lenovo, ZTE

Title:	MHI/UHI Enhancement for SCG Deactivation/Activation

Document for:	Discussion

# 1 Introduction

In RAN2#127 meeting, potential MHI/UHI Enhancement for SCG Deactivation/Activation was discussed and following agreements achieved [1]:

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

Then, in the RAN3#126, RAN3 discussed the solution of MHI/UHI Enhancement for SCG Deactivation/Activation, and understands the UE-based solution is beneficial for monitoring SCG deactivation/activation across RRC state transitions, which included in the reply LS from RAN3 [2]:

RAN3 thanks RAN2 for the LS on MHI enhancement solution for SCG deactivation/activation. RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation. RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

Based on the reply LS, we further provide our consideration for the UE-based solution in this contribution.

# 2 Discussion

2.1 MHI and UHI

The mechanism for SCG activation and deactivation is introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

In our understanding, the information of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity, optimize the configuration of Dual Connectivity, and further optimize the SCG activation and deactivation. .

Currently, Mobility History Information (MHI) and UE History Information (UHI) are used to record Dual Connectivity related information, but no information about SCG activation or deactivation is recorded by the UE in current MHI and by the network in UHI.

Observation 1: The SCG activation/deactivation information could help the network understand the actual utilization of Dual Connectivity, optimize the configuration of Dual Connectivity, and further optimize the SCG activation and deactivation.

Observation 2: Currently, no information about SCG activation/deactivation is recorded by the UE in MHI.

2.2 Necessity of UE-based solution

In RAN2#127, RAN2 reached an agreement that it is beneficial for the network having information about time spent in the PSCell in activated state or deactivated state, and two solutions were discussed.

Option 1: Network based solution: The network collects the time information in PSCell in activated or deactivated state.

Option 2: UE-based solution: The UE collects the time information in PSCell in activated or deactivated state.

With these two solutions, RAN2 sent an LS to RAN3 to ask whether a UE-based solution is needed. Then, in RAN3#126, RAN3 discussed whether network based solution is sufficient for enhancing SCG activation/deactivation, and recognizing the benefits of a UE-based solution for monitoring SCG deactivation/activation across RRC state transitions.

From our perspective, the network based solution will only record UE mobility information in UHI when UE is in connected state, and will not record any information when UE moves into idle/inactive state, which we think is not sufficient enough. To enable the network having a full picture of the UE mobility, the information about UE in idle/inactive state is needed, which can be provided by the UE-based solution as UE can record this information in MHI and report to the network when coming to connected state from idle/inactive. Also, the network will release all the UE context when UE leaves connected state. But if the same UE moves into connected state again, the network will record this UE as a new UE, which means that the UHI reports for the same UE cannot be associated together.

The following example with 3 phases illustrates the benefits of UE-based solution with more details, which including the change of UE states and the corresponding information collected by network in UHI and recorded by UE in MHI respectively.

In Phase 1, the UE works in RRC_CONNECTED state and in the coverage of PCell 1. Initially the UE is configured with PSCell A, and after 30s, it is configured with PSCell B for 20s. Then the UE returns to standalone for 10s. It is observed that, the information recorded in MHI and UHI are the same.

Figure 1: MHI and UHI for UE in RRC_CONNECTED

In Phase 2, the UE goes to RRC_IDLE state for 120s and still in the coverage of PCell 1. It is observed that, the information of 120s in PCell 1 in idle is only recorded in the MHI of UE, but such information cannot be recorded by network, because all the UE context has been removed by the network.

Figure 2: MHI and UHI for UE in RRC_IDLE

In Phase 3, the UE goes back to RRC_CONNECTED state and configured with PCell 1 and PSCell A for another 60s. It is observed that, the information of the another 60s in RRC_CONNECTED is recorded by UE in MHI with the same entry of PCell 1. But for network, it has no idea that the same UE comes back, and it will start to record the mobility information using a new UE identity, i.e. UE2, in its UHI.

Figure 3: MHI and UHI for UE when state transition happens

Therefore, it is observed that when UE moves from RRC_CONNECTED to RRC_IDLE, then back to RRC_CONNECTED state, the network only keeps the the information of UE in RRC_CONNECTED state and could not associate the UHI reports of RRC_CONNECTED state for the same UE for DC optimization, since the network has no idea about the PSCell information neither in previous RRC_CONNECTED state nor in RRC_IDLE state. But UE’s MHI report has all of the information among RRC state transition, which provides a full picture of the UE’s mobility and traffic, and can also help the network understand the coverage of the wireless network.

Observation 3: For network-based solution, the network could only record the mobility information when UE is in RRC_CONNECTED state, and the information of RRC_IDLE is lost.

Observation 4: For network-based solution, the network could not link the two entries of the same UE for DC optimization.

Observation 5: For UE-based solution, the UE MHI report could record the whole mobility history information, including cells visited in RRC_IDLE, and RRC_CONNECTED states, and it is be beneficial for network to monitor SCG deactivation/activation across RRC state transitions.

Besides, in RAN3#126 discussion, major companies see the benefit of UE based solution for monitoring SCG deactivation/activation across RRC state transition, and the following is captured in the Chair Notes:

UHI/MHI: RAN3 agree to support a network-based solution for enhancing PSCell activation/deactivation. Can we agree on if a network-based solution is sufficient for enhancing PSCell activation/deactivation or not? HW:It’s valuable to have UE-based solution as well. CMCC, ZTE, CATT, Lenovo: UE has the whole picture, support UE-based solution SS: The additional information in UE-based solution is the information in IDLE and INACTIVE mode. Do not see the benefits to UE-based solution. Nok: Support both

Observation 6: During the discussion in RAN3#126, major companies see the benefit of UE based solution, and believe that UE based solution is beneficial for monitoring SCG deactivation/activation across RRC state transition, as the network cannot obtain UE trajectory in IDLE/INACTIVE state.

Based on the above consideration and RAN3’s discussion, we believe that the network based solution is insufficient for optimization of dual connectivity configuration, and UE-based solution is necessary to be supported.

Proposal 1: RAN2 supports UE-based solution for the MHI enhancement of SCG activation/deactivation.

2.3 Details of SCG activation/deactivation

For the details information of SCG activation/deactivation included in MHI, the relative time of UE in SCG activated state or percentage of time that SCG activation could be considered. For instance, if the time or percentage of time that SCG activation is quite low, it means that the Dual Connectivity is possibly not so necessary for the UE, and the network might need not configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. On the other hand, if the time or percentage of time that SCG activation is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently.

Observation 7: The detailed information of SCG activation/deactivation could include the time of SCG activation, or percentage of time that SCG activation, which could help the network evaluate whether to configure and further activate SCG.

As RAN3 already supports network based solution, we believe that RAN2 and RAN3 should align on the details of the enhancement.

Proposal 2: Include the information of SCG activation and/or deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation, in MHI, which is aligned with NW-based solution.

# 3 Conclusion

This contribution provides our considerations on the MHI/UHI enhancement for SCG activation and deactivation, enabling network to understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, following observations and proposal are made:

Observation 1: The SCG activation/deactivation information could help the network understand the actual utilization of Dual Connectivity, optimize the configuration of Dual Connectivity, and further optimize the SCG activation and deactivation.

Observation 2: Currently, no information about SCG activation/deactivation is recorded by the UE in MHI.

Observation 3: For network-based solution, the network could only record the mobility information when UE is in RRC_CONNECTED state, and the information of RRC_IDLE is lost.

Observation 4: For network-based solution, the network could not link the two entries of the same UE for DC optimization.

Observation 5: For UE-based solution, the UE MHI report could record the whole mobility history information, including cells visited in RRC_IDLE, and RRC_CONNECTED states, and it is be beneficial for network to monitor SCG deactivation/activation across RRC state transitions.

Observation 6: During the discussion in RAN3#126, major companies see the benefit of UE based solution, and believe that UE based solution is beneficial for monitoring SCG deactivation/activation across RRC state transition, as the network cannot obtain UE trajectory in IDLE/INACTIVE state.

Proposal 1: RAN2 supports UE-based solution for the MHI enhancement of SCG activation/deactivation.

Observation 7: The detailed information of SCG activation/deactivation could include the time of SCG activation, or percentage of time that SCG activation, which could help the network evaluate whether to configure and further activate SCG.

Proposal 2: Include the information of SCG activation and/or deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation, in MHI, which is aligned with NW-based solution.

# Reference

[1]	Draft_R2_127_Meeting_Report

[2] R3-247906, Reply LS on MHI enhancement solution for SCG deactivation/activation, Huawei