# 3GPP TSG-WG2 Meeting #129                                            R2-2501053

Athens, GR, 17th – 21st February 2025

Source: 			CMCC, ZTE, MediaTek, vivo, Huawei, Meta, CATT

Title:  		    Discussion on UE aggregation enhancement

Agenda item:	8.18

Document for: 	Discussion

Introduction

R17 SL relay is mainly focused on single path, where remote UE is connected to network via SL path. R18 SL relay enhancement extends the scenario to multi-path, one direct path (Uu link) and one indirect path (SL or Non-3GPP link). The direct path and indirect path are limited to the same gNB.

For TEI 19, we suggest to consider some enhancements on multi-path to meet the service requirements.

Discussion

## 2.1 The current specifications on Multi-Path Relay

3GPP RAN approved a work item “NR SL relay enhancements”[1] in R18 to address the enhancements and solutions required for supporting the UE-to-network Relay. The inclusion of multi-path with relay in R18, where a remote UE is connected to network via direct and indirect paths, holds the potential to enhance the reliability, robustness and throughput. This multi-path relay solutions are almost be utilized to for UE aggregation, a scenario where a UE is connected to the network via direct path and via another UE using a non-3GPP connection. UE aggregation is designed to cater applications requiring high UL bit rates on 5G terminals, in cases when normal UEs are too limited by UL UE transmission power to achieve required bit rates, especially at the edge of a cell. Moreover, UE aggregation can improve the reliability, stability and reduce delay of services as well, that is, if the channel condition of a terminal is deteriorating, another terminal can be used to make up for the traffic performance unsteadiness caused by channel condition variation.

However, due to time constraints, R18 only supports one direct path and one indirect path (PC5 or N3C) for UE aggregation, which may limit the full potential of multi-path relay solutions in enhancing network performance and service quality. The system gain from a single indirect path in UE aggregation is also constrained by the uplink transmission capability of the relay UE, as well as by mobility and radio link conditions.

Fig 1. The current multi-path relay

Observation 1: In current RAN2 specifications on multi-path relay, only a single indirect path (PC5 or N3C) for UE aggregation is supported.

## 2.2 The motivation on supporting multiple indirect paths

In application scenarios that demand UE aggregation, such as large-scale live broadcasting, there is a significant need for high uplink transmission rates and reliability. This makes it particularly important to increase the number of assisting relay UEs. For instance, in scenarios requiring ultra-long-range coverage, such as maritime fishing operations or offshore oil extraction platforms, terminals would ideally need to emit a transmission power of approximately 33dBm to cover the vast distances. To achieve this with typical terminals that have a transmission power of 23dBm, it would be necessary to aggregate signals from ten such terminals.

- Example of large-scale live broadcasting    2) Example of ultra-long-range coverage

- Fig 2. The example scenarios for UE aggregation (which are generated by AI)

This aggregation is crucial for meeting the high demands of uplink bitrates and ensuring the reliability of services, especially in environments where the transmission power of individual UEs may not be sufficient to achieve the required performance. By increasing the number of assisting relay UEs, the system can better accommodate the needs of high-data-rate applications and improve the overall robustness of the network.

Observation 2: One indirect path in UE aggregation cannot meet the requirements of some use cases, e.g. large-scale live broadcasting, maritime fishing operations or offshore oil extraction platforms.

Fig 3. The example for multiple indirect paths via N3C

Based on observation 1 and observation 2, we suggest to support multiple indirect paths for multi-path relay. Considering there may be more potential spec impacts for PC5 than N3C, and the remaining time is limited in Rel-19, we prefer to only support multi-path relay via N3C in TEI19.

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

In current specs, MP Remote UE and MP Relay UE are in the intra-gNB, and PCell is always on the direct path. Considering to avoid the potential impacts on other WGs (e.g. RAN3) and minimum the workload, we prefer that the multiple indirect paths are under the same gNB, and CU-DU split structure is not considered in TEI19.

Proposal 2：It is proposed to only consider the scenario where multiple indirect paths under the same gNB, and CU-DU split structure is not considered in TEI19.

## 2.3 The potential spec impacts for multiple indirect path

To support the multiple indirect paths via N3C, we think the following issues should be discussed, and the impacts on TS 38.300 [2], TS 38.323 [3], TS 38.321 [4], TS 38.331 [5] and TS 38.306 [6] may be considered. Moreover, we provide the analysis on these issues in the subsequent sections.

- Scenario

- PDCP layer

- Multi-path configuration, deactivation/activation

- Split bearer

- Duplication

- Path failure

- UE capability

### 2.3.1 Description of scenario

- As we proposed in P1 and P2, the description of scenario should be captured in TS 38.300 [2], i.e. only consider the scenario where multiple indirect paths via N3C under the same gNB.

- Proposal 3: It is proposed to add the description of scenario in P1 and P2 to TS 38.300.

- In current spec, the UE can report up to 8 relay UEs’ information to the the network using UEAssistanceInformation message. To minimum the spec impacts, we prefer to reuse the current maximum number for N3C relay UE, i.e. 8, for multiple indirect paths via N3C. That is, the network can configure at most 8 N3C relay UEs which are reported by remote UE in UAI.

n3c-RelayUE-InfoList-r18 SEQUENCE (SIZE (0..8)) OF N3C-RelayUE-Info-r18

- Proposal 4: It is proposed to reuse the maximum number (i.e. 8) for multiple indirect paths via N3C.

- As specified in TS 38.323 [3], for the case of N3C indirect path for multi-path, all PDCP entities of the MP remote UE are associated with the N3C. To support the multiple indirect paths via N3C links, the PDCP entity can be associated with one or more N3Cs.

- Proposal 5: It is proposed to update the PDCP entity can be associated with one or more N3Cs in TS 38.323.

### 2.3.2 Multi-Path configuration, deactivation/activation

Addition and Modification of multiple IndirectPaths

- In the current mechanism, the IE N3C-IndirectPathAddChange in RRCReconfiguration message [5] indicates the N3C indirect path related configuration used by N3C remote UE in MP, and only one relay UE info is included.

- To support multiple indirect paths via N3C links, multiple configurations of relay UEs should be provided or released to the remote UE explicitly. In addition, the enhancements on the current IE n3c-IndirectPathAddChange should be avoid since it will introduce the impacts on legacy UEs. Therefore, the new IEs N3C-IndirectPathAddModList and N3C-IndirectPath-ToReleaseList can be introduced to provide or release the configuration of N3C relay UEs.

- Proposal 6: Introduce the new IEs N3C-IndirectPathAddModList and N3C-IndirectPath-ToReleaseList to provide or release the configuration of N3C relay UEs.

Indication of Primary Path among the multiple Indirect Path

As specified in TS 38.331 [5], the IE primaryPathOnIndirectPath in PDCP-Config indicates that the primary path is SL indirect path, or primary path is associated with the N3C indirect path, for DRB when MP is configured, where the format ENUMERATED {true} is used due to only one indirect path. However, for multiple indirect paths, the format ENUMERATED {true} will no longer be applicable, and which indirect path is primary path should be indicated explicitly. Thus, a new IE which indicates the primary path among the multiple indirect paths should be introduced.

Proposal 7: Introduce a new IE to indicate the primary path among the multiple indirect paths.

Indication of available Indirect Path(s) in case of Split Bearer

- As specified in TS 38.323 [3], the ul-DataSplitThreshold IE in PDCP-config [5] is used to decide which RLC entity or which path the PDCP PDU should be submitted to, when PDCP duplication is deactivated for the RB. If the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold, the PDCP PDU is submitted to either the Uu RLC entity or N3C, which is up to implementation.

- 5.2 Data transfer - 5.2.1 Transmit operation <omitted> - else (i.e., PDCP duplication is deactivated for the RB): - if the total amount of PDCP data volume, RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in either the primary RLC entity (when the MP primary path is the direct path) or the split secondary RLC entity on the direct path (when the MP primary path is the indirect path), and data volume pending for initial transmission in the N3C (if available) or mapped SL RLC entity associated with the SRAP entity, is equal to or larger than ul-DataSplitThreshold: - submit the PDCP PDU to either the Uu RLC entity (i.e., either the primary RLC entity or the split secondary RLC entity) or SRAP entity/N3C; - else: - if the MP primary path is the direct path: - submit the PDCP PDU to the primary RLC entity; - else: - submit the PDCP PDU to the SRAP entity or N3C; <omitted>

- If the multiple indirect paths via N3C are supported, we think there are the following potential solutions for data split:

    - - Option 1: gNB is required to configure the Uu Relay RLC channel and bearer mapping (which Uu Relay RLC channel the split DRB is mapped to) for the relay UE on the indirect path(s), with a new IE to indicate the available indirect path(s) for the data split case to remote UE. Reuse the legacy mechanism, i.e. if the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold, it is up to the UE implementation to determine which path a specific packet takes.

    - - Option 2: Introduce a flexible solution for data split, i.e. introduce multiple threshold for different path, and if the threshold is satisfied, the corresponding path will be used for data split.

    - For option 1, when there are multiple N3C links, gNB is required to configure the Uu Relay RLC channel and bearer mapping (which Uu Relay RLC channel the split DRB is mapped to) for the relay UE on the indirect path(s). In legacy systems, for data split, the splitSecondaryPath is used to indicate the split data path. In the MP case with more than two paths, it is proposed to introduce a new IE to indicate the available indirect path(s) for the data split case to remote UE. That is to say, when the split threshold is met, data can be transmitted through the primary path and the configured indirect path, and it is up to the UE implementation to determine which path a specific packet takes (meaning the proportion of data on the multiple paths is also implemented by the UE); when the split threshold is not met, data is transmitted only through the primary path.

    - For option 2, the spec impacts on PDCP and RRC are needed, i.e. the new threshold will be introduced in RRC signaling, and the mechanism on PDCP PDU transmission will be enhanced.

- Considering to minimum the spec impacts, we prefer to reuse the legacy mechanism with some extention for data split.

Proposal 8: For data split for multiple indirect paths via N3C, reuse the legacy mechanism with some extention, i.e. to introduce a new IE to indicate the available indirect path(s) for the data split when the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold.

- On the other hand, when the multiple indirect paths via N3Cs are configured to the remote UE for data split, whether and how to activate or deactivated should be discussed. As usual, the following solutions are considered for activation/deactivation.

- Option 1 (default way): The indirect path is activated by default when the configuration in RRC signaling is provided

- Option 2 (static way): Introduce separate threshold for different paths activation, i.e. the indirect path is deactivated by when the associated threshold is satisfied, the indirect path is activated, which is the same way as we proposed in P5 for data split

- Option 3 (dynamic way): Use MAC CE to activate/deactivate the path

- For option 1, there is no additional impacts.

- For option 2, the multiple threshold for different path should be introduced to activate/deactivate the path. However, the benefit is that option 2 can be used for both data split and path activation/deactivation.

- For option 3, a new MAC CE should be introduced to activate/deactivate the path via N3C, which has impacts on TS 38.321 [4].

- Based on the above analysis, we prefer to follow option 1 to avoid the additional impacts on specs.

- Proposal 9: The indirect path is activated by default when the configuration in RRC signaling is provided, i.e. the dynamic activation/deactivation mechanism for split bearer on multiple indirect paths is not supported in TEI19.

Duplication

- For PDCP duplication, as specified in TS 38.300 [2], if the split bearer is configured and the PDCP PDU duplication is activated on the PDCP entity, the duplicated PDCP PDUs are delivered via both direct path and indirect path.

- As specified in TS 38.331[5], duplicationState IE indicates the uplink PDCP duplication state. And for multi-path, the index for the indication is determined by ascending order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path. That means the third bit of duplicationState IE indicates indirect path.

- Correspondingly, the Duplication RLC Activation/Deactivation MAC CE in TS 38.321 [4] is used to indicate the duplication RLC activation/deactivation. And for Multi-path, RLC entity i is counted for secondary RLC entities in order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path.

- For multiple indirect paths via N3C, to minimum the impacts on specifications, we prefer to follow the current duplication handling framework, and introduce a new IE similar to extended duplicationState in RRC signaling to indicate PDCP duplication state of multiple indirect paths, and introduce a new MAC CE to activate/deactivate the PDCP duplication dynamically. Furthermore, we prefer the new MAC CE will be used for the scenario where both multi-path and DC are configured, to avoid the network sends two MAC CEs (the legacy Duplication RLC Activation/Deactivation MAC CE and the new MAC CE) to the UE in this scenario.

- Proposal 10: Introduce a new IE in RRC signaling to indicate PDCP duplication state of multiple indirect paths.

- Proposal 11: Introduce a new MAC CE to activate/deactivate the PDCP duplication dynamically for Multi-path and DC at the same time.

### 2.3.5 Path Failure

- As specified in TS 38.300 [2], when the L2 MP Remote UE using N3C indirect path detects N3C link failure and/or Uu link failure on the indirect path, the L2 MP Remote UE triggers path failure reporting through the direct path via a RRC message, if the direct path is not suspended. And for failure indirect path reporting, since there is only one indirect path in R18, the remote UE can report the available relay UE(s) info to the network, without indicating the indirect path failure.

- For multiple indirect paths via N3C, if the failure is detected in at least one indirect N3C path, the UE needs to report the failure info and indicates explicitly which path fails to the network. Therefore, a new IE should be introduced to indicate the failure info of one or more indirect N3C paths within FailureReportIndirectPath.

Proposal 12: A new IE should be introduced to indicate the failure info of one or more indirect N3C paths within FailureReportIndirectPath.

### 2.3.6 UE capability

- In TS 38.306 [6], the UE capability multipathRemoteUE-N3C-r18 is specified which indicates whether L2 multi-path remote UE operation using non-3GPP connection is supported by the UE.

To support the multiple indirect paths via N3C, we think a new capability should be introduced to differentiate with the legacy remote UE capability on N3C.

Proposal 13: A new remote UE capability to support multiple indirect paths via N3C should be introduced.

Based on the above proposals, we have prepared the following CRs to support multiple indirect paths via N3C:

- Draft CR to 38.300 [7]

- Draft CR to 38.323 [8]

- Draft CR to 38.331 [9]

- Draft CR to 38.306 [10]

- Draft CR to 38.321 [11]

Conclusion

According to the above discussion, the following observations and proposals are given:

Observation 1: In current RAN2 specifications on multi-path relay, only a single indirect path (PC5 or N3C) for UE aggregation is supported.

Observation 2: One indirect path in UE aggregation cannot meet the requirements of some use cases, e.g. large-scale live broadcasting, maritime fishing operations or offshore oil extraction platforms.

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

Proposal 2：It is proposed to only consider the scenario where multiple indirect paths under the same gNB, and CU-DU split structure is not considered in TEI19.

- Proposal 3: It is proposed to add the description of scenario in P1 and P2 to TS 38.300.

- Proposal 4: It is proposed to reuse the maximum number (i.e. 8) for multiple indirect paths via N3C.

- Proposal 5: It is proposed to update the PDCP entity can be associated with one or more N3Cs in TS 38.323.

- Proposal 6: Introduce the new IEs N3C-IndirectPathAddModList and N3C-IndirectPath-ToReleaseList to provide or release the configuration of N3C relay UEs.

Proposal 7: Introduce a new IE to indicate the primary path among the multiple indirect paths.

Proposal 8: For data split for multiple indirect paths via N3C, reuse the legacy mechanism with some extention, i.e. to introduce a new IE to indicate the available indirect path(s) for the data split when the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold.

- Proposal 9: The indirect path is activated by default when the configuration in RRC signaling is provided, i.e. the dynamic activation/deactivation mechanism for split bearer on multiple indirect paths is not supported in TEI19.

- Proposal 10: Introduce a new IE in RRC signaling to indicate PDCP duplication state of multiple indirect paths.

- Proposal 11: Introduce a new MAC CE to activate/deactivate the PDCP duplication dynamically for Multi-path and DC at the same time.

Proposal 12: A new IE should be introduced to indicate the failure info of one or more indirect N3C paths within FailureReportIndirectPath.

Proposal 13: A new remote UE capability to support multiple indirect paths via N3C should be introduced.

Reference

- RP-223501 Revised WID on NR sidelink relay enhancements

- TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2

- TS 38.323 NR; Packet Data Convergence Protocol (PDCP) specification

- TS 38.321 NR; Medium Access Control (MAC) protocol specification

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TS 38.306 NR; User Equipment (UE) radio access capabilities

- R2-2501054	Corrections to TS 38.300 on multi-path relay enhancement

- R2-2501055	Corrections to TS 38.321 on multi-path relay enhancement

- R2-2501056	Corrections to TS 38.323 on multi-path relay enhancement

- R2-2501057	Corrections to TS 38.331 on multi-path relay enhancement

- R2-2501058	Corrections to TS 38.306 on multi-path relay enhancement