3GPP TSG-RAN WG2 Meeting #129											R2-2501086

Athens, Greece, Feb.17th–21st, 2025

Agenda item:		8.5.2

Title: 	Remaining issues of on demand SSB SCell operation

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining open issues of on demand SSB SCell.

- Discussion

  - Configuration of on-demand SSB

For the existing SCell Addition/Modification and activation/deactivation, SCell can be added and modified via SCell addition/modification with the initial state set to set as “deactivated”. When a SCell is added but deactivated, UE will still measure SSB, if configured, but would not monitor PDCCH.

Figure 1. Understanding on case#1

As we agreed last meeting that For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated, the SCell addition/modification with the initial state set to “deactivated” cannot be directly apply for on demand SSB case#1.

Observation 1: For case#1, the SCell addition/modification can be reused with additional indication to inform UE that there is no need to perform SSB measurements and PDCCH monitoring.

Figure 2: Understanding on case#2

In the reply LS [1], RAN1 has explained the relation between always on SSB and OD-SSB for case#2:

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?):

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?):

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst.

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB.

Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?):

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?):

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Based on the feedback from RAN1, we understand the Always on SSB can be SSB with larger periodicity in the same frequency for energy saving purpose while the SSB after on demand triggering would be more frequently transmitted.

In this case, if UE does not measure the always on SSB, the case#2 would be the same as case#1 and NW would not be aware of the radio quality of the SCell and there would be no gain compared to case#1 while more power consumption is required for the always on SSB transmission.

However, if UE measure the always on SSB before on demand SSB triggering, NW can get the measurement report in advance and decide whether to trigger on demand SSB transmission and activate the SCell.

Thus, we see benefits if UE measure the always on SSB before on demand SSB transmission is activated for case#2.

Proposal 1: For case#2, UE should perform measurements on the always on SSB before on demand SSB transmission is activated.

In case#2, the SCell addition/modification with the initial state set to “deactivated” can be applied, with which UE would measure the always on SSB but not monitor the PDCCH. And the SSB transmission update is expected upon on demand SSB transmission triggering and such update should be indicated to UE.

Then two options can be considered to provide the configuration for on demand SSB for case#1 and case#2:

- Option 1: Reuse the SCell addition and modification

  - For case#1, the OD-SSB SCell is added or modified as a SCell with additional indication to UE so that UE will only perform measurements on SSB for the corresponding SCell upon on demand SSB activation.

  - For case#2, the OD-SSB SCell is added or modified as a SCell with always-on SSB and the initial state set to “deactivated”.

    - Alternative 1: The OD-SSB configuration, e.g. the periodicity and time location, is also provided and will be applied upon on demand SSB activation.

    - Alternative 2: The OD-SSB configuration e.g. the periodicity and time location, is provided via on demand SSB activation MAC CE.

- Option 2: Introduce on demand SSB SCell configuration

From our perspective, we understand the OD-SSB SCell configuration would be quite similar as that for SCell addition and modification thus there is no need to have such duplication.

Proposal 2: Reuse the SCell addition and modification for OD-SSB SCell configuration:

  - For case#1, the OD-SSB SCell is added or modified as a SCell with additional indication to UE so that UE will only perform measurements on SSB for the corresponding SCell upon on demand SSB activation.

  - For case#2, the OD-SSB SCell is added or modified as a SCell with always-on SSB and the initial state set to “deactivated”.

    - Alternative 1: The OD-SSB configuration, e.g. the periodicity and time location, is also provided and will be applied upon on demand SSB activation.

    - Alternative 2: The OD-SSB configuration e.g. the periodicity and time location, is provided via on demand SSB activation MAC CE.

  - L3 Measurement

With the above feedback from RAN1, we can focus on the case that the on demand SSB is the same as the frequency of always on SSB from RAN2’s perspective.

Case#1

For case#1, as we mentioned above, UE need to be aware that it does not need to perform measurements on SSB for the SSB less SCell.

Thus we understand that the measurement configuration for the SSB less SCell can be configured but with extra indication to UE that the measurements are required only after on demand SSB activation.

Proposal 3a: For case#1, the measurement configuration for the SSB less SCell will be provided but with extra indication to UE that the measurements are required only after on demand SSB activation.

Case#2

As shown in the following figure, four options can be considered for the handling of L3 measurements for case#2:

- Option 1: UE Performs L3 measurement based on always-on SSB, no L3 measurement enhancement for case#2

- Option 2: UE measures OD-SSB (based on OD-SSB pattern) for serving cell, and legacy SMTC for neighbor cells

- Option 3: UE measures OD-SSB (based on OD-SSB pattern and always-on SSB in legacy SMTC) for serving cell, and legacy SMTC for neighbor cells

- Option 4: Configure specific SMTC for OD-SSB, UE measures OD-SSB in the specific SMTC and always on-SSB in legacy SMTC for serving cell, and legacy SMTC for neighbor cells

- Option 5: Configure specific SMTC for OD-SSB, UE measures OD-SSB in the specific SMTC and legacy SMTC for neighbor cells

Figure 3: L3 measurements for case#2

Regarding the questions from RAN4, it is highly related to the solution we select while option 2/3/4 would all have RAN4 impact while option 1 would be the simplest way to go. For option 2 and 3, RAN4 has to define L3 measurement requirement based on SSB pattern. For option 4, additional SMTC would be introduced and RAN4 needs to consider the impact.

Proposal 3b: For case#2, RAN2 to discuss the following options for L3 measurement handling and inform RAN1/4:

- Option 1: UE Performs L3 measurement based on always-on SSB, no L3 measurement enhancement for case#2

- Option 2: UE measures OD-SSB (based on OD-SSB pattern) for serving cell, and legacy SMTC for neighbor cells

- Option 3: UE measures OD-SSB (based on OD-SSB pattern and always-on SSB in legacy SMTC) for serving cell, and legacy SMTC for neighbor cells

- Option 4: Configure specific SMTC for OD-SSB, UE measures OD-SSB in the specific SMTC and always on-SSB in legacy SMTC for serving cell, and legacy SMTC for neighbor cells

- Option 5: Configure specific SMTC for OD-SSB, UE measures OD-SSB in the specific SMTC and legacy SMTC for neighbor cells

  - On-demand SSB procedure

The on demand SSB procedure can be triggered when UL or DL traffic arrives and more activate SCells are needed. Thus we understand both UE and NW trigger can be considered.

Proposal 4: The on demand SSB procedure can be triggered by NW or UE.

    - On demand SSB procedure triggered by NW

Issue1: On demand SSB activation MAC CE design

As we agreed last meeting that New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE, we understand a MAC-CE as follows can be introduced with each bit indicating the transmission of SSB for each SCell.

Figure 4: An example of on demand SSB activation MAC CE of four octets

For case#1 with Scenario#2, the above example of SSB transmission indication MAC CE would be sufficient and UE would start to measure the SSB if the bit for the corresponding SCell is set to 1.

Proposal 5a: For case#1, one bit per SCell should be introduced in the MAC CE to indicate the on demand SSB activation. UE starts to measure SSB upon reception of the on demand SSB activation MAC CE for a SCell if the corresponding bit is set to 1.

For case#2 with Scenario#2, two options can be considered:

- Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

Option 1: Two sets of SSB configuration via RRC with 1 bit per SCell in MAC CE to switch

- Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Proposal 5b: For case#2, two options can be considered for the SSB transmission update upon on demand SSB activation:

- Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

- Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Issue 2: On demand SSB deactivation

There has been discussion on how to indicate the transmission of on demand SSB while we have not mentioned whether we can deactivate the on demand SSB transmission. Per our understanding, the deactivation via MAC CE should be also supported.

For case#1, UE would stop measurements on the SSB upon reception of the deactivation. For case#2, UE is informed of the SSB transmission update, e.g. periodicity update, based on the deactivation indication.

Proposal 6: On demand SSB deactivation via MAC CE should be supported:

- For case#1, UE stop SSB measurements upon reception of the deactivation.

- For case#2, UE is informed of the SSB transmission update, e.g. the periodicity update, upon reception of the deactivation.

    - On demand SSB procedure triggered by UE

From RAN2’s perspective, UE can send on demand SSB request to network via UE assistance information if we go for pure RAN2 solution without much impact in RAN1. Upon reception of the request, it is still up to network to decide whether to activate the on demand SSB transmission while the activation/deactivation procedure for NW triggered procedure can be reused.

Proposal 7: UE request the on demand SSB transmission via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation procedure for NW triggered procedure can be reused.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Configuration of on-demand SSB

Observation 1: For case#1, the SCell addition/modification can be reused with additional indication to inform UE that there is no need to perform SSB measurements and PDCCH monitoring.

Proposal 1: For case#2, UE should perform measurements on the always on SSB before on demand SSB transmission is activated.

Proposal 2: Reuse the SCell addition and modification for OD-SSB SCell configuration:

  - For case#1, the OD-SSB SCell is added or modified as a SCell with additional indication to UE so that UE will only perform measurements on SSB for the corresponding SCell upon on demand SSB activation.

  - For case#2, the OD-SSB SCell is added or modified as a SCell with always-on SSB and the initial state set to “deactivated”.

    - Alternative 1: The OD-SSB configuration, e.g. the periodicity and time location, is also provided and will be applied upon on demand SSB activation.

    - Alternative 2: The OD-SSB configuration e.g. the periodicity and time location, is provided via on demand SSB activation MAC CE.

L3 measurement

Proposal 3a: For case#1, the measurement configuration for the SSB less SCell will be provided but with extra indication to UE that the measurements are required only after on demand SSB activation.

Proposal 3b: For case#2, RAN2 to discuss the following options for L3 measurement handling and inform RAN1/4:

- Option 1: UE Performs L3 measurement based on always-on SSB, no L3 measurement enhancement for case#2

- Option 2: UE measures OD-SSB (based on OD-SSB pattern) for serving cell, and legacy SMTC for neighbor cells

- Option 3: UE measures OD-SSB (based on OD-SSB pattern and always-on SSB in legacy SMTC) for serving cell, and legacy SMTC for neighbor cells

- Option 4: Configure specific SMTC for OD-SSB, UE measures OD-SSB in the specific SMTC and always on-SSB in legacy SMTC for serving cell, and legacy SMTC for neighbor cells

- Option 5: Configure specific SMTC for OD-SSB, UE measures OD-SSB in the specific SMTC and legacy SMTC for neighbor cells

On-demand SSB procedure

Proposal 4: The on demand SSB procedure can be triggered by NW or UE.

Proposal 5a: For case#1, one bit per SCell should be introduced in the MAC CE to indicate the on demand SSB activation. UE starts to measure SSB upon reception of the on demand SSB activation MAC CE for a SCell if the corresponding bit is set to 1.

Proposal 5b: For case#2, two options can be considered for the SSB transmission update upon on demand SSB activation:

- Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

- Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Proposal 6: On demand SSB deactivation via MAC CE should be supported:

- For case#1, UE stop SSB measurements upon reception of the deactivation.

- For case#2, UE is informed of the SSB transmission update, e.g. the periodicity update, upon reception of the deactivation.

Proposal 7: UE request the on demand SSB transmission via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation procedure for NW triggered procedure can be reused.

- Reference

- R2-2500014 Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell (R1-2410917; contact: Ericsson)