3GPP TSG-RAN WG2 Meeting #129											R2-2501088

Athens, Greece, Feb.17th–21st, 2025

Agenda item:		8.5.4

Title: 	Common signal/channel adaption

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the further details of common signal/channel adaption.

- Discussion

  - Paging occasion adaptation

The following agreements have been made in the recent meeting on paging adaptation [1][2]:

RAN2#127bis:

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signalling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

RAN2#128

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

And we understand the following issues are still open and require further agreements.

Issue 1: Value range for N and Ns

We understand the following parameters will be impacted by paging occasion adaption: nAndPagingFrameOffset, ns, firstPDCCH-MonitoringOccasionOfPO. We have made agreements for Ns=8 and N= T/32 with FFS left for other values.

The main concern from companies is the signaling overhead introduced if we introduce more values while the adaption in firstPDCCH-MonitoringOccasionOfPO would contribute most to the signaling overhead.

The value range of firstPDCCH-MonitoringOccasionOfPO depends on SCS and N (number of PFs in paging cycle T):

The existing values range and potential new values with newly introduced N are summarized as follows:

| N | SCS/KHz | The maximum number of PDCCH monitoring occasions for a PF | N | SCS/KHz | The maximum number of PDCCH monitoring occasions for a PF | N | SCS/KHz | The maximum number of PDCCH monitoring occasions for a PF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | 15 | 140 | T/2 | 15 | 280 | T/4 | 15 | 560 |
|  | 30 | 280 |  | 30 | 560 |  | 30 | 1120 |
|  | 60 | 560 |  | 60 | 1120 |  | 60 | 2240 |
|  | 120 | 1120 |  | 120 | 2240 |  | 120 | 4480 |
|  | 480 | 4480 |  | 480 | 8960 |  | 480 | 17920 |
| T/8 | 15 | 1120 | T/16 | 15 | 2240 | T/32 | 15 | 4480 |
|  | 30 | 2240 |  | 30 | 4480 |  | 30 | 8960 |
|  | 60 | 4480 |  | 60 | 8960 |  | 60 | 17920 |
|  | 120 | 8960 |  | 120 | 17920 |  | 120 | 35840 |
|  | 480 | 35840 |  | 480 | 71680 |  | 480 | 143360 |
| T/64 | 15 | 8960 | T/128 | 15 | 17920 | T/256 | 15 | 35840 |
|  | 30 | 17920 |  | 30 | 35840 |  | 30 | 71680 |
|  | 60 | 35840 |  | 60 | 71680 |  | 60 | 143360 |
|  | 120 | 71680 |  | 120 | 143360 |  | 120 | 286720 |
|  | 480 | 286720 |  | 480 | 573440 |  | 480 | 1146880 |

If we introduce N with new value T/32, the following new parameters are needed for firstPDCCH-MonitoringOccasionOfPO:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359)

While if we introduce N with new value T/32 and T/64, the following new parameters would be needed:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT-SCS15KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT-SCS30KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT-SCS60KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS120KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359),

sCS480KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..286719)

With N=T/64 and Ns=16 introduced, there will be 144 more bits required in addition to N=T/32 and Ns=8.

Issue 2: PEI handling

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

For the association between PEI and PO, it is identified by the number of POs (po-NumPerPEI) following the PEI while the POs can span over one or more PFs.

The number of POs associated to PEI need to balance the PEI overhead per PO and the paging latency/ UE power consumption and is now decided with the maximum value of 8. Also the maximum value of POs in a PF is currently 4 thus it is allowed to map one PEI to POnumPerPEI PO(s) in one or two PF(s).

With the newly introduced Ns=8 and N=T/32, we will support at most 8 POs in a PF. If we keep the existing value range for POnumPerPEI, no additional delay will be introduced. If we introduce Ns=16 and N=T/64, we would need to extend the value range for POnumPerPEI as all the POs in one PF cannot be associated to the same PEI.

Observation 1: Introducing Ns with value 16 and N with value T/64, T/128 or T/256 would require much more signaling overhead and extension of the POnumPerPEI value range.

Proposal 1: Introduce value for Ns=8 and N= T/32. No other values will be considered in this release.

Proposal 2: No extension to the value range of POnumPerPEI. R19 NES UEs will monitor the same PEI with legacy UEs and it is up to the network to configure the association between PEI and POs.

  - RACH adaptation in time domain

Regarding RACH adaptation in time domain, RAN1 [2] has agreed to introduce additional PRACH resources for NES-capable UEs in additional to PRACH resources for legacy UEs and use DCI to indicate the adaptation for additional PRACH resources for UE in idle/inactive and connected.

From RAN2’s perspective, we understand the following issues need to be considered:

- Issue 1: CBRA, CFRA or both?

- Issue 2: 2-step RACH, 4-step RACH or both?

- Issue 3: Whether to support RACH partitioning? Or exclude some features?

In our understanding, the additional PRACH resources is mainly used to provide more RACH resources when traffic arrives so that the transmission delay can be reduced. Thus, we understand the additional PRACH resources should offer similar functions as the baseline RACH resources so that all kinds of traffic with differentiated requirements can be supported.

Thus, we understand both CBRA and CFRA, 2-step RACH and 4-step RACH can be supported in the additional PRACH resource and it is up to network to configure.

Proposal 3a: Both CBRA and CFRA are supported for PRACH adaption in time domain.

Proposal 3b: Both 2-step RACH and 4-step RACH are supported for PRACH adaption in time domain.

Regarding the RACH partitioning, the following features are considered: RedCap, SDT, slicing and CE. We understand all these features can be supported by an NES cell thus would like to support all of them.

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                    OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                                    OPTIONAL,  -- Need R

eRedCap-r18                ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                    OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

Proposal 4: RACH partitioning with all the features, i.e. RedCap, SDT, Slicing, and CE, and feature combinations, are supported for PRACH adaption in time domain.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Observation 1: Introducing Ns with value 16 and N with value T/64, T/128 or T/256 would require much more signaling overhead and extension of the POnumPerPEI value range.

Proposal 1: Introduce value for Ns=8 and N= T/32. No other values will be considered in this release.

Proposal 2: No extension to the value range of POnumPerPEI. R19 NES UEs will monitor the same PEI with legacy UEs and it is up to the network to configure the association between PEI and POs.

Proposal 3a: Both CBRA and CFRA are supported for PRACH adaption in time domain.

Proposal 3b: Both 2-step RACH and 4-step RACH are supported for PRACH adaption in time domain.

Proposal 4: RACH partitioning with all the features, i.e. RedCap, SDT, Slicing, and CE, and feature combinations, are supported for PRACH adaption in time domain.

- Reference

- Draft_RAN2_127b_Meeting_Report_v3

- Draft_RAN2_128_Meeting_Report_v1