3GPP TSG-RAN WG2 Meeting #129										R2-2501091

Athens, Greece, Feb. 17th – 21st, 2025                                     Revision of R2-2410168

Agenda item:		8.4.4

Title: 	Procedures for LP-WUS in RRC_CONNECTED

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure of LP-WUS for RRC_CONNECTED state and give our proposals.

- Discussion

2.1 Activation/deactivation for LP-WUS monitoring in RRC_CONNECTED state

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. Considering that the coverage of LP-WUS may be less than or equal to the cell coverage, even the LP-WUS related configuration is provided, the LP-WUS may not be monitored, e.g. when the UE is at the cell edge. The LP-WUS monitoring activation/deactivation should be considered.

Usually, there are two solutions for LP-WUS monitoring activation/deactivation:

- Solution 1: similar as RRC_IDLE/RRC_INACTIVE state, a cell edge condition (e.g. RSRP threshold) is configured to UE, and UE monitors LP-WUS in RRC_CONNECTED state when it is not at the cell edge. For this solution, Network cannot know whether the UE is monitoring the LP-WUS and should always send LP-WUS, whichever the UE monitors it or not. In this case, for Option 1-1, Network couldn’t know whether UE starts the drx-onDurationTimer.

- Solution 2: Network activate or deactivate the LP-WUS explicitly, e.g. with RRC message, DCI or MAC CE. For this solution, UE need monitor LP-WUS only when it is activated by network, and network can activate or deactivate the LP-WUS based on the CQI info or UE assistance information reported by UE. So that network can determine whether and when to activate or deactivate the LP-WUS monitoring in RRC_CONNECTED state, LP-WUS monitoring entry and exit condition (e.g. RSRP and optional RSRQ threshold) should be configured to UE(e.g. by RRCReconfiguration message), and UE can report LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message .

To keep alignment between UE LP-WUS monitoring and gNB LP-WUS transmission, solution 2 is preferred. For light signaling load and not impact RAN1 on DCI design, MAC CE is suggested to be used for network to activate or deactivate the LP-WUS explicitly.

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

## 2.2 LP-WUS monitoring schemes with C-DRX configured

RAN1 has identified the following 3 options for the LP-WUS monitoring with C-DRX configured, and has made the following working assumptions for Option 1-1 and Option 1-2:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

The working assumption in RAN1 aligns with RAN2 agreement, e.g.

RAN2 #126 Agreement: - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 #127bis Agreement: - For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

Proposal 2: Confirm the RAN1 working assumption that both Option 1-1 and Option 1-2 are supported:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

In the current specification, secondary DRX Group can be configured for UE power saving when both FR1 and FR2 cells are configured in one cell group, in which case the UE power consumption can be reduced if a separate drx-InactivityTimer and drx-onDurationTimer in secondary DRX Group is configured for the FR2 cells enabling FR2 to go to sleep more quickly. Since both secondary DRX Group and LP-WUS are used for UE power saving, LP-WUS is used to trigger the UE enters into DRX active time, and DRX active time is started per DRX group, it should support to configure LP-WUS and secondary DRX Group simultaneously for UE power saving.

Proposal 3a: The LP-WUS can be configured with secondary DRX group simultaneously.

When LP-WUS is configured with secondary DRX Group simultaneously, how to correlate LP-WUS with DRX group should be considered. One of the following options can be considered:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered.

Since Option 2 needs less signaling overhead than Option 1, but Option2 depends on RAN1 design for the LP_WUS.

Proposal 3b: When LP-WUS is configured with secondary DRX Group simultaneously, down select from the following options with an LS to RAN1 to inform our decision:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered

In RAN1#118bis meeting, it has agreed that LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode, as follows:

Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Based on the current specification [6], “In MR-DC, separate DRX configurations are provided for MCG and SCG. A secondary DRX group can be configured in MR-DC for a cell group that includes cells in different Frequency Ranges”. Since LP-WUS is used to trigger UE to enter DRX active time, LP-WUS should be configured for MCG and LCG separately.

Proposal 4a: In NR-DC, LP-WUS is configured for MCG and LCG separately.

Since C-DRX can be configured for MCG in NE-DC case and can be configured for SCG in EN-DC case, and LP-WUS is used to trigger UE to enter DRX active time, LP-WUS can be configured for MCG in NE-DC case, and LP-WUS can be configured for SCG in EN-DC case.

Proposal 4b: LP-WUS can be configured for MCG in NE-DC case, i.e. the MN is a gNB, and LP-WUS can be configured for SCG in EN-DC case, i.e. the SN is a gNB.

Considering that legacy, in case DCP is monitored but not detected, the legacy drx-onDurationTimer is not started and UE does not enter into active time. Same mechanism can be reused for LP-WUS in RRC_CONNECTED, e.g. If LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started and the UE does not enter into active time.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. drx-onDurationTimer-r19) is not started for option 1-2.

For Option 1-2, as ‘UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)’ in RAN1, UE can determine the LP-WUS monitoring occasion using the similar formula as that to start legacy drx-onDurationTimer. Considering that LP-WUS monitoring consumes much less UE power than PDCCH monitoring, and smaller LP-WUS cycle can decrease service delay, one LP-WUS cycle with smaller value can be configured, which does not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6: For Option 1-2, RAN2 confirms that one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which does not differentiate long LP-WUS cycle and short LP-WUS cycle.

In Rel-18, non-integer C-DRX cycle is defined to match the non-integer traffic periodicity (e.g. XR periodicity), considering the low power consumption, small granularity of LP-WUS cycle (e.g. value of 1ms) can be used to match the non-integer C-DRX periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. value of 1ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

In RAN2#117 meeting, it has been agreed that “For Option 1-2,The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring.”, that means that a timeOffset should be defined for the GAP between LP-WUS monitoring and the start of newly defined timer (e.g. drx-onDurationTimer-r19). Since legacy drx-startOffset is defined to determine the PDCCH monitoring start occasion, drx-startOffset - timeOffset can be used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion.

Proposal 7:  For Option 1-2, RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. the new timer triggered by LP-WUS).

Similar as the drx-onDurationTimer start occasion, For Option 1-2, the LP-WUS monitoring occasion can be determined with the LP-WUS cycle and LP-WUS start offset (e.g. drx-startOffset - timeOffset) as follows: if LP-WUS is configured and activated, the UE monitors the LP-WUS at the SFN and subframe determined by:

[(SFN × 10) + subframe number - ( drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) =0:

Proposal 8: For Option 1-2, the LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

About the Option 1-3 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state (LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring): This option can be used to decrease the UE power when intermittent DL data burst arrives at gNB, e.g. in the C-DRX active time, only if LP-WUS is detected, UE monitors PDCCH. Since LP-WUS consumes little UE power, a small C-DRX cycle, large drx-OndurationTimer and/or large drx-inactiveTimer can be configured to keep the UE always in the DRX active time. This solution can also reduce the UE power consumption, but the following issue should be considered: Whether the LP-WUS will be monitored when the HARQ timer is running. If LP-WUS will be monitored when the HARQ timer is running, the UE performance should be evaluated, e.g. whether UE can monitor LP-WUS and perform main radio Tx/Rx simultaneously; if LP-WUS will be monitored only after the HARQ timer expires, the LP-WUS monitoring may lead PDCCH monitoring delay, and further lead traffic transmission delay, which depends on the RAN1 design for LP-WUS, e.g. the time GAP between LP-WUS monitoring and PDCCH monitoring.

Observation 1: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 9: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

## 2.3 About RAN1 LS on LP-WUS operation in CONNECTED mode

In the RAN1 LS [9], the following information is included:

Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution.

For Option 1-1: UE will use the DRX cycle to determine the LP-WUS monitoring occasion, e.g. with a timeoffset before the drx-onDurationTimer start occsion, the existing DRX Command MAC CE and Long DRX Command MAC CE can be used as legacy, e.g. use DRX Command MAC CE and/or Long DRX Command MAC CE to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, and to switch between Long DRX cycle and short DRX cycle (if configured).

For Option 1-2: UE does not use the DRX cycle to determine the LP-WUS monitoring occasion, but uses a LP-WUS cycle to determine the LP-WUS monitoring occasion. Based on the proposal 6 above, only one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which does not differentiate long LP-WUS cycle and short LP-WUS cycle. The existing DRX Command MAC CE or Long DRX Command MAC CE can be used to stop the new defined onDuration timer and drx-InactivityTimer for the DRX group.

Based on this understanding, RAN2 can reply the RAN1 LS to indicate the RAN2 understanding that:

- For Option 1-1: When UE is not in C-DRX active time, UE only monitors LP-WUS with a timeoffset before the drx-onDurationTimer start occsion. The existing DRX Command MAC CE and Long DRX Command MAC CE can be used as legacy, e.g. use DRX Command MAC CE and/or Long DRX Command MAC CE not only to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, but also to switch  the DRX cycle between Long DRX cycle and short DRX cycle(if configured).

- For Option 1-2: When UE is not in C-DRX active time, UE uses a LP-WUS cycle to determine the LP-WUS monitoring occasion. Only one LP-WUS cycle will be defined, which does not differentiate long LP-WUS cycle and short LP-WUS cycle. Either the existing DRX Command MAC CE or Long DRX Command MAC CE will be used to stop the new defined onDuration timer and drx-InactivityTimer for the DRX group.

Proposal 10: RAN2 can reply the RAN1 LS to indicate the RAN2 understanding that:

- For Option 1-1: When UE is not in C-DRX active time, UE only monitors LP-WUS with a timeoffset before the drx-onDurationTimer start occsion. The existing DRX Command MAC CE and Long DRX Command MAC CE can be used as legacy, e.g. use DRX Command MAC CE and/or Long DRX Command MAC CE not only to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, but also to switch  the DRX cycle between Long DRX cycle and short DRX cycle(if configured).

- For Option 1-2: When UE is not in C-DRX active time, UE uses a LP-WUS cycle to determine the LP-WUS monitoring occasion. Only one LP-WUS cycle will be defined, which does not differentiate long LP-WUS cycle and short LP-WUS cycle. Either the existing DRX Command MAC CE or Long DRX Command MAC CE will be used only to stop the new defined onDuration timer and drx-InactivityTimer for the DRX group.

- Conclusion

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

Proposal 2: Confirm the RAN1 working assumption that both Option 1-1 and Option 1-2 are supported:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

Proposal 3a: The LP-WUS can be configured with secondary DRX group simultaneously.

Proposal 3b: When LP-WUS is configured with secondary DRX Group simultaneously, down select from the following options with an LS to RAN1 to inform our decision:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered

Proposal 4a: In NR-DC, LP-WUS is configured for MCG and LCG separately.

Proposal 4b: LP-WUS can be configured for MCG in NE-DC case, i.e. the MN is a gNB, and LP-WUS can be configured for SCG in EN-DC case, i.e. the SN is a gNB.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. drx-onDurationTimer-r19) is not started for option 1-2.

Proposal 6: For Option 1-2, RAN2 confirms that one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which does not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. value of 1ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 7: For Option 1-2, RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. the new timer triggered by LPWUS).

Proposal 8: For Option 1-2, The LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

Observation 1: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 9: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

Proposal 10: RAN2 can reply the RAN1 LS to indicate the RAN2 understanding that:

For Option 1-1: When UE is not in C-DRX active time, UE only monitors LP-WUS with a timeoffset before the drx-onDurationTimer start occsion. The existing DRX Command MAC CE and Long DRX Command MAC CE can be used as legacy, e.g. use DRX Command MAC CE and/or Long DRX Command MAC CE not only to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, but also to switch  the DRX cycle between Long DRX cycle and short DRX cycle(if configured).

For Option 1-2: When UE is not in C-DRX active time, UE uses a LP-WUS cycle to determine the LP-WUS monitoring occasion. Only one LP-WUS cycle will be defined, which does not differentiate long LP-WUS cycle and short LP-WUS cycle. Either the existing DRX Command MAC CE or Long DRX Command MAC CE will be used only to stop the new defined onDuration timer and drx-InactivityTimer for the DRX group.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, RAN#103, March, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- 3GPP TS 37.340 V18.3.0 (2024-09), Multi-connectivity; Overall Description; Stage 2

- Draft_RAN2_128_Meeting_Report_v1

- Draft_Minutes_report_RAN1#119_v030

- R1-2410909 LS on LP-WUS operation in CONNECTED mode