3GPP TSG-RAN2 Meeting #129	R2-2501093

Athens, Greece, 17 - 21 February 2025 	Revision of R2-2410085

Agenda Item:	8.4.2 Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS in Idle and Inactive

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS operation in RRC_IDLE and RRC_INACTIVE is discussed.

For the RAN1 and RAN2 agreements in previous meetings the reader is referred to the Annex.

# Further discussion on LP-WUS in Idle and Inactive

## SA2 LS

SA2 replied to the RAN2 LS about LP-WUS subgrouping [4] and SA2 captured for LP-WUS similar corrections as for CN-based subgrouping with PEI in CRs to 23.501 and 23.502 [5,6].

### LP-WUS terminology

SA2 commented that the Wake-Up Signal is not a low power signal, and therefore the abbreviation LP-WUS could be misunderstood. SA2 is correct in their understanding that WUS is not a low power signal.

To avoid changes and impact on other working groups. And to be able to indicate the “signal” and “receiver” separately it is proposed to use the following abbreviations:

- LP-WUS:	Low Power receiver Wake Up Signal

- LR:		Low power Receiver

- MR:		Main Receiver

The following abbreviations are adopted:

The following abbreviations are adopted: Low Power receiver Wake Up Signal (LP-WUS), Low power Receiver (LR) and Main Receiver (MR).

### LP-WUS and emergency PDU session

How to handle UE-ID based subgrouping during an emergency PDU session is FFS in RAN2:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In SA2 the use of LP-WUS during an emergency PDU session is also FFS (see section 5.4.12a.2 in 23.501):

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

Paging delay

With LP-WUS the paging delay is increased significantly due to the large time offset between LP-WUS and the associated PO, i.e. high probability that the paging message is delayed with one DRX cycle:

During the study phase (TR 38.869 [3]) a time offset of 400 ms between LP-WUS and PO was used, and 800 ms was mentioned as an alternative. In RRC_IDLE typically a paging DRX cycle of 1.28 sec or 640 ms can be observed in the field, and in RRC_INACTIVE a short DRX cycle down to 320 ms can be configured to improve the latency. Based on these values the increased paging delay with LP-WUS can be estimated and compared with PEI:

| DRX cycle | Offset | Delay legacy | Delay LP-WUS | % increase | ms increase |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1280 | 40 | 640 | 660 | 3% | 20 | <= PEI offset |
| 1280 | 300 | 640 | 790 | 23% | 150 |  |
| 1280 | 600 | 640 | 940 | 47% | 300 |  |
| 640 | 300 | 320 | 470 | 47% | 150 |  |
| 640 | 600 | 320 | 620 | 94% | 300 |  |

The time offset may even exceed the DRX cycle, e.g. DRX cycle is 640 ms and time offset is 800 ms. Which means that the paging message is always delayed due to the time offset.

When the UE has an emergency PDU session then eDRX is not used because eDRX may delay an (emergency) callback. Furthermore CN-assigned PEI is also not allowed when the UE has an emergency PDU session (see 24.501 section 5.3.25) for latency reasons:

When the UE has an emergency PDU session LP-WUS is not used.

If proposal 2 is agreeable, then RAN2 should inform CT1, RAN3 and SA2. These WGs can then make the appropriate changes in their specification.

Similar as with PEI the UE is expected to use LP-WUS with UE-ID based subgrouping when it is configured in the cell and CN-assigned subgrouping is not used. The signalling for the emergency PDU session is handled in NAS. In 38.304 it can be specified that the UE does not use UE-ID based subgrouping when it has an emergency PDU session:

The UE shall not use UE_ID based subgrouping when NAS indicates that the UE has an emergency PDU session.

SA2 status

In the context of eDRX in RRC_INACTIVE RAN2 asked SA2 and RAN3 whether there can be cases where the UE has an emergency PDU session, but the gNB is not aware of this, i.e. the gNB may release the UE to RRC_INACTIVE with eDRX configured [7].

SA2 did not reach a conclusion about this, i.e. some companies think that the NW implementation can ensure that when the UE has an emergency PDU session that the UE is not released and kept in RRC_CONNECTED. Other companies think that CN should inform RAN implicitly when the UE has an emergency PDU session e.g. by omitting the eDRX configuration in RRC_IDLE (Tedrx, CN) in the Core Network Assistance Information for RRC INACTIVE in message. Note that the UE can only be configured with eDRX in RRC_INACTIVE when eDRX in RRC_IDLE is configured (see conditional presence RANPaging in 38.331).

### LP-WUS in lastUsedCellOnly

With PEI it is possible to limit the use of PEI to the cell where the UE was last in RRC_CONNECTED, i.e. when lastUsedCellOnly flag is set in PEI-Config in SIB1. In such case the UE only monitors PEI when it is (still) camped on the cell where it was released from RRC_CONNECTED the last time. Unless noLastCellUpdate was received in the last RRCRelease message, i.e. the NW can use this flag in the release when the last visited cell info is not sent/updated in the CN e.g. when there was SDT without UE context relocation. When lastUsedCellOnly is configured then the PEI transmissions are limited to the first paging attempt of the NW, i.e. PEI is not transmitted during paging escalation when there is paging to all the cells of the registration / RNA area:

Similar as with Rel-17 PEI the LP-WUS transmissions can be limited to lastUsedCellOnly.

In case the LP-WUS transmissions are not limited to the last used cell, then further power saving gain is provided to the UE at the expense of additional NW resources. Even further power saving gains can be achieved by introducing a “mobility bit”, which can be used to prevent waking up “stationary” UEs when the NW is paging for “mobile” UEs:

- The gNB sets the mobility bit when it is paging a UE that has moved, i.e. when the UE is paged in another cell than it was last released.

- When a UE is monitoring paging in the same cell as it was released and the mobility bit is set in LP-WUS then the UE does not wake-up.

This approach has the benefit that UEs that have moved, can also use LP-WUS, but UEs that have not moved can avoid waking up when the NW is paging for UEs that have moved:

LP-WUS indicates when there is paging for a UE that has moved, i.e. the gNB is paging the UE in a cell different from the last used cell. A UE monitoring LP-WUS in its last used cell does not wake-up when this indication is received in LP-WUS.

### Radio paging capabilities

SA2 indicated in the LS [4] that there can be inter-operability issues when the UE performs initial registration with a legacy gNB that does not understand/support the (latest) paging capabilities signalled by the UE. It is the task of the gNB to copy the paging capabilities received from the UE in UECapabilityInformation message into the paging capability container on NGAP interface (i.e. UERadioPagingInformation IE).

In LTE the UE sends the paging capabilities separately in a paging capability container, and the eNB only have to copy the paging capability container received over Uu into the paging capability container sent over NGAP interface. This implies that when new paging capabilities are introduced, that no changes to the eNB is required to ensure that these new capabilities are correctly transferred to CN.

In NR a paging capability container on Uu interface was introduced in Rel-17 and the UE-ID based PEI capability was added to the paging container. To avoid backwards compatibility issues the supported NR band list for paging and scheduling offset for PDSCH was not moved into the container. Unfortunately other Rel-17 and Rel-18 paging capabilities have been overlooked to be included in the Rel-17 paging container:

UERadioPagingInformation ::= SEQUENCE {

…

ueRadioPagingInformation            UERadioPagingInformation-IEs,

…

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR   OPTIONAL,

…

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}                         OPTIONAL,

…

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}                         OPTIONAL,

…

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)    OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

…

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

…

UERadioPagingInformation-v1840-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

…

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

...

}

The impact when paging capabilities not included in the paging container are missing in the PAGING message from the CN is briefly described in the table below:

| Paging capability | Impact when missing in the paging capabilities in the PAGING message from CN |
| --- | --- |
| supportedBandListNRForPaging halfDuplexFDD-TypeA-RedCap | gNB may page the (RedCap) UE where it does not support paging |
| dl-SchedulingOffset-PDSCH-TypeA and B (for NTN) | Cross-slot scheduling cannot be used to page the UE. However note that K0 = 0 has to be used for the other UEs that are paged in the PO that have not indicated to support cross-slot scheduling. |
| inactiveStatePO-Determination | In case the DRX cycle in idle and inactive is different the UE may calculate a different PO for monitoring CM-IDLE and RAN paging, in which case the UE would not detect RRC state mismatch when receiving paging in inactive. However the RRC state mismatch is resolved when the UE enters connected mode again. |
| numberOfRxERedCap numberOfRxRedCap supportOf2RxXR | The gNB does not know if the (e)RedCap UE or XR UE supports only 1 or 2 Rx, i.e. does not know when boosting is required. |

From this table it can be concluded that there is not inter-operability issue with Paging when these paging capabilities are missing. But for example in case of (e)RedCap there can be some wasted system resources (e.g. paging in bands that the UE does not support, or boosting when it is not needed):

There are no inter-operability issues when the paging capabilities outside the paging container (i.e. ue-RadioPagingInfo-r17 IE) are missing the PAGING message from CN.

In case of missing (e)RedCap paging capabilities the gNB may use boosting when it is not needed or page in a band that the UE does not support.

ASN.1 corrections to the Rel-17 and Rel-18 paging capabilities have been discussed and not agreed in the past [8, 9]. These Rel-17/18 are not backwards compatible.

In case the gNB does not transfer the LP-WUS paging capability during initial registration to the CN, then there is an inter-operability issue with paging on a gNB that has LP-WUS configured:

The LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container.

It is proposed to reply to SA2 about the RAN2 understanding and agreements.

## When LP-WUS and PEI are configured and supported

RAN1 already agreed:

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

RAN2 has discussed whether PEI should be used in tandem with LP-WUS. But to monitor PEI the MR has to be activated which eliminates to great extend the power saving gains. Furthermore when 32 subgroups are supported then the false alarm rate is already significantly reduced and monitoring PEI as well does not add that much gain. Finally when the number of UEs supporting both LP-WUS and PEI is small compared to the number of UEs supporting PEI only, then the false paging alarm for UEs supporting PEI only is increased in favour of the few UEs supporting LP-WUS and PEI:

When LP-WUS and PEI are used in tandem then the power saving gains are significantly reduced when the UE activates the MR to receive PEI. Network resources are wasted when LP-WUS and PEI are configured in the cell, and the UE decides to monitor LP-WUS only. For the UEs supporting PEI only the false alarm rate is increased when the group of UEs supporting LP-WUS and PEI in tandem is small.

When the NW decides to deploy LP-WUS after PEI has been implemented, the NW should have the control over how the network resources are used. Because it is really questionable whether the UE will activate MR to monitor PEI after LP-WUS. The network should not be required to transmit PEI after LP-WUS in case the UE supporting both LP-WUS and PEI only monitors PEI when it is outside LP-WUS coverage:

When both LP-WUS and PEI are configured and the UE supports both LP-WUS and PEI, then the NW is not required to use PEI for Paging when the UE is outside LP-WUS coverage.

RAN2 agreed to discuss the following options for the LP-WUS subgrouping formula:

Further discussions on the formula

FFS on the following options

- - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Option 3 should be ruled out, because there is no benefit to monitor PEI after LP-WUS when the same subgroup ID is used.

Option 1 and 2 are both feasible, but it is preferred not to use UE_ID bits for PEI when PEI is not configured:

The UE-ID based LP-WUS subgrouping formula is LP-WUSSubgroupID = (floor(UE_ID/(N*Ns* #PEIsubgroups)) mod #LP-WUSsubgroups. With #PEIsubgroups and #LP-WUSsubgroups being the number of UE-ID based subgroups for PEI and LP-WUS respectively. When UE-ID based PEI is not configured then #PEIsubgroups is 1.

Whether the LP-WUS UE-ID based subgroups start at 0 or an offset depends on the LP-WUS i_po mapping to codepoint mapping in RAN1 (e.g. see i_po for PEI in section 10.4A in 23.213).

## Minimum wake-up delay in UE capability

RAN1 is discussing that the gNB may configure a single or multiple time offset(s) in SIB. And that the UE signals the minimum wake-up delay it supports in its UE capability, from a set of 3 candidate values {[80ms], [500ms] and [900ms]}. The minimum wake-up delay includes the synchronization delay using SSB(s). Furthermore RAN1 has discussed that when the configured time offset is shorter than the minimum wake-up delay of the UE, then the UE falls back to legacy PO monitoring, or the UE receives the Paging message in the next PO when it is ready to receive paging.

For the sake of discussion it is assumed that all UE implementations are able to wake-up within 900 ms from ultra-deep sleep (and deep sleep). When the UE is able to wake-up within 500 ms, but a time offset of 900 ms is configured, then the UE has to wait 400 ms until the next PO after it is ready to receive paging.

Different estimates have been provided during the study phase for the power consumption when LR is On a range between 0,01 and 30 was assumed, i.e. more than a factor 1000 between the lowest and highest value, see TR 38.840 and 38.869:

|  | Relative Power (unit) | Transition time (ms) | Transition energy (ms) |
| --- | --- | --- | --- |
| UL | 250 (0 dBm) - 700 (23 dBm) |  |  |
| PDCCH + PDSCH | 300 |  |  |
| SSB | 100 |  |  |
| PDCCH-only | 100 |  |  |
| Micro sleep | 45 | 0 | 0 |
| Light Sleep | 20 | 6 | 100 |
| Deep Sleep | 1 | 20 | 450 |
| Ultra deep sleep (MR) | 0.015 | 400 (800) | 15000 (40000) |
| On (LR) | 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 | FFS | TLR, ramp-up *(PON-POFF)/2 |
| Off (LR) | 0.001/ 0.02/ 0.1 |  |  |

Furthermore the transition time to switch the LR On/Off is FFS, i.e. it is unclear for which durations it is beneficial to switch the LR On/Off. But it is clear that for some durations and LR On power consumption values it is beneficial to switch the LR Off and On when the UE wakes-up too early, and for others the impact when the LR is activated too early is not significant:

When the relative power consumption of LR On is low then there is no significant power consumption impact when the UE wakes-up too early.

When the relative power consumption of LR On is high and there is sufficient time, then the UE can switch the LR Off and On before the next PO.

The support of multiple time offset should not lead to increased number of LP-WUS occasions and increased number of LP-WUS transmissions, as depicted below:

But this is not necessary the case, i.e. the UE that requires a larger time offset can monitor an LP-WUS occasion configured with another PO:

When a single time offset is configured then there is one LP-WUS occasion associated with the PO. But without configuring more LP-WUS occasions, multiple LP-WUS occasions can be associated with a single PO.

In case it is motivated to support multiple time offsets for UEs with different minimum wake-up delay:

A single time offset is configured per PO but multiple LP-WUS occasions can be associated with a single PO.

The proposal above is explained with an example:

- UE signals its minimum time gap from a candidate set {[80ms], [500ms] and [900ms]} in UE capability.

- A single time offset (80 ms) is configured per PO.

- UEs that require 80 ms wake-up time use the configured LP-WUS of the PO.

- UEs that requires a longer wake-up time use the LP-WUS occasion that is closest to the PO but that has a time offset that is larger or equal to the minimum wake-up latency of the UE.

- UE monitors legacy PO determined based on the UE identity, where the UE monitors LP-WUS or not.

In case the UE would monitor a different PO when it is using LP-WUS then this would increase the number of LP-WUS transmissions because the gNB does not know whether the UE is monitoring LP-WUS or not.

## Enabling/disabling LP-WUS operation in the UE

First there is a difference between enabling/disabling and activation/deactivation. Enabling/disabling LP-WUS operation in the UE is semi-static e.g. LP-WUS is disabled when statistics show that the UE is less reachable when using LP-WUS. While activation/deactivation of LP-WUS is more dynamic e.g. when the UE roams in and out of LP-WUS coverage.

CN-assigned based subgrouping using LP-WUS can be disabled by the CN, i.e. the CN can decide not to configure a CN subgroup in the registration accept. However the UE-ID based subgrouping is configured in the RAN and is transparent to the CN. But the NAS and CN signalling can easily be extended to enable the CN to enable/disable UE-ID based LP-WUS for a specific UE:

- The UE can indicate in the registration request that it supports UE-ID based LP-WUS

- The CN can indicate in the registration accept whether the UE is allowed to use UE-ID based LP-WUS

- In case UE-ID based LP-WUS is disabled in the UE, the CN informs RAN:

  - Using the INITIAL CONTEXT SETUP REQUEST message for RAN paging

  - Using the PAGING message for CN paging

Given that the existing signalling already enables the CN to disable CN-assigned LP-WUS for a specific UE, we think it is easy to extend the existing signalling with UE-ID based LP-WUS. It is more complex using RRC signalling to disable UE-ID based subgrouping in RAN for a specific UE:

The CN has the ability to disable/enable LP-WUS in the UE during initial registration, but also has the opportunity to change this in the UE during registration triggered by mobility or periodic location update:

The existing signalling for CN assigned subgrouping is extended to enable the CN to enable/disable UE-ID based LP-WUS in the UE.

## LP-WUS entry/exit condition

There are many examples in NR where the threshold is based on RSRP and/or RSRP measurements, e.g. thresholdMBS. Typically both the RSRP and RSRQ threshold are optionally present, i.e. one of them or both can be configured:

The entry/exit threshold may include RSRP threshold, RSRQ threshold or both.

The exit condition is defined as:

The exit condition is met when one or more of the measured quantities are below the configured threshold(s).

To avoid ping pong with cell reselection the NW can configure a time to trigger:

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA. Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselection ::=                   INTEGER (0..7)

To avoid ping pong with entry/exit for LP-WUS monitoring:

The NW can configure a time to trigger (seconds), i.e. minimum time during which the entry condition has to be fulfilled, before the UE may start LP-WUS monitoring (again).

## LP-WUS configuration

RAN2 agreed that the LP-WUS configuration can be provided in SIB. But RAN2 did not reach consensus whether LP-WUS configuration via dedicated signalling is needed. We think that the LP-WUS configuration is common, i.e. cell specific, and there is no need for RRC signalling to enable UE specific LP-WUS configurations. In case LP-WUS configuration is provided in RRCRelease then there is complexity for how long the dedicated LP-WUS configuration remains valid, and in which cells/areas the LP-WUS configuration is valid. But it is possible to provide different LP-WUS configurations in SIB for certain types of UEs e.g. different LP-WUS configuration for UEs in RRC_IDLE and RRC_INACTIVE. Furthermore LP-WUS configuration in dedicated signalling has been confused with enabling/disabling LP-WUS in a specific UE which is a separate discussion:

LP-WUS configuration is not provided in dedicated RRC signalling.

RAN2 agreed that separate thresholds can be configured for OFDM-based and OOK-based WUR. This implies that the presence of OFDM-based and/or OOK-based thresholds in SIB indicates whether the UE may use OFDM-based and/or OOK-based LP-WUS monitoring in the cell, e.g. when OOK-based threshold are not configured in the cell then the UE may not use OOK-based LP-WUS monitoring in the cell. When the UE uses OOK-based LP-WUS monitoring, then the NW has to use more resources in the LP-WUS signal to reach the UE. Furthermore the NW has to configure the LP-SS to enable LR measurements. Less resources are required for the OFDM-based LP-WUS signal, and OFDM-based UE can use PSS/SSS for sync and serving RRM measurements:

gNB can configure in SIB whether only OFDM-based or both OFDM-based and OOK-based WUR are supported in the cell.

When OFDM-based WUR is supported only, then the gNB is not required to configure LP-SS. When both OFDM-based and OOK-based WUR is configured in SIB, then the gNB is required to use sufficient resources in the LP-WUS signal when it has to wake-up UEs that support OOK-based WUR only. Otherwise the gNB can use less resources in the LP-WUS signal:

When the UE supports both OFDM and OOK-based WUR then it is up to UE implementation to use OFDM and/or OOK-based WUR for LP-WUS reception.

When the gNB knows that it has to page a UE that supports both OFDM and OOK, then the gNB sends LP-WUS that can be received by OOK-based WUR. It is thus up to the UE implementation to stop monitoring earlier when LP-WUS has been detected.

## UEID-based and CN-assigned subgroup signalling

RAN2 assumes that for LP-WUS the same signalling procedure and signalling messages as with PEI will be used. It is up to SA2/CT1/RAN3 to decide the signalling details, e.g. introduction of new IEs [1]:

Concerning LP-WUS subgrouping UE capability and LP-WUS configuration in SIB the same principles can be used as with PEI:

If the UE supports LP-WUS then the UE also supports UEID-based subgrouping. The UE may in addition also support CN-assigned subgrouping.

The gNB can indicate in the LP-WUS configuration in SIB that only UEID-based subgrouping, only CN-assigned based subgrouping, or both are supported in the cell.

If the UE is configured with a CN-assigned subgroup ID and the gNB has configured both UEID-based and CN-assigned subgrouping then the UE uses the CN-assigned subgrouping ID.

In the UEID-based subgrouping capability the UE can indicate in which of the supported bands the UE supports LP-WUS by including a band list.

The gNB does not use LP-WUS when the UE is paged in a band where the UE does not support LP-WUS, based on the radio paging capabilities in the PAGING message from CN for UEs in CM_IDLE and based on the UE capabilities for RAN paging for UEs in RRC_INACTIVE.

## LP-WUS monitoring rules

RAN1 agreed that when the UE does not detect LP-WUS or LP-WUS is received but the LP-WUS  subgroup is not included then the UE is not required to monitor the following PO. RAN1 continues to discuss how to MOs can be grouped and used in case of multi beam operation e.g. MOs are repeated on a beam or the UE only monitors specific MOs on a beam.

There can be cases where the UE is not able to monitor LP-WUS, e.g. during cell re-selection. In such case the UE should monitor the associated PO:

If the UE is not able to monitor the LP-WUS (FFS for which use cases) then the UE is required to monitor the following PO.

In case of multibeam operation each beam includes at least one LO and one MO:

Similar as with PEI/Paging it is up to UE implementation which beam(s) to monitor for Paging:

It is up to UE implementation on which beam(s) to monitor the LO.

## LP-WUS and eDRX

LP-WUS can be supported with eDRX with limited specification impact, similar as eDRX is supported with PEI, i.e. for PO monitoring within the PTW the same monitoring rules apply as for normal DRX. The only change to the specification agreed for PEI and eDRX was to allow for a larger UEID value (5G-S-TMSI mod 32768 instead of 1024) in 38.304 to properly distribute the subgroups with a larger DRX cycle.

However the use of LP-WUS with eDRX may not always be beneficial as simulations results have shown [4]. For example when the eDRX cycle is very long (e.g. larger than 500 sec), then the MR ultra-deep sleep power consumption (Relative Power unit 0,015) becomes the determining factor for the battery lifetime and it does not matter whether LP-WUS is used or not.

But the UE implementation may decide to switch off the modem when the eDRX cycle is very long. This may increase the sync delay and time to wake-up, because then there was no low power/accuracy clock running when the UE is sleeping, but this may be beneficial in the end with a very long eDRX:

eDRX is supported with LP-WUS if it can be supported without additional changes, i.e. no separate time offset for eDRX and the UE monitors LP-WUS and POs during PTW similar as during normal DRX.

# MR and LR operating in different bands

Based on company contributions [1, 2] a way forward for the operation of MR and LR in different bands was endorsed in the RAN plenary [3]:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).  For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

- Detailed discussions and final solution selection is left up to WGs.

Four possible solutions are described in [1]:

| Scenarios |  | Overload on LR band | Limited Usage of LP-WUS | Expected specification efforts |
| --- | --- | --- | --- | --- |
| Intra-Frequency | Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band) | Yes | No | - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline. - Frequency priority configured for LP-WUS capable UE in SIB |
|  | Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) | No | No | - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline - Frequency priority configured for LP-WUS capable UE in SIB or dedicated signalling. - UE needs to perform cell reselection following the legacy frequency priority after waking up. |
|  | Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation) | Highly dependent on the awareness of load in LR band by the gNB | Dependent on LR band load | - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline - Frequency priority configured for LP-WUS capable UE in dedicated signalling. |
|  | No solution- MR is not prioritized to camp on the same band as LR (i.e. the UE would only use LP-WUS whenever it was already camped on LR band) | Dependent on LR band load | Yes | - No specification impact is expected but very limited usage of LP-WUS procedures |
| Inter-Frequency | Solution 4: MR (SI/Paging/RACH) is camped on a different band than LR. LR can wake up MR on the band it is camped on | No | FFS | - LP-WUS - Paging offset design in RAN1/RAN2. - Entry/exit conditions in RAN2/RAN4. - Validity of measurements in RAN4 as LR and MR are operating in different frequencies. Note: Might be applicable for co-located case only. Non co-located case is FFS. |

In this contribution the “LR frequency” is a frequency/band where the UE supports LP-WUS. On the MR frequency the UE does not support LP-WUS. Furthermore it is assumed that the LR frequency is in low-band and the MR frequency in mid-band.

In our view solution 2 is incorrectly described as an intra-frequency solution, because when the UE responds to paging the UE performs access in another band than where the UE received the paging. Solution 4 described as an inter-frequency solution does the same. The difference between solution 2 and 4 is that in solution 2 the UE receives LP-WUS and Paging in the same frequency, while with solution 4 the UE is receiving LP-WUS on an LR frequency (e.g. low-band) and Paging on an MR frequency (e.g. mid-band), i.e. the difference is when the UE switches from LR to MR frequency. With solution 4 the UE is camped on two cells simultaneously, while with solution 2 there is a hard switch:

Solution 2 is an inter-frequency solution, i.e. UE monitors paging in one band and triggers access in another band.

For discussion sake the following deployment scenario is assumed (see also picture below):

- Low-band (coverage band) provides full coverage including indoor. Mid-band provides additional capacity in urban areas, but indoor mid-band coverage may be limited. In rural areas there may be low-band coverage only, while in urban areas the majority of UEs is camped on mid-band.

- In majority of cases idle mode mobility is provided via intra-frequency cell reselection. When camped on mid-band inter-frequency cell reselection may be required when going in and out of building or when the UE is in challenging radio propagation conditions.

- Mid-band frequencies are configured as higher priority frequencies in system information. With redirection and dedicated priorities in RRCRelease the NW ensures that available resources are used optimally and the UEs are distributed over the available frequencies in idle mode.

- The mid-band cell may or may not be co-located with the low-band gNB. In case the mid and low-band gNBs are not co-located then it may depend on the distance between mid and low-band gNB whether the mid-band gNB is aware of the congestion level in low-band gNB:

The majority of UEs are camped on mid-band, i.e. in case the UE would only support LP-WUS in low-band then this would be a serious limitation in the usage of the feature. However the LP-WUS band restrictions need to be discussed further:

- The possibility/solution that the UE supports LP-WUS on (almost) all supported bands should also be evaluated and compared w.r.t. cost and complexity of the other solutions. In our understanding it is feasible to support LP-WUS on (almost) all of the supported bands, but this may come with a cost/complexity and the LP-WUS coverage in higher bands may be reduced. It should also be noted that RAN1 recently agreed that LP-WUS is supported in FR2, i.e. higher frequencies are not necessarily excluded.

- In case it is agreed that LP-WUS is not supported on all bands:

  - Are these band restrictions temporary, e.g. only in the first phase? In that case the number of LP-WUS UEs is low and solutions like solution 1 and 3 may be acceptable, because LP-WUS UEs do not cause significant congestion.

  - In case these band restrictions are more permanent of nature, then there should be consensus in which bands LP-WUS is supported. In case the UE’s band restrictions are diverse and do not overlap then it is more difficult to find an effective solution.

  - In case a solution is agreed the solution should be mandatory for a UE supporting LP-WUS to avoid the specification and deployment of a feature that will not be used. The network is not required to support a “mixed” scenario where some UEs have band restrictions while other UEs do not have band restrictions.

RAN2 should first discuss if any solution is required:

RAN2 should first evaluate whether it is less costly/complex for the UE to support LP-WUS on (almost) all supported bands or support solutions like 1, 2 or 3.

RAN2 should have a clear and consistent picture of the LP-WUS band restrictions:

RAN2 should discuss whether the band restrictions are the same for all the UEs and on which bands LP-WUS is supported. For example LP-WUS is only supported in low-band (< 1 GHz). Then the UE does not indicate bands outside low-band in the band-list of the LP-WUS UE capability. And the UE does not monitor LP-WUS when camped on a low-band frequency/band omitted from the band-list.

RAN2 should discuss whether a temporary work around is sufficient or a future proof solution is required:

In case the restrictions are only temporary in the first phase then solutions like 1 and 3, which are less costly/complex compared to solution 2, may be suitable because the number of LP-WUS UEs is limited and do not cause significant congestion on the LR band.

RAN2 needs to ensure that time and cost is spend well:

In case a solution like 2 is adopted then the solution is mandatory for an LP-WUS UE. In case LP-WUS is only supported in low-band then the NW is not expected to deploy in LP-WUS in other bands.

The solutions are evaluated further in the text below:

Evaluation of solution 1

The NW cannot configure LP-WUS frequency(ies) as the highest priority frequency, because this would cause all UEs within range to camp on the LP-WUS frequency and cause congestion. But when the number of LP-WUS UEs is limited it is feasible to introduce (higher priority) frequency information only to be used by LP-WUS UEs in SIB2/SIB4. For example it is indicated in SIB on which frequency(ies) LP-WUS is supported and when the UE wants to use LP-WUS or is using LP-WUS to monitor paging then the UE may consider the LP-WUS frequency to have the highest priority, or an explicit frequency priority is included in the SIB.

With solution 1 the LP-WUS UE may prioritize the LP-WUS frequency:

With solution 1 the UE may prioritize the frequency on which it can receive LP-WUS when it is using or wants to use LP-WUS to monitor paging. FFS whether information in SIB needs to be provided or existing information can be re-used.

In case congestion is experienced on the LP-WUS frequency:

With solution 1 in case the LP-WUS frequency is congested the NW can indicate in RRCRelease that the LP-WUS UE should deprioritize the LP-WUS frequency(ies) (FFS whether deprioritisationReq can be re-used) or the NW can remove/modify the LP-WUS frequency priority in SIB, if configured.

The NW can disperse other UEs from the congested frequency using redirection, dedicated priorities or deprioritization in the RRCRelease.

Evaluation of solution 2

Solution 2 enables the UE to monitor LP-WUS and receive Paging in low-band and respond to paging (or when the UE has UL data) to perform RACH in mid-band. Dependent on the deployment and coverage the UE monitors LP-WUS or Paging PO directly and performs RACH in mid-band or low-band.

In rural areas, i.e. when there is no mid-band coverage, the UE is expected to perform cell reselection while camped on low-band. In urban areas however, i.e. when there is mid-band coverage, the UE is expected to perform cell reselection while camped on mid-band. The mid-band provides full coverage and the UE does not reselect to low-band to go back to mid-band again. Furthermore, in urban areas the UE is not expected to reselect while camped on low-band, because the LP-WUS exit condition is expected to be fulfilled first before the UE finds a better low-band cell while camped on low-band. When the UE is outside LP-WUS coverage in urban areas, the UE is no longer allowed to prioritize the low-band, and higher priority frequency measurements will cause the UE to reselect to mid-band:

With solution 2 the MR measurements in mid-band cannot be used for the LP-WUS entry/exit condition in low-band, because the MR and LR cell have different coverage and the gNBs may not be co-located. The same issue applies to MR serving cell offloading in mid-band to LR serving cell measurements in low-band.

Solution 2 introduces two changes to the operation of LR and MR in the same band (i.e. intra-frequency operation):

- Normally the UE selects the PCell from connected mode after it has been released. If LP-WUS is not supported on the PCell frequency, then an LP-WUS UE will try to select a different cell/frequency after release.

- When the UE is monitoring LP-WUS and the UE needs to respond to paging or has UL data, then the UE should try to find a suitable cell on a frequency in another band to trigger RACH. For example when the UE is monitoring LP-WUS in low-band, then the UE should try to trigger RACH in mid-band to avoid congestion in low-band.

For the two changes see the dashed green arrows below:

Further discussion is needed whether the UE performs cell selection or reselection when responding to paging, i.e. whether the MR cell only has to be suitable, or additional measurement/reselection rules apply. In any case, this is a “forced” cell reselection, i.e. the LR cell may be perfectly suitable and no intra-/inter-frequency measurements on the LR frequency are triggered. Perhaps it is sufficient when the UE considers the LP-WUS frequencies or frequencies in low-band as the lowest priority frequencies and cell reselection is triggered:

With solution 2 when the UE is interested to monitor or is monitoring LP-WUS and while the LP-WUS entry condition is fulfilled then the UE may consider the LP-WUS frequency as the highest priority frequency and the UE is not required to perform higher priority measurements. With solution 2 when the UE responds to paging or there is UL data then the UE considers the LP-WUS frequency and other frequencies in the same band as the lowest priority frequency and the UE is required to perform higher priority measurements.

When the frequency is not congested then the UE can be allowed to trigger RACH on the same frequency:

With solution 2 in some cases the UE is allowed to trigger RACH on the LR frequency, e.g. when the LR frequency is not congested (e.g. in SIB or Paging message) or when the  establishment/resume cause is e.g. emergency or highPriorityAccess.

When the UE is moving around and there is a long time between paging, then the UE may have lost the stored MR cell, i.e. the cell where the UE was released, or the UE may have roamed out of mid-band coverage all together. When the UE has to trigger inter-frequency measurements to find a suitable MR cell then the response to paging or of UL data may be significantly delayed. Thus there has to be a time limit during which the UE is allowed to search for a suitable MR cell after which the UE should trigger RACH on the LR frequency:

With solution 2 there is a time limit during which the UE can search for a suitable MR cell, after which the UE should trigger RACH on LR frequency.

When the UE needs to trigger RACH, then this event can be compared with the event when the UE wakes-up from a long eDRX, i.e. the UE may have lost the old cell and has to start measurements from scratch. The following delay components can be identified when the UE responds to paging or when there is UL data:

- MR wakeup latency (400-800 ms)

- Synchronization (e.g. 2 SSBs)

- Intra-frequency measurements (2 samples spaced half DRX cycle)

- Strongest cell selection

- MIB/SIB1 acquisition

- SI acquisition, if needed

The additional latency introduced by LR and MR operating in different bands should be evaluated further:

The additional latency introduced by LR and MR operating in different bands is evaluated further.

The NW can include redirectedCarrierInfo in the RRCRelease. The UE will try to select a suitable cell on that carrier. Thus when an LP-WUS UE is released on a mid-band frequency the NW redirect the UE to a low-band frequency where the UE can monitor LP-WUS. Alternatively this is left to UE implementation.

When the UE triggers RACH on another frequency then where it received the paging, then the NW should retain control over the frequency where the UE triggers RACH for load balancing and traffic steering purposes. It should be possible for the NW to include rachRedirectedCarrierInfo in the release, i.e. when the UE replies to paging the UE tries to select that RACH carrier:

In case the UE monitors LP-WUS and receives Paging or UL data then the UE should try to (re-)select to the RACH carrier and trigger RACH there:

With solution 2 the UE can be configured with rachRedirectedCarrierInfo in RRCRelease, i.e. the UE tries to select a cell on the RACH carrier when the UE is monitoring LP-WUS and RACH is triggered.

Further discussion is needed whether it is beneficial if the UE performs relaxed MR frequency measurements while the UE is monitoring LP-WUS. Because the cost of activating the MR and perform measurements on the MR frequency quickly outweigh the benefit of monitoring LP-WUS. In case the MR frequency measurements are too relaxed then they also may be outdated when they are needed (e.g. after 1 minute):

With solution 2 it is FFS whether it is beneficial to perform RRM measurements on the MR frequency when the UE is monitoring LP-WUS on the LR frequency (e.g. when UE is moving).

It can be discussed further whether the UE should first camp on the MR cell after release to be reachable for paging, or that the UE can immediately try to select an LR cell on another frequency. In case the old LR cell is no longer found, and the UE triggers inter-frequency measurements in low-band then the UE is not reachable for paging for some time. The UE may be outside low-band coverage (not very likely) or inside low-band coverage but outside LP-WUS coverage (more likely):

With solution 2 it is FFS whether the UE should first camp on the PCell/MR frequency to not miss paging or that the UE can immediately try to select a cell on the LR frequency where the LP-WUS entry condition is fulfilled.

When LP-WUS exit condition is fulfilled then the UE shall trigger higher priority measurements and consider the current frequency and frequencies in that band as the lowest priority frequency. Only when the UE cannot find a suitable cell in another band then the UE may camp on the LR frequency/band while the LP-WUS exit condition is fulfilled. The UE should try to move out of the LR frequency/band when the LP-WUS exit condition is fulfilled, because the delay is increased to respond to paging or initiate UL transmissions but there is no power saving benefit, i.e. the UE cannot monitor LP-WUS. While the UE is outside LP-WUS coverage, the UE can trigger relaxed inter-frequency measurements to detect when it is inside LP-WUS coverage again:

When the LP-WUS exit condition is fulfilled then the UE no longer considers this LP-WUS frequency as the highest priority frequency:

With solution 2 when the LP-WUS exit condition is fulfilled then the UE is required to perform higher priority measurements and consider the LP-WUS frequency and other frequencies in the same band as the lowest priority frequency.

Above we have identified and discussed details of solution 2 and these details can be discussed further. But there are a few topics that should be discussed first, perhaps impacting the feasibility and other WGs:

- When the UE is released the UE performs cell selection, i.e. the UE is free to select any cell that is suitable, i.e. the S criterion is fulfilled. But typically the UE selects the strongest cell on the frequency where the UE is released, i.e. the cell that was the PCell in connected mode. And the PCell is the first last visited cell in the Recommended Cell List in the UE CONTEXT RELEASE COMPLETE and PAGING message, see section 9.3.1.71 in 38.413:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Cell List |  | 1 |  |  |
| >Recommended Cell Item |  | 1..<maxnoofRecommendedCells> |  | Includes visited and non-visited cells, where visited cells are listed in the order the UE visited them with the most recent cell being the first in the list. Non-visited cells are included immediately after the visited cell they are associated with. |
| >>NG-RAN CGI | M |  | 9.3.1.73 |  |
| >>Time Stayed in Cell | O |  | INTEGER (0..4095) | This is included for visited cells and indicates the time a UE stayed in a cell in seconds. If the UE stays in a cell more than 4095 seconds, this IE is set to 4095. |

The NW uses this information to page the UE only in the last visited cell in the first paging attempt. When the UE has not moved and the first paging attempt is limited to the last used cell, then paging in the complete registration/RNA area can be avoided.

However the LP-WUS UE will try to select a cell on another frequency, when the current frequency does not support LP-WUS. Thus the first paging attempt to an LP-WUS UE is likely to fail when the UE was released on a frequency where LP-WUS is not supported and the NW may escalate the paging to the complete registration/RNA area in the second paging attempt to find the UE. This will delay the paging, but also significantly increase in the paging load. This problem applies to both RAN and CM-IDLE paging. A potential solution is that the gNB includes a lpwusRedirectedCellCarrierInfo in the RRCRelease, e.g. a cell and frequency pair where LP-WUS is supported that overlaps with the cell in which the UE is released on a frequency where LP-WUS is not supported:

The lpwusRedirectedCellCarrierInfo is also included in the signalling where the paging assistance information is included. The RAN includes this information in the UE CONTEXT RELEASE COMPLETE, and the CN includes this information in the PAGING message. The RAN and CN use this information for the first paging attempt. In case the gNB does not have the information about the overlapping LP-WUS cell-frequency then the information can be configured via OAM or LP-WUS is not supported for that case:

- When LP-WUS transmissions are intended to be restricted, i.e. lastUsedCellOnly flag is set in SIB1, then there is a problem. The LP-WUS transmissions can be limited to the redirected cell in lpwusRedirectedCellCarrierInfo IE. If redirectedCellOnly flag is configured in SIB1 then LP-WUS is only used when paging the UE in the redirected cell, and the UE is only monitoring paging when camped on this cell:

- In case the UE selects a cell after release from the MR cell that belongs to a different registration area, then this will trigger a Tracking Area Update (TAU) and Mobility Registration Update in NAS. Similar when the UE is released to RRC_INACTIVE and selects a cell that does not belong to the configured ran-NotificationAreaInfo, then the UE will trigger an RNA update. When the UE responds to paging then the UE shall not reselect to a cell to perform RACH that belongs to a different registration area or RNA, i.e. the NW assumes that the UE responds to paging in the registration area or RNA where the UE is paged. However AS is not aware when the UE changes registration area, i.e. this information resides in NAS. FFS how it can be prevented that the inter-frequency cell reselection from LR to MR frequency does not trigger TAU update.

- The time when RAN/CN trigger another paging attempt when the UE did not respond to paging in the first attempt are dependent on the NW implementation. It needs to be discussed further whether the paging response delay with MR and LR in different bands may exceed these NW dependent delays.

- The access control on the MR cell where the UE triggers RACH needs further discussion. What establishment/resume cause should the UE use when receiving Paging on the LR cell and performing RACH on the MR cell? Normally the UE uses mt-Access when responding to Paging, and the gNB does not page the UE when the cell is congested and RACH should be avoided. When Access Category 0 with mt-Access is used the UE considers the access attempt as allowed. But the LR cell may not be aware of the congestion status on the MR cell or know which MR cell the UE will select for access. FFS whether the UE should use mo-Data and apply UAC on the MR cell and return to the LR cell when it is barred. When mo-Data is used the potential impact on NAS/CT1 should be discussed.

The potential issues and possible solutions described above need to be evaluated further:

With solution 2 the following issues need further discussions: last visited cell, lastUsedCellOnly, paging does not trigger TAU update, paging response delay and access control.

Evaluation of solution 3

The NW in the RRCRelease can redirect the UE supporting LP-WUS to an LP-WUS frequency and configure a (high) dedicated priority for the LP-WUS frequency. The NW needs to configure a dedicated priority, otherwise the UE may have to reselect to a higher priority frequency when a higher priority frequency cell is detected:

With solution 3 when the LP-WUS frequency is not congested the NW can redirect the LP-WUS UE to an LP-WUS frequency and configure a dedicated priority.

When the gNB knows that the LP-WUS frequency is congested, then the gNB can omit this information in the release to manage the congestion. In case the gNB of the connected mode cell does not have up to date information about the LP-WUS frequency congestion, then the gNB may decide to redirect based on the time of day or throttle the redirection rate to a certain maximum.

Comparison of the solutions

|  | Solution 0 (LP-WUS on all supported bands) | Solution 1 | Solution 2 | Solution 3 |
| --- | --- | --- | --- | --- |
| Complexity/cost | Considerable | Low | High | Low |
| Specification impact | None | Low | High | None |
| UE impact | Considerable | Low | High | None |
| NW impact | None | Low | High | Moderate |
| Limitations | None | Not scalable | FFS | Not scalable |

Solution 0 is the preferred solution from a NW perspective, provided that the UE impact is acceptable. When the band restrictions are only in the first phase, then solution 1 is preferred because it is the simplest. Solution 2 may be required when LP-WUS cannot be supported in certain band(s). But for solution 2 several issues need to be resolved:

It is preferred that the UE supports LP-WUS on (almost) all supported bands. Solution 1 is preferred when the band restrictions only apply to the first phase. Solution 2 may be required when it is agreed that LP-WUS will only be supported in certain bands but several issues with solution 2 need to be resolved.

# Summary

RAN2 is kindly asked to discuss LP-WUS in idle and inactive:

Observation 1	There are no inter-operability issues when the paging capabilities outside the paging container (i.e. ue-RadioPagingInfo-r17 IE) are missing the PAGING message from CN.

Observation 2	In case of missing (e)RedCap paging capabilities the gNB may use boosting when it is not needed or page in a band that the UE does not support.

Observation 3	When LP-WUS and PEI are used in tandem then the power saving gains are significantly reduced when the UE activates the MR to receive PEI. Network resources are wasted when LP-WUS and PEI are configured in the cell, and the UE decides to monitor LP-WUS only. For the UEs supporting PEI only the false alarm rate is increased when the group of UEs supporting LP-WUS and PEI in tandem is small.

Observation 4	When the relative power consumption of LR On is low then there is no significant power consumption impact when the UE wakes-up too early.

Observation 5	When the relative power consumption of LR On is high and there is sufficient time, then the UE can switch the LR Off and On before the next PO.

Observation 6	When a single time offset is configured then there is one LP-WUS occasion associated with the PO. But without configuring more LP-WUS occasions, multiple LP-WUS occasions can be associated with a single PO.

Observation 7	Solution 2 is an inter-frequency solution, i.e. UE monitors paging in one band and triggers access in another band.

Observation 8	With solution 1 the UE may prioritize the frequency on which it can receive LP-WUS when it is using or wants to use LP-WUS to monitor paging. FFS whether information in SIB needs to be provided or existing information can be re-used.

Observation 9	With solution 1 in case the LP-WUS frequency is congested the NW can indicate in RRCRelease that the LP-WUS UE should deprioritize the LP-WUS frequency(ies) (FFS whether deprioritisationReq can be re-used) or the NW can remove/modify the LP-WUS frequency priority in SIB, if configured.

Observation 10	With solution 3 when the LP-WUS frequency is not congested the NW can redirect the LP-WUS UE to an LP-WUS frequency and configure a dedicated priority.

Proposal 1	The following abbreviations are adopted: Low Power receiver Wake Up Signal (LP-WUS), Low power Receiver (LR) and Main Receiver (MR).

Proposal 2	When the UE has an emergency PDU session LP-WUS is not used.

Proposal 3	Similar as with Rel-17 PEI the LP-WUS transmissions can be limited to lastUsedCellOnly.

Proposal 4	LP-WUS indicates when there is paging for a UE that has moved, i.e. the gNB is paging the UE in a cell different from the last used cell. A UE monitoring LP-WUS in its last used cell does not wake-up when this indication is received in LP-WUS.

Proposal 5	The LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container.

Proposal 6	When both LP-WUS and PEI are configured and the UE supports both LP-WUS and PEI, then the NW is not required to use PEI for Paging when the UE is outside LP-WUS coverage.

Proposal 7	The UE-ID based LP-WUS subgrouping formula is LP-WUSSubgroupID = (floor(UE_ID/(N*Ns* #PEIsubgroups)) mod #LP-WUSsubgroups. With #PEIsubgroups and #LP-WUSsubgroups being the number of UE-ID based subgroups for PEI and LP-WUS respectively. When UE-ID based PEI is not configured then #PEIsubgroups is 1.

Proposal 8	A single time offset is configured per PO but multiple LP-WUS occasions can be associated with a single PO.

Proposal 9	The existing signalling for CN assigned subgrouping is extended to enable the CN to enable/disable UE-ID based LP-WUS in the UE.

Proposal 10	The entry/exit threshold may include RSRP threshold, RSRQ threshold or both.

Proposal 11	The exit condition is met when one or more of the measured quantities are below the configured threshold(s).

Proposal 12	The NW can configure a time to trigger (seconds), i.e. minimum time during which the entry condition has to be fulfilled, before the UE may start LP-WUS monitoring (again).

Proposal 13	LP-WUS configuration is not provided in dedicated RRC signalling.

Proposal 14	gNB can configure in SIB whether only OFDM-based or both OFDM-based and OOK-based WUR are supported in the cell.

Proposal 15	When the UE supports both OFDM and OOK-based WUR then it is up to UE implementation to use OFDM and/or OOK-based WUR for LP-WUS reception.

Proposal 16	If the UE supports LP-WUS then the UE also supports UEID-based subgrouping. The UE may in addition also support CN-assigned subgrouping.

Proposal 17	The gNB can indicate in the LP-WUS configuration in SIB that only UEID-based subgrouping, only CN-assigned based subgrouping, or both are supported in the cell.

Proposal 18	If the UE is configured with a CN-assigned subgroup ID and the gNB has configured both UEID-based and CN-assigned subgrouping then the UE uses the CN-assigned subgrouping ID.

Proposal 19	In the UEID-based subgrouping capability the UE can indicate in which of the supported bands the UE supports LP-WUS by including a band list.

Proposal 20	If the UE is not able to monitor the LP-WUS (FFS for which use cases) then the UE is required to monitor the following PO.

Proposal 21	It is up to UE implementation on which beam(s) to monitor the LO.

Proposal 22	eDRX is supported with LP-WUS if it can be supported without additional changes, i.e. no separate time offset for eDRX and the UE monitors LP-WUS and POs during PTW similar as during normal DRX.

Proposal 23	RAN2 should first evaluate whether it is less costly/complex for the UE to support LP-WUS on (almost) all supported bands or support solutions like 1, 2 or 3.

Proposal 24	RAN2 should discuss whether the band restrictions are the same for all the UEs and on which bands LP-WUS is supported. For example LP-WUS is only supported in low-band (< 1 GHz). Then the UE does not indicate bands outside low-band in the band-list of the LP-WUS UE capability. And the UE does not monitor LP-WUS when camped on a low-band frequency/band omitted from the band-list.

Proposal 25	In case the restrictions are only temporary in the first phase then solutions like 1 and 3, which are less costly/complex compared to solution 2, may be suitable because the number of LP-WUS UEs is limited and do not cause significant congestion on the L band.

Proposal 26	In case a solution like 2 is adopted then the solution is mandatory for an LP-WUS UE. In case LP-WUS is only supported in low-band then the NW is not expected to deploy in LP-WUS in other bands.

Proposal 27	With solution 2 when the UE is interested to monitor or is monitoring LP-WUS and while the LP-WUS entry condition is fulfilled then the UE may consider the LP-WUS frequency as the highest priority frequency and the UE is not required to perform higher prority measurements. With solution 2 when the UE responds to paging or there is UL data then the UE considers the LP-WUS frequency and other frequencies in the same band as the lowest priority frequency and the UE is required to perform higher priority measurements.

Proposal 28	With solution 2 in some cases the UE is allowed to trigger RACH on the LR frequency, e.g. when the LR frequency is not congested (e.g. in SIB or Paging message) or when the  establishment/resume cause is e.g. emergency or highPriorityAccess.

Proposal 29	With solution 2 there is a time limit during which the UE can search for a suitable MR cell, after which the UE should trigger RACH on LR frequency.

Proposal 30	The additional latency introduced by LR and MR operating in different bands is evaluated further.

Proposal 31	With solution 2 the UE can be configured with rachRedirectedCarrierInfo in RRCRelease, i.e. the UE tries to select a cell on the RACH carrier when the UE is monitoring LP-WUS and RACH is triggered.

Proposal 32	With solution 2 it is FFS whether it is beneficial to perform RRM measurements on the MR frequency when the UE is monitoring LP-WUS on the LR frequency (e.g. when UE is moving).

Proposal 33	With solution 2 it is FFS whether the UE should first camp on the PCell/MR frequency to not miss paging or that the UE can immediately try to select a cell on the LR frequency where the LP-WUS entry condition is fulfilled.

Proposal 34	With solution 2 when the LP-WUS exit condition is fulfilled then the UE is required to perform higher priority measurements and consider the LP-WUS frequency and other frequencies in the same band as the lowest priority frequency.

Proposal 35	With solution 2 the following issues need further discussions: last visited cell, lastUsedCellOnly, paging does not trigger TAU update, paging response delay and access control.

Proposal 36	It is preferred that the UE supports LP-WUS on (almost) all supported bands. Solution 1 is preferred when the band restrictions only apply to the first phase. Solution 2 may be required when it is agreed that LP-WUS will only be supported in certain bands but several issues with solution 2 need to be resolved.

# References

- RP-242656, On LR and MR operating in different bands in release 19, VDF, Vivo, Huawei, DISC, RAN#106

- RP-243168, Discussion on Rel-19 LP-WUS separate band issues, ZTE, DISC, RAN#106

- RP-243266, Way forward on Rel-19 LP-WUS separate band issues, RAN2 chair, WF, RAN#106

- S2-2412876, LS Reply on LP-WUS subgrouping, LS out, To: RAN2, RAN3, CT1, SA2#166

- S2-2412875, Support of LP-WUS Assistance, CR 23.501, vivo, Ericsson, Vodafone, Huawei, HiSilicon, SA2#166

- S2-2412877, LP-WUS subgroup support, CR 23.502, Ericsson, vivo, Huawei, HiSilicon, Vodafone, SA2#166

- R2-2411041, LS on emergency services and eDRX, LS out, To: SA2, CT1, CC: RAN3, RAN2#128

- R2-2208937, [AT119-e][013][NR17] RRC I, Ericsson, Report, RAN2#119-e

- R2-2208654, Correction on UERadioPagingInformation and UERadioPagingInfo container, Ericsson, CR 38.331, Rel-17, RAN2#119-e

# Appendix: Agreements

## RAN1 agreements (selection)

R1-2410863	Summary #6 (Final) on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Agreement

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Conclusion

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Conclusion

No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

R1-2409815	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

Agreement

At least for the 1:1 LO to PO mapping,

- The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Conclusion

There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

R1-2409816	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

R1-2410861	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Conclusion

From RAN1 perspective, there is no consensus on the following proposal:

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

R1-2410862	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

For future meetings:

Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.

## RAN2 agreements (selection)

Subgrouping

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

Further discussions on the formula

FFS on the following options

- - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Further discussions on R1 LS in R2-2407921/ R1-2407559

R2-2410955 Minutes of offline discussion: [AT128][205][LPWUS] R2 impact analysis and draft reply LS on LP-WUS supporting in different bands, VDF

We will use the following to draft the LS. The LS is not CC to RP.

State of discussions in RAN WG2:

During RAN WG2 discussions, many companies raised concerns that if LP-Receivers support only subset of the bands supported by the main receiver and if LP-WUS, Paging and RACH are executed in the LP-WUS bands it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority. This will impact user performance and decrease the utilisation of higher frequency bands. In order to ensure the LP-WUS capable UEs are camping on the band LR implemented, the NW would need to offload LP capable UEs to LR band.

On the other hand if the NW operator does not offload the UEs to the band LP receiver supports, only a subset of UEs would profit from LP-WUS feature.

Some companies do not see that overload problem would occur as the NW implementation can control the offload of the UEs to the lower bands in dedicated way.

In regards to the overload problem, RAN WG2 has not finished evaluations and would need to study the solutions more.

Actions:

RAN2 kindly asks RAN WG1 and WG4 to take the information above into account once deciding on the support of the use case mentioned above.

The LS is approved (unseen) in R2-2410957.