3GPP TSG-RAN2 Meeting #129	R2-2501094

Athens, Greece, 17 - 21 February 2025	Revision of R2-2410086

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS and RRM measurements are discussed further.

For the RAN2 and RAN4 agreements the reader is referred to the Annex.

# Discussion

## MR serving cell offloading and further relaxation

RAN4 agreed to study different use cases for MR serving cell offloading (case #1) and relaxation (case #2 and #3):

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

The WID objective also includes further MR serving cell relaxation and offloading (RP-241824):

- Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

Case 1 (MR serving cell fully offloading)

RAN2 has agreed that for the entry condition of fully offloading (RAN4 case #1) an MR threshold is configured, and optionally an LR threshold (see annex). But RAN2 did not explicitly agree that when the entry condition is fulfilled and the LR is performing serving cell measurements, that the MR is then not required to perform serving cell measurements. This aspect is essential and needs to be captured in the specification:

When the LR entry condition for serving cell offloading is fulfilled and the LR performs serving cell measurements, then the MR is not required to perform serving cell measurements (i.e. RAN4 case #1 is supported).

Case 2 (MR serving cell relaxation aka partial offloading)

When the MR serving cell measurements can be “fully” offloaded to the LR, then in our view there is no (strong) need for “partial” offloading. What is the use case when MR performs relaxed serving cell measurements and the LR continuous serving cell measurements (RAN4 case #2)?. What are the relaxed MR serving cell measurements used for? They cannot be combined with the LR measurements. Furthermore another entry condition would need to be defined for this case which is unnecessarily complex:

The case when MR performs relaxed serving cell measurements while the LR performs continuous serving cell measurements is not supported (RAN4 case #2, i.e. partial offloading, is not supported).

Case 3 (MR serving cell relaxation with MR neighbour cell relaxation)

Similar as with RAN4 case #2, it is not clear why and what the relaxed MR serving cell measurements are used for, while the LR is performing continuous LR serving cell measurements.

The serving cell exit condition is solely dependent on LR measurements, i.e. does not require MR measurements. When the MR performs relaxed intra-frequency measurements, i.e. MR serving cell measurements come for free in such case. But this is not the case for relaxed inter-frequency neighbour cell measurements.

There can be use cases where the MR and LR have to perform serving cell measurements simultaneously. For example when MR measurements are used to retune the LR timing/frequency e.g. when the LR quality is bad [1]. Or MR and LR measurements are used for the entry condition. But in such cases the MR performs serving cell measurements continuously every DRX cycle, i.e. they are not relaxed.

Perhaps relaxed MR serving cell measurements performed during intra-frequency neighbour cell measurements can be used to assist the LR exit condition, but how to use these MR serving cell measurements can be left to UE implementation. Furthermore, it is assumed that cell reselection is performed based on MR measurements only and for cell reselection both serving and neighbour cell measurements are required:

RAN2 to discuss what the relaxed MR serving cell measurements of RAN4 case #3 are used for? RAN2 to discuss whether the use of relaxed MR serving cell measurements of RAN4 case #3, if available, should be left to UE implementation (e.g. used for exit condition).

RAN2 to confirm that cell (re-)selection is based on MR measurements.

## Entry/exit condition for MR serving cell offloading

RAN2 has been discussing entry/exit condition for MR serving cell offloading separately from entry/exit condition for LP-WUS monitoring. In both cases, MR and optionally LR measurements are used for entry. And in both cases LR measurements are used for the exit condition.

There is no power saving gain when the entry condition for offloading is fulfilled but the entry condition for LP-WUS monitoring is not fulfilled, i.e. the MR still has to wake-up every DRX cycle to monitor paging. The same applies when entry for LP-WUS monitoring is fulfilled, but not for serving cell offloading.

It is also not motivated or clear why the entry/exit condition for offloading and LP-WUS monitoring should be different, i.e. the same type of physical signal is used (LP-SS and LP-WUS):

It should be motivated why a separate entry/exit condition for serving cell offloading and LP-WUS monitoring is needed, and why this useful from a power saving perspective.

The same entry/exit condition is configured for both serving cell offloading and LP-WUS monitoring.

RAN2 has not agreed explicitly the UE requirements when the exit condition is fulfilled:

When the LR exit condition is fulfilled, then the UE shall stop using LR measurements and the UE is required to start MR serving cell measurements.

The NW can configure optionally an LR threshold in addition to the MR threshold for entry. In case an LR threshold is configured it can be left to UE implementation when to start/stop these LR measurements:

If an LP-SS threshold is configured for the entry condition, it is left to UE implementation when to start/stop the LP-SS measurements.

## Further MR neighbour cell relaxation

RAN2 agreed to use the Rel-16 relaxation criteria (“not at cell edge” and “low mobility”) as a baseline for further neighbour cell relaxation in Rel-19:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

With Rel-16 neighbour cell relaxation a scaling factor of 3 is used. RAN4 agreed to use a scaling factor ≥ 4 in Rel-19 for further MR neighbour cell relaxation, see annex.

### The need for intra-frequency neighbour cell measurements

In legacy the neighbour cell measurements are configured using intra-frequency (Sintrasearch) and inter-frequency (SnonIntraSearch) measurement thresholds. Typically the UE has to perform intra-frequency neighbour cell measurements in the complete cell and inter-frequency measurements are only triggered when the UE is near the cell border:

Practically the cell border is determined when the UE performs intra-frequency cell reselection, e.g. a neighbour becomes 4 dB better than the serving cell. In case the UE does not find a better intra-frequency neighbour and ends up in poor coverage, then the UE is triggered to perform inter-frequency measurements and find a suitable cell on another frequency. When the UE finds a suitable cell on a higher priority frequency during background measurements, then the UE can leave the serving cell at any time.

It is preferred that the UE performs intra-frequency cell reselection for mobility (shorter interruption and missed paging probability) and that inter-frequency measurements (which take longer time and consume more power) are deferred after intra-frequency measurement attempts are “exhausted”.

In urban environments the intra-frequency (Sintrasearch) cannot be configured close to the inter-frequency (SnonIntraSearch) measurement threshold, because due to the radio propagation conditions a stronger intra-frequency neighbour cell may be detected close to the cell center:

In urban environments the radio conditions may be also change rapidly and dramatically, e.g. when you turn a corner. In case of multi-beam operation the cell coverage may also be beam dependent, i.e. beams of the same cell provide different quality, which may trigger intra-frequency cell reselection:

There can be scenarios where the UE has to measure intra-frequency neighbour cells most of the time and these intra-frequency measurements cannot be relaxed.

For this reason it is beneficial to discuss neighbour cell measurements performed by (OFDM-based) LR, i.e. for the use case when intra-frequency neighbour cell measurements are needed and cannot be relaxed. If the LR performs intra-frequency measurements and ranking then the MR is only woken up when cell reselection is initiated, i.e. the MR is only activated when an intra-frequency neighbour is Q-hyst (db) better than the serving cell based on LR measurements:

RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by (OFDM-based) LR in scenarios where the UE continuously needs to perform intra-frequency measurements and these measurements cannot be relaxed.

### Rel-16 relaxed neighbour cell measurements

Neighbour cell relaxation and LP-WUS coverage

Using the legacy measurements thresholds (SIntraSearch, SNonIntraSearch) and Rel-16 threshold (Not-at-cell-edge) different areas can be identified with different measurement requirements, which also depend on the UE mobility (lowMobility):

The LP-WUS coverage depends on the entry/exit conditions which are independent from the neighbour cell measurements criteria and areas depicted above. The LP-WUS coverage can cover the complete cell or only a small portion in the cell centre independent from the neighbour cell measurement areas.

It is obvious that the legacy neighbour cell measurement requirements apply when the LP-WUS UE is outside LP-WUS coverage or the cell does not support LP-WUS. But also when the LP-WUS UE is inside the LP-WUS coverage area, then the legacy neighbour cell measurement requirements apply, because having an LR that can perform serving cell measurements does not justify that the LP-WUS UE can (further) relax neighbour cell measurements:

The neighbour cell measurement requirements are decoupled from the LP-WUS entry/exit conditions and LP-WUS coverage area.

More specifically this means: when the UE that is within LP-WUS coverage but the UE is moving around, especially towards the cell border, then the UE is not allowed to relax neighbour cell measurements and benefit from LP-WUS power savings.

Neighbour cell relaxation and LR serving cell measurements

In case the serving cell offloading entry condition is fulfilled and only the LR is performing serving cell measurements, then the UE determines, based on the LR serving cell measurements, when the MR has to trigger neighbour cell measurements again, i.e. when low mobility and/or not-at-cell-edge criterion are no longer fulfilled:

When the MR serving cell offloading entry condition is fulfilled then, based on the LR serving cell measurements the UE shall determine when low mobility and/or not-at-cell-edge criterion are no longer fulfilled and the MR has to start neighbour cell measurements again. The low mobility and/or not-at-cell-edge criterion are not part of the LR serving cell exit condition, i.e. the UE is allowed to continue offloading the serving cell measurements from MR to LR when the MR is performing neighbour cell measurements.

The OFDM-based serving cell measurements using PSS/SSS are quite similar in latency and performance compared to the MR serving cell measurements, if not identical:

Ask RAN4 whether the Rel-16 neighbour cell relaxation configuration (relaxedMeasurement-r16) can be re-used with (OFDM-based) LR serving cell measurements. Or if separate parameter value(s) need to be configured with (OOK-based) LR serving measurements.

### Rel-19 further relaxed neighbour cell measurements

RAN4 started to discuss scaling factors for neighbour cell measurements larger than 3 used in Rel-19. For example scaling factors ≥ 8, which allow the MR to enter ultra-deep sleep.

In the figures below it is exemplified how Rel-19 relaxation (i.e. new scaling factors) fits into the legacy and Rel-16 measurement requirements, for the case when Rel-19 relaxation is configured above and below the inter-frequency measurement threshold:

However it is not clear what the added value of Rel-19 further relaxation is because:

When the UE is stationary and above the not-at-cell-edge threshold (Rel-16) then the UE is not required to measure intra- or inter-frequency neighbour cells, i.e. the scaling factor is infinite, i.e. much better than the Rel-19 scaling factor.

And as discussed before in certain scenarios further relaxation is not be feasible. For example in an urban environment where the coverage is very unpredictable and irregular and the propagation conditions can suddenly dramatically change. In such an environment relaxation can only be allowed for a stationary UE, that while it was mobile did not find a better intra-frequency neighbour cell:

There is a risk that paging/cell reselection is impacted when a moving UE does not measure neighbour cells for 10-20 seconds or more (scaling factor between 8 and 16 with DRX cycle 1.28 sec).

It is not clear why neighbour cell measurements can be relaxed for such a long time for a mobile UE, except that such a long time is needed to support ultra-deep sleep and for LP-WUS to be beneficial. In certain scenarios a moving UE cannot be allowed to relax neighbour cell measurements for such a long time:

It is up to the network implementation to configure LP-WUS with or without Rel-19 further neighbour cell relaxation for a moving UE. The Rel-19 scaling factors are further discussed in RAN4.

Higher priority frequency measurements are essential for load balancing, and they can already be relaxed up to 1 hour with Rel-16 relaxation:

Higher priority frequency measurements are not further relaxed.

## RAN2 and RAN4 work split

RAN4 assumes that RAN2 will discuss the offloading and relaxation criteria:

Agreement:

- Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

We agree that RAN2 should discuss the offloading and relaxation criteria and that RAN4 should discuss the relaxed measurement requirements.

During Rel-16 relaxed RRM measurements in idle/inactive a similar work split was envisioned. However the relaxation criteria, i.e. the possible configuration options and the possible combinations of fulfilment of low mobility and/or not-at-cell-edge were captured in both 38.304 and 38.133. Furthermore the way this was captured was different in 38.304 and 38.133 which made it difficult to compare them:

This duplication has led to many LS exchanges between RAN2 and RAN4, and recently discrepancies between RAN2 and RAN4 specification have been discussed [2].

It should be discussed how to avoid overlap between RAN2 and RAN4 specification. One option is that only the “not at cell edge” and “low mobility” criterion are specified in RAN2 specification. And that the different use cases, that depend on what criteria are configured and which criteria are fulfilled with the associated measurement requirements are captured in RAN4 specification:

RAN2 captures the relaxation criteria (e.g. “not at cell edge” and “low mobility”) while RAN4 captures the different uses that can be identified based on the configuration and which criteria are fulfilled.

## LP-SS measurement accuracy and latency

In legacy the UE typically measures the serving cell every DRX cycle (e.g. DRX cycle 1.28 sec), and it takes the average of 2 RSRP/RSRQ measurement samples made during consecutive DRX cycles, see 38.133:

4.2.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

-	otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

Thus the DRX cycle determines the latency in which the UE detects when the serving cell exceeds or drops below a threshold.

For LP-SS measurements RAN1 agreed that the accuracy requirements should be met within 1.28 sec:

Agreement

For LP-SS design from RAN1 perspective, consider at least the following as the design target:

- For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s.

  - FFS: X

  - Note: Y is chosen for evaluating LP-SS design.

  - Network overhead and network power consumption are to be considered

In case of OOK-based WUR and LP-SS the measurement latency is independent of the DRX cycle. The assumption is that the UE is required measure 5 samples of LP-SS measurements that are spaced 320 ms apart:

This means that if the DRX cycle is 320 ms, then the LR measurements are 2x slower compared to the MR measurements. RAN1 agreed to support 320 ms periodicity, and other periodicities are FFS. And RAN4 agreed that the measurement delay depends on the LP-SS periodicities:

Agreement

For LP-SS periodicity, support at least 320ms.

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

RAN4 has been discussing how to capture the LP-SS measurement accuracy requirements, e.g. as dedicated accuracy requirements, potentially including test cases, or as a cell reselection margin (see legacy 3dB margin below) to the core requirements:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

In legacy the UE is required to be able to determine that an intra-frequency neighbour cell is 3 dB better within Tevaluate,NR_Intra in FR1. For LP-WUS a larger margin would need to be defined, and this margin may depend on the LP-SS periodicity. A larger cell reselection margin implies that cell reselection is delayed:

RAN2 to discuss the impact of an increased cell reselection margin when LP-SS measurements or Rel-19 further RRM relaxation are configured.

# Summary

RAN2 is kindly asked to discuss LP-WUS and RRM measurements:

Observation 1	There can be scenarios where the UE has to measure intra-frequency neighbour cells most of the time and these intra-frequency measurements cannot be relaxed.

Observation 2	When the UE is stationary and above the not-at-cell-edge threshold (Rel-16) then the UE is not required to measure intra- or inter-frequency neighbour cells, i.e. the scaling factor is infinite, i.e. much better than the Rel-19 scaling factor.

Observation 3	There is a risk that paging/cell reselection is impacted when a moving UE does not measure neighbour cells for 10-20 seconds or more (scaling factor between 8 and 16 with DRX cycle 1.28 sec).

Proposal 1	When the LR entry condition for serving cell offloading is fulfilled and the LR performs serving cell measurements, then the MR is not required to perform serving cell measurements (i.e. RAN4 case #1 is supported).

Proposal 2	The case when MR performs relaxed serving cell measurements while the LR performs continuous serving cell measurements is not supported (RAN4 case #2, i.e. partial offloading, is not supported).

Proposal 3	RAN2 to discuss what the relaxed MR serving cell measurements of RAN4 case #3 are used for? RAN2 to discuss whether the use of relaxed MR serving cell measurements of RAN4 case #3, if available, should be left to UE implementation (e.g. used for exit condition).

Proposal 4	RAN2 to confirm that cell (re-)selection is based on MR measurements.

Proposal 5	It should be motivated why a separate entry/exit condition for serving cell offloading and LP-WUS monitoring is needed, and why this useful from a power saving perspective.

Proposal 6	The same entry/exit condition is configured for both serving cell offloading and LP-WUS monitoring.

Proposal 7	When the LR exit condition is fulfilled, then the UE shall stop using LR measurements and the UE is required to start MR serving cell measurements.

Proposal 8	If an LP-SS threshold is configured for the entry condition, it is left to UE implementation when to start/stop the LP-SS measurements.

Proposal 9	RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by (OFDM-based) LR in scenarios where the UE continuously needs to perform intra-frequency measurements and these measurements cannot be relaxed.

Proposal 10	The neighbour cell measurement requirements are decoupled from the LP-WUS entry/exit conditions and LP-WUS coverage area.

Proposal 11	When the MR serving cell offloading entry condition is fulfilled then, based on the LR serving cell measurements the UE shall determine when low mobility and/or not-at-cell-edge criterion are no longer fulfilled and the MR has to start neighbour cell measurements again. The low mobility and/or not-at-cell-edge criterion are not part of the LR serving cell exit condition, i.e. the UE is allowed to continue offloading the serving cell measurements from MR to LR when the MR is performing neighbour cell measurements.

Proposal 12	Ask RAN4 whether the Rel-16 neighbour cell relaxation configuration (relaxedMeasurement-r16) can be re-used with (OFDM-based) LR serving cell measurements. Or if separate parameter value(s) need to be configured with (OOK-based) LR serving measurements.

Proposal 13	It is up to the network implementation to configure LP-WUS with or without Rel-19 further neighbour cell relaxation for a moving UE. The Rel-19 scaling factors are further discussed in RAN4.

Proposal 14	Higher priority frequency measurements are not further relaxed.

Proposal 15	RAN2 captures the relaxation criteria (e.g. “not at cell edge” and “low mobility”) while RAN4 captures the different uses that can be identified based on the configuration and which criteria are fulfilled.

Proposal 16	RAN2 to discuss the impact of an increased cell reselection margin when LP-SS measurements or Rel-19 further RRM relaxation are configured.

# References

- R4-2412505, Discussion on LP-WUS RRM requirement, Ericsson, DISC, RAN4#112

- R2-2407341, Correction to Relaxed measurement, LG, CR 38.304, Rel-16, RAN2#127

# Annex: Agreements

## RAN1 agreements (selection)

Agreement

For LP-SS design from RAN1 perspective, consider at least the following as the design target:

- For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s.

  - FFS: X

  - Note: Y is chosen for evaluating LP-SS design.

  - Network overhead and network power consumption are to be considered

Agreement

To determine the binary sequences for LP-SS, consider the following for performance comparison:

- …

- RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s

  - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison

- …

Agreement

For LP-SS periodicity, support at least 320ms.

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement

Regarding whether to support additional sync signal to LP-SS, down-selection from the following:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms]

    - FFS: whether different values can be applicable for different M values

- Option 1B: No additional sync signal, LP-SS periodicity =320ms

  - No additional support of other periodicities

- Option 2: With additional sync signal, LP-SS periodicity =320ms

  - FFS additionally support other LP-SS periodicities

  - FFS details on additional sync signal

  - FFS additional sync signal is configurable and/or conditionally present

## RAN2 agreements (selection)

Serving cell measurements related

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

Neighboring cell measurements related

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

General aspects

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

Serving cell measurment offloading (i.e., there is no serving cell measurement by MR)

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

RRM relaxation for serving and neighbor cell

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Serving and neighbor cell measurements relaxation

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements.

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

## RAN4 agreements (selection)

Issue 1-4-2: LP-SS periodicity for evaluation

Agreement:

- As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded.

  - Not consider the MR DRX configuration in the simulation.

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Issue 1-1-2: Cases/states to be considered for RRM relaxation

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Issue 1-1-1-1: For case 3, whether to combine MR and LP-WUR samples when serving cell measurement when results from both MR and LP-WUR are available

Agreement:

- From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Agreement

RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR

Issue 1-1-8: LP-WUR operating carrier frequency

Agreement:

- RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

- From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

- For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

Issue 1-2-5: Nubmer of Rx for LP-WUR requirements

Agreement

Single Rx is the baseline for LR-WUR RRM measurement.

Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

Agreement:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

- based on LO periodicity

  - FFS whether to define an upper bound

  - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Agreement:

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

- The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Agreement:

Scaling factor(s) is/are >= 4. Further down-select from

- Option 1: 1 fixed scaling factor

- Option 2: multiple values configured by NW

- Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | TBD |
| 0.64 | TBD |
| 1.28 | TBD |
| 2.56 | TBD |

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

Agreement:

Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.